3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212467

e-meeting, 11th – 19th October 2021	(revision of S6-212371 rev4 was S6-212351 rev 2)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.554 | CR | 0012 | rev | 8 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMARCH store and forward |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH |  |  |  |  |  | Date: |  |  | 2021-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add 5GMARCH store-and-forward procedure, consistent with the approved Rel-17 5GMARCH exception sheet |  |  |  |  |  |  |  |  |
| Summary of change: |  | A solution for the 5GMARCH store-and-forward procedure is proposed based on clause 8.3.x of S6-211904_rev4 from SA6 #45. The solution addresses first all the remaining blocking comments received at SA6#45 for S6-211904_rev4 and in the final review period for S6_212118. The solution also addresses the comments received during SA6 #45bis. This includes removal of an initial alternative A, and for the remaining Alternative B: - Combining initial steps 1-3 in new step 1 - Updated descriptions of the two steps for providing responses. - Introduction of new step 4 showing recipient UE becoming available. - Clarifications of activity at expiration time - Cover page corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A 5GMARCH exception sheet item remains open. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ***************** Change 1************************

### 8.3.x	MSGin5G Store and Forward

Figure 8.3.x-1 shows the procedure for providing store and forward services for MSGin5G message requests.

This procedure applies when an MSGin5G inbound message is received at the MSGin5G Server and the recipient UE is not available; otherwise, the procedure detailed in clause 8.3.2 applies.

Pre-conditions:

- The MSGin5G Client or Application Server has registered to the MSGin5G Server.

- The MSGin5G server has determined that the recipient UE is not available.

NOTE: In addition to UE registration status, the MSGin5G Server can use e.g, UE reachability status monitoring specified in clause 8.2 or the recipient’s Communication Availability information specified in clause 8.9.2  to determine whether the recipient is available, i.e. reachable for message delivery.

Figure 8.3.x-1: Store and forward procedure

- Inbound MSGin5G message handling, see steps 1-3 in clause 8.3.2. The value of the Store and forward flag IE (see Table 8.3.2-1) in the MSGin5G message indicates that store and forward services are requested by the sender.

- The MSGin5G Server checks the registration information of the recipient UE. If the Store and forward option IE (see Table 8.2.1-3) indicates that the recipient UE opts out of store and forward services, the message is discarded and the procedure ends. If the Store and forward flag IE (see Table 8.3.2-1) in the received message indicates that store and forward services are not requested by the sender, the message is discarded and the procedure ends.

If store and forward processing is required, the MSGin5G Server uses the Application specific store and forward information IE (see Table 8.3.2-2) to determine storage and forwarding

- The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the delivery had been deferred

- The recipient UE becomes available.

- When the recipient UE becomes available, the MSGin5G Server attempts delivery of the request using the procedure specified in clause 8.3.3.

If the UE does not become available  prior to the expiration time, the MSGin5G Server attempts delivery of the request at the message expiration time and the stored message is discarded afterwards.

- The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the message was discarded.