3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212469

# e-meeting, October 11 – 19, 2021	(revision of S6-212327, 2424)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0004 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.783, the Sol#24 and Sol#28 address the MBS session release and de-announcement aspect when the MBS session is no longer required. The MBS session release is supported by the following clauses/procedures in TS 23.247. Clause in 23.247 (v 17.0.0); Analysis. 7.1.1.4 Removal of MBS session configuration without PCC; No open issues nor editor’s note in this clause.. 7.1.1.5 Removal of MBS session configuration with PCC; No open issues nor editor’s note in this clause.. 7.2.2.3 SMF removing joined UEs from MBS session; No open issues nor editor’s note in this clause.. 7.2.2.4 Release of shared delivery toward RAN node; No open issues nor editor’s note in this clause.. 7.3.2 MBS Session Release for Broadcast; No open issues nor editor’s note in this clause.. As shown above, the MBS session release feature and procedures are stable without any opening issue and editor’s note in latest 23.247 v17.0.0, which allows the SA6 to make conclusion for the corresponding aspect in TR 23.783. As a result, it is proposed to also add the proposal in the conclusion to TS 23.289. |
| Summary of change: |  | Introduce Sol#24 and Sol#28 of TS 23.783 in TS 23.289. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no description of MBS session release in current technical specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new), 7.x.y (new), 7.x.y.1 (new), 7.x.y.1 (new), 7.x.y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G MBS

### 7.X.Y	MBS session release

##### 7.X.Y.1	General

The MC service server can decide to release a certain MBS session, once it is no longer further utilized for the associated MC service group communication, e.g., the MC service group is no longer active, the MC media transmission is over and no further MC media to be delivered, group communication is terminated. The MBS session release procedure leads to releasing the network resources associated to that MBS session.

NOTE:	It is up to implementation of MC service server to decide whether to release the MBS session or re-use it for subsequent group operations.

To release the MBS session, the MC service server sends an MBS session release request to the 5GS providing the corresponding MBS session ID. The MBS session release request is sent to the MB-SMF (directly or via NEF/MBSF) when PCC is not used. Further details are provided in 3GPP TS 23.247 [x]. MC service server further informs the MC service client with the MBS session de-announcement, so that the MC client UE stops monitoring the broadcast MBS session or leaves the multicast MBS session.

This procedure is applied for both broadcast MBS session and multicast MBS session.

##### 7.X.Y.2	Procedure

The procedure in figure 7.X.Y.2-1 describes the MBS session release aspects for group communication.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	An MBS session is configured to address the corresponding MC service group with certain service requirements and optionally with a certain broadcast/multicast service area. The session is announced and established for group communication purposes for the MC service group.

Figure 7.X.Y.2-1: MBS session release for MC group communication.

1.	The MC service server decides to release using the MBS session for the associated MC group communication, either multicast or broadcast session.

2.	The MC service server sends an MBS service de-announcement message with the MBS session ID towards the MC service client(s). Upon receiving the MBS service de-announcement message, either 3a or 3b is performed.

3a.	If the MBS session identified by MBS session ID is broadcast MBS session, the MC service client(s) stops monitoring the broadcast MBS session and removes the broadcast MBS session related information.

3b.	If the MBS session identified by MBS session ID is multicast MBS session, the joined MC service client(s) initiate an MBS session leave procedure to leave the indicated MBS session in order to release the respective network resources, as defined in 3GPP TS 23.247 [x].

NOTE:	Steps 2 and 4 can be executed in parallel or step 2 can be executed after step 4.

4.	The MC service server initiates the MBS session release procedure with the 5GC (either directly or through NEF/MBSF) in order to stop using the configured MBS session. The MC service server indicates within the MBS session release request the corresponding MBS session ID. Hence, the associated network resources are released. The MBS session release procedures are performed according to 3GPP TS 23.247 [x].

# * * * * End of changes * * * *