3GPP TSG-SA WG6 Meeting #45	S6-212032

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0015 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the C2 operation mode switching confirmation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Step 3 in clause 7.4.2.4 indicates “The UAE Server determines the switching of the C2 mode from direct to network assisted or vice versa or to USS/UTM navigated …” and step 4 also indicates “… The UAE Server sends this request to obtain confirmation from the UAS application specific server before proceeding with switching to USS/UTM navigated. o However, table 7.4.3.5-1 indicates “The type of the C2 mode switching to be applied as (direct to network-assisted, or network-assisted to direct). It is missing the "to USS/UTM navigated" mode. - Step 3 in clause 7.4.2.4 indicates “The UAE Server sends a C2 mode switching confirmation request to the UAS application specific server, which includes the UAS identifier as well as the cause for switching and the switching option (direct to network-assisted or network-assisted to direct or to USS/UTM navigated) …”. However, the cause of switching is not listed in table 7.4.3.5-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the information flow specified in clause 7.4.3.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.4.3.5	C2 mode switching confirmation request

Table 7.4.3.5-1 describes the information flow C2 mode switching confirmation request from the UAE server to the UAS application specific server.

Table 7.4.3.5-1: C2 mode switching confirmation request

| Information element | Status | Description |
| --- | --- | --- |
| UAE server ID | M | The identifier of the UAE server which requests the C2 mode switching confirmation from UTM/USS |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g. group ID; or individual identifiers for the UAV and UAV-C, e.g. CAA ID, GPSI. |
| C2 operation mode switching type | M | The type of the C2 mode switching to be applied (direct to network-assisted, or network-assisted to direct, or to USS/UTM navigated). |
| Switching cause | O | Cause information for initiating the switching (e.g. poor radio link quality) |