3GPP TSG-SA WG6 Meeting #45	S6-212139

e-meeting, 25th August – 3rd September 2021	(revision of S6-212031)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0014 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the usage of UAS ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | All information flows in procedures specified in clause 7.4.2 use UAS ID, while the information flow specified in clause 7.4.3.10 uses UAV ID and Peer UAV ID separately. UAS ID already includes the UAV ID and Peer UAV ID, hence UAS ID should be consistently used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the information flow in clause 7.4.3.10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion may develop for stage 3 implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.1, 7.4.3.5, 7.4.3.8, 7.4.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.4.3.1	C2 operation mode management request

Table 7.4.3.1-1 describes the information flow C2 operation mode management request from the UAS application specific server to the UAE server.

Table 7.4.3.1-1: C2 operation mode management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the C2 operation mode management. This ID can be the UTM/USS identifier, when the UAS application specific server is the UTM/USS. |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM navigated. |
| Allowed C2 communication modes | M | direct, network assisted, USS/UTM navigated |
| Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| Policy of C2 switching | M | Parameters for C2 switching (NOTE) - QoS thresholds on active link - QoS thresholds on target link |
| C2 service area | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the capability is active. |
| NOTE: A complete list of parameters for the QoS-policy of C2 switching is specified by stage 3. |  |  |

* * * Next Change * * * *

#### 7.4.3.5	C2 mode switching confirmation request

Table 7.4.3.5-1 describes the information flow C2 mode switching confirmation request from the UAE server to the UAS application specific server.

Table 7.4.3.5-1: C2 mode switching confirmation request

| Information element | Status | Description |
| --- | --- | --- |
| UAE server ID | M | The identifier of the UAE server which requests the C2 mode switching confirmation from UTM/USS |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| C2 operation mode switching type | M | The type of the C2 mode switching to be applied (direct to network-assisted, or network-assisted to direct). |

* * * Next Change * * * *

#### 7.4.3.8	C2 communication modes configuration request

Table 7.4.3.8-1 describes the information flow C2 communication modes configuration request from the UAE server to the UAE client.

Table 7.4.3.8-1: C2 communication modes configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA ID, GPSI, IP address. |
| C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM navigated. |
| Allowed C2 communication modes | M | direct, network assisted, USS/UTM navigated |
| Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| Policy of C2 switching | M | Parameters for C2 switching (NOTE) - QoS thresholds on active link - QoS thresholds on target link |
| NOTE: A complete list of parameters for the QoS-policy of C2 switching is specified by stage 3. |  |  |

* * * Next Change * * * *

#### 7.4.3.10	C2 communication mode notification

Table 7.4.3.10-1 describes the information flow C2 communication mode notification from the UAE client to the UAE server and from the UAE server to the UAS application specific server.

Table 7.4.3.10-1: C2 communication mode notification

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| Selected primary C2 communication mode | M | Selected primary C2 communication mode (direct, network assisted) |
| Selected secondary C2 communication mode | O | Selected secondary C2 communication mode (direct, network assisted) |