3GPP TSG-SA WG6 Meeting #46-e	S6-212563

e-meeting, 15th – 23rd November 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0084 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Replace the NSCM with NSCE to align the terminologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A SI for enhancement of network slice capability exposure is ongoing. Two terminologies for the network slice capablility exposure functions are introduced. To avoid the confusion, the terminologies need to be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the NSCM with NSCE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The terminologies may lead confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.3, 16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

15.3	Service-based interfaces

Table 15.3-1 specifies the service-based interfaces supported by SEAL.

Table 15.3-1: Service-based interfaces supported by SEAL

| Service-based interface | Application functionEntity | Mapping server entity | APIs offered |
| --- | --- | --- | --- |
| Slm | Location management function | Location Management Server | Specified in subclause 9.4 |
| Sgm | Group management function | Group management server | Specified in subclause 10.4 |
| Scm | Configuration management function | Configuration management server | Specified in subclause 11.4 |
| Sim | Identity management function | Identity management server | Specified in subclause 12.4 |
| Skm | Key management function | Key management server | Specified in subclause 13.4 |
| Snrm | Network resource management function | Network resource management server | Specified in subclause 14.4 |
| Snsce | Network slice capability exposure function | Network slice capability exposure server | Specified in subclause 16.4 |
| Cccf | CAPIF core function | Not applicable | Specified in subclause 10 of 3GPP TS 23.222 [8] |

* * * Next Change * * * *

16	Network slice capability exposure

16.1	General

The network slice capability exposure is a SEAL service that offers network slice capability exposure capabilities, such as support for vertical application to slice re-mapping (which can be defined as the mapping of the UEs running a vertical application to different slice), to one or more vertical applications. In particular, network slice capability exposure uses a network-based mechanism to apply the slice re-mapping based on the network slice capability exposure server configuration, where the network slice capability exposure server acting as AF influences the URSP rules for the application traffic per UE by providing a guidance on the route selection parameters (including the S-NSSAI and DNN mapping).

NOTE:	The network slice capability exposure server, acts as an AF within the trusted domain (MNO-owned, or trusted third party).

16.2	Functional model

16.2.1	General

The functional model for the network slice capability exposure is based on the generic functional model specified in clause 6.2. It is organized into functional entities to describe a functional architecture which addresses the support for network slice capability exposure aspects for vertical applications. Since the slicing is a feature which considers the Uu interfaces, only the on-network functional model is specified in this clause.

16.2.2	Functional model description

Figure 16.2.2-1 illustrates the generic functional model for network slice capability exposure.

Figure 16.2.2-1: Functional model for network slice capability exposure

The network slice capability exposure client communicates with the network slice capability exposure server over the NSCE-UU reference point. The network slice capability exposure client provides the support for network slice capability exposure functions to the VAL client(s) over NSCE-C reference point. The VAL server(s) communicates with the network slice capability exposure server over the NSCE-S reference point. It is assumed that the network slice capability exposure server is deployed at the 5G system domain. The network slice capability exposure server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point (for interactions with PCF).

16.2.3	Functional entities description

16.2.3.1 General

The functional entities for network slice capability exposure SEAL service are described in the following subclauses.

16.2.3.2 Network slice capability exposure server

The network slice capability exposure server functional entity provides the enablement of the network slicing aspects to support the VAL applications. Such enablement supports the mapping or migration of one or more vertical applications to one or more network slices (from a set of network slices, as provided by the 3GPP network system) as described in procedure in clauses 16.3.2.3 and 16.3.2.4. Such adaptation assumes that the UE is subscribed to more than one slice and is done via providing a guidance to update the URSP rules at the 5GS (denoted in clause 16.3.3 as network-based mechanism).

16.2.3.3 Network slice capability exposure client

The network slice capability exposure client functional entity acts as the application client for the slice enablement. The network slice capability exposure client interacts with the network slice capability exposure server to trigger a network slice adaptation due to an application requirement change. This trigger may be due to an application QoS requirement change, a service operation change. The NSCE client may receive a network slice / DNN re-mapping notification from the NSCE server. The NSCE client may optionally notify the VAL client on the network slice / DNN re-mapping.

16.2.4	Reference points description

16.2.4.1	General

The reference points for the functional model for network slice capability exposure are described in the following subclauses.

16.2.4.2	NSCE-UU

The interactions related to network slice capability exposure functions between the network slice capability exposure server and the network slice capability exposure client are supported by NSCE-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.501 [10].

16.2.4.3	NSCE-C

The interactions related to network slice capability exposure functions between the VAL client(s) and the network slice capability exposure client within a VAL UE are supported by the NSCE-C reference point. The NSCE client may receive an application requirement change over NSCE-C. Further, the NSCE client may provide a notification on the network slice adaptation upon successful adaptation of the slice to application mapping.

16.2.4.4	NSCE-S

The interactions related to network slice capability exposure functions between the VAL server(s) and the network slice capability exposure server are supported by the NSCE-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

16.2.4.5	N33

The reference point N33 supports the interactions between the network slice capability exposure server and the NEF and is specified in 3GPP TS 23.501 [10]. N33 is used for the network-based mechanism for slice re-mapping, where NSCE server acting as AF influences the URSP rules for the application traffic per UE by providing a guidance on the route selection parameters (including the S-NSSAI and DNN mapping), as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4.

16.3	Procedures and information flows for network slice capability exposure

16.3.1	General

16.3.2	Network slice adaptation for VAL application

16.3.2.1	General

This subclause describes the procedure for network slice adaptation at the Network Slice Capability Exposure (NSCE) server, based on a request from a VAL server to adapt the network slice for the VAL application. This request is handled between the NSCE server and the NSCE client per each VAL UE of the VAL application.

16.3.2.2	Information flows

16.3.2.2.1	Network slice adaptation request

Table 16.3.2.2.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCE server.

Table 16.3.2.2.1-1: Network slice adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation may corresponds to. |
| List of VAL UE IDs | M | List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds |
| Network slice adaptation cause | O | The cause which necessitates the network slice change (e.g. VAL service profile change, VAL service operation change). |
| Requested S-NSSAI | O | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |

Editor's note: The details of Network slice adaptation request is FFS

16.3.2.2.2	Network slice adaptation response

Table 16.3.2.2.2-1 describes the information flow network slice adaptation response from the NSCE server to the VAL server.

Table 16.3.2.2.2-1: Network slice adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation with the underlying network. |

16.3.2.2.3	Network slice adaptation trigger

Table 16.3.2.2.3-1 describes the information flow Network slice adaptation trigger from the NSCE client to the NSCE server.

Table 16.3.2.2.3-1: Network slice adaptation trigger

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation trigger applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice configuration may corresponds to. |
| Requested S-NSSAI | M | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |

16.3.2.2.4	Network slice adaptation notification

Table 16.3.2.2.4-1 describes the information flow network slice adaptation notification from the NSCE server to the NSCE client and optionally to the VAL client.

Table 16.3.2.2.4-1: Network slice adaptation notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation. |

16.3.2.3	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.3-1 illustrates the VAL server-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Figure 16.3.2.3-1: Network slice adaptation for VAL application

1.	The VAL server sends a network slice adaptation request to the NSCE server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN).

2.	The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

3.	The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE:	NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

4.  Upon successful adaptation of the route selection parameters, the NSCE server provides a network slice adaptation response to the VAL server, providing information on the fulfilment of the network slice adaptation request per VAL application.

16.3.2.4	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.4-1 illustrates the VAL UE-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Pre-condition:

-	The NSCE client has connected to the NSCE server;

Figure 16.3.2.4-1: Network slice adaptation for VAL application

1.	The VAL client provides a new application requirement to the NSCE client, indicating a new service profile for the VAL application. This may be in the form of a change at the application QoS requirements, service operation change, or other application-related parameters.

2.	The NSCE client sends a network slice adaptation trigger to the NSCE server for the VAL application. This trigger may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN).

3.	The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

4.	The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE:	NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

5. The NSCE server may send a notification to the NSCE client indicating success or failure.

16.4	SEAL APIs for network slice capability exposure

16.4.1	General

Editor's note: The definition of SEAL APIs for NSCE-UU is FFS

Table 16.4.1-1 illustrates the SEAL APIs for network slice capability exposure.

Table 16.4.1-1: List of SEAL APIs for network slice capability exposure

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkSliceAdaptation | Network_slice_adaptation | VAL server | Request /Response |

16.4.2	SS_NetworkSliceAdaptation API

16.4.2.1	General

API description: This API enables the VAL server to communicate with the network slice capability exposure server for network slice adaptation over NSCE-S.

16.4.2.2	Network_Slice_Adaptation

API operation name: Network_Slice_Adaptation

Description: Requesting for network slice adaptation.

Known Consumers: VAL server.

Inputs: See subclause 16.3.2.2.1

Outputs: See subclause 16.3.2.2.2

See subclause 16.3.2.3 for the details of usage of this API operation.