3GPP TSG-SA WG6 Meeting #47-e	S6-220057

e-meeting, 14th – 22nd February 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0304 | rev | <Rev#> | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCPTT Group ID for pre-configured group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | At&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 5 Feb, 2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | R18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MCPTT Group ID IE in table10.6.2.2.7 of Group call request is optional which is not correct. The Group call request is used to start a pre-configured group call and the group ID shall be included so that the MCPTT server knows who are the group members that need to be invited to the call. Here is the change history of this Group ID IE over the years: - The group ID IE is defined as “M” from the beginning and remained so when R14 was completed and approved in SA#74 (12-2016). - In Feb 2017 in SA6#15 meeting, the agreed R14 CR in S6-170190 changed the group ID IE to “O” as a solution to support the temporary group call - user regroup. When it is a temporary user regroup call there is no pre-configured group hence the Group ID is optional. This change has been stayed in the spec since that time. - The solution for the above-mentioned temporary group call - user regroup had been through many discussions and eventually (with no SA3 endorsed security solution) it was finally agreed to be removed for R16 in the CR agreed in S6-192217 in SA6#34 in Nov 2019. But the MCPTT Group ID IE remains O which is an overlook and is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the MCPTT Group ID IE to “M” in table 10.6.2.2.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stgage 3 implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.6.2.2.7	Group call request (MCPTT client – MCPTT server)

Table 10.6.2.2.7-1 describes the information flow group call request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.7-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP offer | M | Media parameters of MCPTT clients |
| Implicit floor request | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Location information | O | Location of the calling party. |
| Requested priority | O | Application priority level requested for this call |