3GPP TSG-SA WG6 Meeting #47-e	S6-220273

e-meeting, 14th – 22nd February 2022	(revision of S6-220138)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0028 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the MBS session release related terminology and aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The introduced CR provides an update for terminology alignment related to deletion an MBS session, as described in 3GPP TS 23.247. Furthermore, the session deletion procedure may be with a dynamic or without a dynamic PCC rule, as indicated in 3GPP TS 23.247. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MBS session deletion to delete an MBS session once it is no longer used. - The MBS session deletion procedure depends on whether a dynamic PCC rule is involved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No alignment with 3GPP TS 23.247 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.3 and its subsequent clauses: 7.3.3.3.1 and 7.3.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.3.3.3	MBS session deletion request

##### 7.3.3.3.1	General

The MC service server can decide to release a certain MBS session once it is no longer further utilized for the associated MC service group communication, e.g., the MC service group is no longer active, the MC media transmission is over and no further MC media to be delivered, group communication is terminated. The MBS session deletion procedure leads to releasing the network resources associated to that MBS session.

NOTE:	It is up to implementation of MC service server to decide whether to release the MBS session or re-use it for subsequent group operations.

To delete the MBS session, the MC service server sends an MBS session deletion request to the 5GS providing the corresponding MBS session ID. The MBS session deletion request is sent to the MB-SMF (directly or via NEF/MBSF) when PCC is not used. However, if dynamic PCC rule is utilized, a policy authorization deletion request is initially sent to the PCF. Further details are provided in 3GPP TS 23.247 [15].

MC service server further informs the MC service client with the MBS session de-announcement, so that the MC client UE stops monitoring the broadcast MBS session or leaves the multicast MBS session. This procedure is applied for both broadcast MBS session and multicast MBS session.

##### 7.3.3.3.2	Procedure

The procedure in figure 7.3.3.3.2-1 describes the MBS session deletion aspects for group communication.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	An MBS session is configured to address the corresponding MC service group with certain service requirements and optionally with a certain broadcast/multicast service area. The session is announced and established for group communication purposes for the MC service group.

Figure 7.3.3.3.2-1: MBS session deletion procedure..

1.	The MC service server decides to delete  the MBS session for the associated MC group communication, either multicast or broadcast session.

2.	The MC service server sends an MBS session de-announcement message with the MBS session ID towards the MC service client(s). Upon receiving the MBS session de-announcement message, either 3a or 3b is performed.

3a.	If the MBS session identified by MBS session ID is a broadcast MBS session, the MC service client(s) stops monitoring the broadcast MBS session and removes the broadcast MBS session related information.

3b.	If the MBS session identified by MBS session ID is a multicast MBS session, the joined MC service client(s) initiate an MBS session leave procedure to leave the indicated MBS session in order to release the respective network resources, as defined in 3GPP TS 23.247 [15].

4. Subsequently, the MC service clients may send an MBS session de-announcement acknowledgement message to the MC service server indicating the status of MBS session.

5.	The MC service server initiates the MBS session deletion procedure with the 5GC (either directly or through NEF/MBSF) in order to stop using the configured MBS session and release the corresponding network resources. The MC service server indicates within the MBS session release request the corresponding MBS session ID. The MBS session deletion procedure can either be with or without a dynamic PCC rule, as indicated in 3GPP TS 23.247 [15].

# * * * * Second change * * * *

#### 7.3.2.x MBS session de-announcement

Table 7.3.2.x-1 describes the information flow of an MBS session de-announcement sent from the MC service server to the MC service clients.

Table 7.3.2.x-1: MBS session de-announcement.

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | The identity of the MBS session(s) to be de-announced. |

#### 7.3.2.y MBS session de-announcement acknowledgement

Table 7.3.2.y-1 describes the information flow of an MBS session de-announcement acknowledgement message sent from the MC service clients to the MBS service server, to acknowledge the reception of the MBS session de-announcement.

Table 7.3.2.y-1: MBS session de-announcement acknowledgement.

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service identity of the MC service client, whose group is no longer associated to the MBS session(s) |
| MBS session ID(s) | M | The identity of the MBS session(s) to be de-announced. |
| MBS session de-announcement status | M | The de-announcement status per MBS session ID. |