3GPP TSG-SA WG6 Meeting #48-e	S6-220585rev1

e-meeting, 5th – 14th April 2022	(revision of S6-22xxxx)

Source:	AsiaInfo Technologies Inc, Huawei，CMCC

Title:	Solution to KI #11 on application layer slice SLA alignment capability

Spec: 	3GPP TR 23.700-99 v1.0.0

Agenda Item:	9.4

Document for: 	Approval

Contact:	Limeng Ma, malm5@asiainfo.com

1. Introduction

The key issue of dynamic slice SLA alignment is introducing in TR 23.700-99 v1.0.0., it could be possible to achieve the maximum return of investment and slice/service self-management, the verticals/enterprises expect a more optimal service profile which contains the exact value of those SLA parameters to support to the executing services' demands, i.e., dynamic SLA alignment.

2. Reason for Change

-	Whether and How the SEAL network slice capability management service supports better alignment between vertical needs and SLA parameters for initial service requirements and ongoing service conditions?

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-99 v1.0.0.

* * * First Change * * * *

- 6.X	Solution X: Slice SLA alignment capability

- 6.X.1 Solution description

- 6.X.1.1	General

This solution aims to address the issue identified in Key Issue 11.

This solution provides a procedure to illustrate slice SLA alignment capability by the network slice capability exposure server.

To enable the VAL server to align slice SLA. The NECE layer should help the VAL server to align the vertical needs with the SLA parameters to achieve the maximum return of investment and slice/service self-management.

- 6.X.1.2	Slice SLA alignment capability

Figure 6.2.1.2-1 illustrates slice SLA alignment capability process to address the key issue 11 of Application layer slice SLA alignment capability exposure described in clause 5.4.

Pre-conditions:

- The VAL Server has subscribed to the slice SLA alignment capability

- The VAL Server has subscribed to the capability of Network slice related performance and analytics exposure.

- The network slice enabler layer is capable to interact with NEF/NWDAF and OAM system

- The network slice enabler layer is registered/capable for interacting with 5GS such as triggering network slice modification operations, and has collected current network slice capabilities.

Figure 6.X.1.2-1 SLA alignment process

- The VAL server sends a request to NSCE server to slice SLA alignment.

- The NSCE server shall check if the VAL server is authorized to get the network slice SLA parameters and performance data.

- The NSCE server response to VAL server with the result of slice SLA alignment request.

- On receiving the request from VAL server, NSCE server retrieves the slice SLA parameters as defined in clause 6.3.3, TS 28.541[14] from 5GS. Form OAM system, the network slice instance related performance data defined in TS 28.552[12] and for CN functions service experience analytics for a network slice as defined in TS 23.288 [17] are collected.

- NSCE server may send a request to application layer to request the KQI data.

- VAL server may send the requested KQI data to NSCE server.

- NSCE server may send a request to NSCE client to collect the KQI data if possible.

- NSCE client report the requested data collected by itself if possible, the information collected by the NSCE client depends on the third-parties’ implementations.

- NSCE server compares the SLA parameters of network slice instance with the KQI data of the VAL and network slice performance data to generate the optimal SLA for the required service and determine the network slice modification operations should be taken depended on implementation (e.g., if the VAL client experience are satisfied while the network slice resources are with low utilization, the resources required in the SLA could be reduced).

- Based on decision made by NSCE server, the NSCE server performs the network slice modification operation(s), as defined in clause 7.6, TS 28.531[5] exposed by EGMF defined in SA5.

- NSCE server notifies the updated SLA parameters which contains the exact value of those SLA parameters to support to the executing services' demands.

- 6.X.2 Solution evaluation

The solution addresses the key issue #11 of dynamic slice alignment. In this solution, the NSCE server interacts with VAL Server and 5GS respectively to compare the the SLA requirements with network current situations to estimate if the current SLA requirements are properly configured. The network situations can be monitored by utilize the the performance assurance related management services exposed by EGMF. The SLA requirements can be derived from the VAL server via the APIs over S-NSCE interface.