3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221605

e-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0337 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the migration terminology and related aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The introduced CR provides a proper definition of migration, and clarifies related aspects, for example, the migrated MC service user utilizes the application layer or platform in the partner MC system. The CR highlights that during migration authorization procedure (described in clause 10.6.3.3), the MC service server in the primary MC system marks the migrated MC service users and their corresponding partner systems, in order to locate the migrated users, e.g., once they are targeted for a private communication. Finally, the CR emphasizes that the migrated MC service users are reachable via their MC service ID, which is assigned to by their primary MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding migration definition. - Highlighting clause 10.6.3.3 that the MC service server keeps track of the migrated MC service users and their corresponding partner MC systems. - The migrated MC service users are reachable via their original MC service IDs, which are assigned to them by their primary MC system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion and unclarity related to migration definition and corresponding aspects are not solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.2.9	Migration

#### 5.2.9.1	General

Migration provides means for MC users to obtain MC services when moving to a partner MC system(s). The migrated MC service users are then able to participate in private and group communications both with users from their primary MC system as well as with users from the partner MC system.

MC service interconnection needs to be provided between MC systems that wish to provide migration of their MC service users.

Migration of MC service users between any two MC systems can be on a bilateral or unilateral basis.

NOTE:	Whether migration is bilateral or unilateral between any two MC systems is left to business agreements between the two MC service providers, and is outside the scope of the present document.

Via migration, the MC service user can utilize the application platform (layer) at the partner MC system and is able to receive MC services.

Prior to migration to a partner MC system, the MC service user informs the primary MC system the target partner system. This is done once the MC service server in the primary MC system authorizes the migration. Hence, the primary MC system keeps track of its migrated MC users and the corresponding partner MC systems.

During migration of an MC service user to a partner MC system, media of the MC service user's calls may need to be routed to the MC service user's primary MC system e.g. for logging purposes.

The migrated MC service user is still reachable by users from both its primary and partner MC systems via its MC service ID, which is provided by the primary MC system as described in clause 8.1.2.

Editor`s note: how migrated MC service user is reachable by users from partner MC system is FFS.

# * * * * End of changes * * * *