3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221607

e-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0338 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Private call towards an already migrated MC service user |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces a generic procedure to be followed for a private communication taken place towards an already migrated MC service user. In this scenario, both the private call-initiated MC service user and the targeted MC service user have the same primary MC system. The private communication can either be an MCPTT private call as described in 3GPP TS 23.379, or an MCVideo as in 3GPP TS 23.281. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A procedure describing a private call targeting an already migrated MC service user is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behavior during a private call towards a migrated MC service user is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.16.X Private call towards an already migrated MC service user

#### 10.16.X.1 General

This subclause describes the procedure to be considered in order to initiate a private call, e.g., a private MCPTT call or a private MCVideo targeting a migrated MC service user.

#### 10.16.X.2 Procedure

Figure 10.16.X.2-1 describes a private call procedure towards an already migrated private call.

Pre-conditions:

- MC service client 1 and 2 both belong to the primary MC system A.

- MC service client 2 has migrated to the partner MC system B.

- MC system A has already marked that user 2 is migrated to MC system B during migration authorization procedure in clause 10.6.3.

Figure 10.16.X.2-1: Private call procedure towards an already migrated MC service user.

- The user at MC service client 1 initiates a private call towards MC service client 2, e.g., an MCPTT private call as described in 3GPP TS 23.379 [16], or an MCVideo private call as in 3GPP TS 23.281 [12]. For the case of an MCPTT private call with floor control, the floor control is to be established.

- MC service client 1 sends a private call request towards MC service server A. The private call request includes among others the MC service ID of user 2 and SDP information. For the case of an MCPTT private call with floor control, the private call request includes elements indicating that user 1 requests the floor. The MCPTT private call request includes the information elements described in Table 10.7.2.1.1-1 in 3GPP TS 23.379 [16], whereas for MCVideo the information elements in Table 7.2.2.2.1-1 in 3GPP TS 23.281 [12].

- MC service server A checks whether MC service client 1 is authorized to initiate a private call.

- MC service server A locates that user 2 has migrated to MC system B.

NOTE 1: it is implementation specific how the MC service server stores information related to migrated MC service users and the corresponding partner systems.

- MC service server 1 may provide a progress indication to MC service client 1 to indicate progress in the call setup process.

NOTE 2: step 5 can occur any time after step 4 and before step 12.

- MC service server A sends a private call request towards the MC service server B. The private call request includes among others the MC service IDs of the initiated private call user and the targeted user. The MCPTT private call request includes information elements described in Table 10.7.2.1.2-1 in 3GPP TS 23.379 [16], whereas for MCVideo the information elements in Table 7.2.2.2.2-1 in 3GPP TS 23.281 [12]. If location information is included in the private call request, the MC service server A checks the privacy policy of MC service user 1 to decide if the location information of client 1 can be provided to the targeted MC service user during the private call.

- MC service server B checks whether the MC service client 2 is authorized to receive a private call.

- If authorized, the MC service server B provides the private call request information towards MC service client 2, which are determined in the initial private call request. The MCPTT private call request includes information elements described in Table 10.7.2.1.2a in 3GPP TS 23.379 [16], whereas MCVideo private call request the information elements in Table 7.2.2.2.2-1 in 3GPP TS 23.281 [12].

- User 2 is notified about the incoming private call.

- MC service client 2 accepts the private call and acknowledges it by sending a private call response to MC service server B. MC service client 1 can be informed about the successful call established once MC service client 2 has sent a private call response. The MCPTT private call response includes information elements described in 10.7.2.1.3-1 in 3GPP TS 23.379 [16], whereas the MCVideo private call response the information elements in Table 7.2.2.2.3-1 in 3GPP TS 23.281 [12].

- MC private call response is sent from MC service server B to MC service server A. The MCPTT private call response includes information elements described in 10.7.2.1.4-1 in 3GPP TS 23.379 [16], whereas the MCVideo private call response the information elements in Table 7.2.2.2.4-1 in 3GPP TS 23.281 [12].

- MC private call response is sent from MC service server A to MC service client 1. The MCPTT private call response includes information elements described in 10.7.2.1.4-1 in 3GPP TS 23.379 [16], whereas the MCVideo private call response the information elements in Table 7.2.2.2.4-1 in 3GPP TS 23.281 [12].

- A media plane is successfully established between MC service client 1 and 2 for communication and media transmission. For successful call establishment of a private call with implicit floor control request from MCPTT client 1, floor participant at MCPTT client 1 is granted floor by the floor control server at MCPTT service server A, giving it permission to transmit. At the same time floor participant at MCPTT client 2 is informed by the floor control server at MCPTT service server B that the floor is taken.

# * * * * End of changes * * * *