3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221617

e-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0109 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify location user consent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2022-06-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, the EAS may already obtain end user’s consent for retrieving end user’s location authorization before invoking Eees_UELocation API. This case is not supported by SA3 in TS 33.558 and is also under SNAAPP study in TR 23.700-95 as solution#4. Therefore, such EAS obtained user consent is not supported in Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the EAS obtained user consent handling is not supported. Remove access token description which is out of SA6 remit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarity in stage 2 TS leads to stage 3 implementation confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.2.2, 8.6.2.2.3.2, 8.6.2.3.2, 8.6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.6.2.2.2	Request-response model

Figure 8.6.2.2.2-1 illustrates the interactions between the EES and the EAS for one-time location report.

Pre-conditions:

1.	The EAS is authorized to discover and to use UE location API provided by the EES.

2.	The EES is authorized to use Nnef Event Exposure API for UE location, based on service level agreement with MNO.

3.	UE Identifier between EAS and the EES is authorized for the UE location API.

NOTE 0:	EAS obtained user consent handling is not supported in the current release.

Figure 8.6.2.2.2-1: UE location API request-response model

1.	The EAS sends UE location request to the EES. The UE location request shall include the UE Identifier. It may also include location granularity to indicate requested format of the location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the EAS and location QoS.

NOTE 1:	The trigger condition of the UE location API is up to application service logic, which is out of scope of this specification.

2.	The EES checks the UE location:

a.	If the UE location request from the EAS includes the location granularity, the EES considers the location granularity parameter for checking the location of the UE.

b.	If the EES has a valid locally cached location of the UE, the EES may use this cached UE location to respond to the EAS. Otherwise, the EES utilizes the capabilities of the 3GPP core network as specified in clause 8.10.3.

c.	The EES may modify the format of the UE location to fit to the location granularity requested from the EAS in the step 1, if supported. For example, if the EES receives the UE location in a format of tracking area ID or cell ID, the EES can modify the representation of the location information to the GPS Coordinates, or civic addresses (e.g. streets, districts, etc.) as requested by the EAS.

NOTE 2:	Format modification mechanism of the UE location to fit the location granularity is outside the scope of this specification. Step 2.c. can be performed if the EES is deployed by the PLMN operator.

3.	If successful, the EES responds to the EAS with the location of the UE, and optionally the timestamp of the location.

* * * Next Change * * * *

###### 8.6.2.2.3.2	Subscribe

Figure 8.6.2.2.3.2-1 illustrates the subscribe operation between the EAS and the EES for continuous UE location reporting.

Pre-conditions:

1.	The EAS is authorized to discover and to use UE location API provided by the EES;

2.	The EES is authorized to use 3GPP network exposure capability (e.g. Nnef Event Exposure API) for UE location, based on service level agreement with MNO; and

3.	UE Identifier between EAS and the EES is authorized for the UE location API.

NOTE:	EAS obtained user consent handling is not supported in the current release.

Figure 8.6.2.2.3.2-1: UE location API: Subscribe operation

1.	The EAS sends UE location subscribe request to the EES for tracking a UE's location continuously. The UE location subscribe request shall include the UE Identifier or UE group Identifier and may include proposed expiration time. It may also include location granularity to indicate requested format of the location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the EAS and location QoS.

2.	The EES checks if the request from the EAS is authorized. If authorized, the EES sends success response to the EAS and utilizes the UE location exposure capability of the 3GPP core network as specified in clause 8.10.3. The EES requests continuous location reporting from the 3GPP system to have up to date location information of the UE.

3.	The EES may subscribe to UE expected behaviour analytics (UE mobility) as described in 3GPP TS 23.288 [18].

4.	The response includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EAS shall send a UE location subscription update request prior to the expiration time. If a UE location subscription update request is not received prior to the expiration time, the EES shall treat the EAS as implicitly unsubscribed. If it is not authorized, the EES sends failure response with rejection cause.

* * * Next Change * * * *

##### 8.6.2.3.2	UE location request

Table 8.6.2.3.2-1: UE location request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | Identifies the UE for which location will be reported (e.g. GPSI) |
| Security credentials | M | Security credentials of the EAS |
| Location granularity | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the EAS |
| Location QoS | O | Indicate the location quality of service as described in clause 4.1b of 3GPP TS 23.273 [9]. |

* * * Next Change * * * *

##### 8.6.2.3.4	UE location subscribe request

Table 8.6.2.3.4-1: UE location subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| UE ID (NOTE) | O | Identifies the UE for which location will be reported (i.e GPSI) |
| UE Group ID (NOTE) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| Security credentials | M | Security credentials of the EAS |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Event reporting information | O | Event Reporting Information as specified in Table 4.15.1-1 of 3GPP TS 23.502 [3], e.g. event reporting mode. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| Location Granularity | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the EAS |
| Location QoS | O | Indicate the location quality of service as described in clause 4.1b of 3GPP TS 23.273 [9]. |
| NOTE: Either UE ID or UE Group ID shall be provided. |  |  |

* * * End of Changes * * * *