3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221653

e-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0049 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | removal of EN in clause 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-06-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The ENs in clause is “The number "65535" may be revisited in Stage 3.”. The Stage 3 of Rel-17 has been finished. The Application ID, Messaging Topic, Broadcast Area ID and Failure Cause IEs are string and encoded in JSON format in MSGin5G-1 reference point so they can identify more than 65535 different value of the related IEs. If the IEs, i.e. Application ID and Failure Cause are presents in MSGin5G-5 reference, the IEs in the L3 message are at least 2 octets so they can also identify more than 65535 different value of the related IEs. The IEs in clause 10 can be removed without action. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of EN in clause 10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN is in clause 10. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Change 1************************

10	Information Elements

10.1	Payload

The Payload Information Element carries the application payload that is transferred by the MSGin5G Service, of which the content is transparent to the MSGin5G Service.

If the message originates from an MSGin5G UE, the Payload IE is a string of maximum length that can be transported without segmentation but not more than 2048 octets.

10.2	Application ID

The Application ID Information Element identifies the Application Client on the UE or in the Application Server.

The Application ID is a string, which shall allow identifying 65535 different Application Clients. The Application ID is configured or provisioned in the Application Client or the Application Server.

10.3	Messaging Topic

The Messaging Topic Information Element indicates the topic of the message, which an interested UE or Application Server can subscribe to.

1.	The Messaging Topic IE is a string, which shall allow identiying 65535 different Messaging Topics. Allocating and populating the Messaging Topic IE is done by the Application Client or the Application Server.

10.4	Broadcast Area ID

The Broadcast Area ID Information Element identifies the service area where the Broadcast Message will be delivered.

The Broadcast Area ID IE is a string, which shall allow identiying 65535 different Broadcast Areas. The Broadcast Area ID is provisioned on the Application Client or the Application Server and is mapped by the Legacy 3GPP Message Gateway onto the Broadcast Area as used by the broadcast service in the 5GC.

10.5	Message ID

The Message ID Information Element uniquely identifies a specific MSGin5G message in the MSGin5G Service. If message delivery status report is requested by an MSGin5G message, the Message ID IE in this MSGin5G message is used by the sender of the MSGin5G message to match the message delivery status report with the original MSGin5G message. It is also used by the MSGin5G Server in aggregating the message delivery status report message delivery status reports.

The Message ID is globally unique within the MSGin5G Service and is generated by the sender of a new message.

10.6	Failure Cause

The Failure Cause Information Element indicates the the failure reason of an MSGin5G message, if this MSGin5G message can not be delivered successfully.

The Failure Cause IE is a string, which shall allow identiying 65535 different failure reasons.