3GPP TSG-SA WG6 Meeting #49-Bis-e	S6-221656

E-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on new solution for the KI#1: Architecture for the 5G-enabled fused location service

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Chunshan Xiong, chunshan.xiong@cictmobile.com

1. Introduction

This contribution provides a new solution for the KI#1.

2. Reason for Change

During the previous SA6 meetings, there is no architecture agreed upon. The new architecture is proposed based on the agreed part of solution#1 and solution#3.

It is proposed that the SEAL LMS cannot retrieve the target UE location information from the FLS via the FLS-2 interface based on the discussion in the paper S6-220872  (https://www.3gpp.org/ftp/tsg_sa/WG6_MissionCritical/TSGS6_048-e/docs/S6-220872.zip) in the SA6#480E meeting. If it is proposed to support this function, then it is proposed that the SUPL/SLP and Le interfaces are removed from the SEAL LMS and the FLS support the Le/N33/SLP interface which is proposed in the paper S6-220872 in the SA6#480E meeting.

3. Proposal

It is proposed to agree with the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change (all new texts)* * * *

7.x	Solution #1: Architecture for fused location service

7.x.1	Solution description

This solution addresses key issue #1: Architecture enhancement of application enablement for location.

7.x.1.1	Functional architecture

Figure 7.x.1.1-1 identifies the architecture of fused location service.

Figure 7. x.1.1-1: Functional architecture of fused location service

The architecture is composed of logical function modules that are not necessarily physical entities and can reside in or co-locate with existing application layer entities as appropriate.

In the architecture, the Fused Location Server (FLS) and Application Specific Server can be within the MNO domain or third-party service provider domain.

FLS fuses different location information from multiple resources and provides a better location service/information to the Application Server via its northbound API. And the SEAL LMS can be one of its location sources as described in figure 7.x.1.1-1.

The FLS needs to get the location information from other PLMNs if the target UE is with multiple PLMN accesses, in such cased, the FLS-1 and or the LM-S reference point is to provide such location information from different PLMNs.

The FLS-3 reference point is defined for storing and retrieving location information for the target UE and user profile for the target UE.

The FLS architecture supports multiple possible sources of location information including:

-	LCS location retrieved from SEAL LMS (as defined in 3GPP TS 23.434 [13]);

-	Retrieve the target UE positioning from 3rd party location server, e.g. the SLP (as defined in OMA AD SUPL [10]) of SUPL network;

-	Retrieve the target UE Positioning via the FLS-1 interface.

7. x.1.2	Functional components and reference points

The Fused Location Server provides location information of the target UE based on positioning or location data retrieved from one or multiple location sources. The Fused Location Server can get the location information from the Fused Location Client and additionally can get location information via the SEAL LMS and or 3rd party location server. The Fused Location Server selects one or more location sources, and one or more location methods based on the requested location QoS. The Fused Location Server provides a normalized description of location data to the application-specific server (e.g. of ecosystem partners) through the northbound API.

The Fused Location Client represents the client of the target UE providing the UE-based positioning and location-related information and providing the UE location information to the Fused Location Server via an IP connection.

NOTE: 	How does the Fused Location Client get its UE location information is out of the scope of 3GPP.

To retrieve the location related information directly from the Fused Location Client, the FLS also needs to support the following types of interaction with the Fused Location Client:

-	Get the UE location information

-	Provide location notification to the FLC in the target UE and get a guarantee to get location information from the target UE;

-	Install location event triggers in the FLC in the target UE to support the target UE terminated deferred location information;

The SEAL Location management client represents the client of the requestor/sender UE for location reporting as defined in 3GPP TS 23.434 [13].

The SEAL location management server is a functional entity that receives and stores user location information and provides user location information as defined in 3GPP TS 23.434 [13]. The location management server acquires location information from one or more sources including:

-	the NEF (as defined in 3GPP TS 23.501 [5] and 3GPP TS 23.273 [4]) via N33 reference point;

-	the GMLC (as defined in 3GPP TS 23.273 [4]) via Le reference point which is not defined in 3GPP TS23.434[13];

-	the SLP (as defined in OMA AD SUPL [10]) of the SUPL network which is not defined in 3GPP TS23.434[13];

-	the SEAL location management client as defined in 3GPP TS 23.434[13].

The 3rd Party Location Server provides the location of certain location technology (typically the network-based positioning).

The interfaces are described as followed:

FLS-1:	Reference point supporting location reporting, location determination, location management, and exchange of location contextual information (e.g. UE ID, location capabilities of the target UE, the available positioning methods supported by the target UE) between the Fused Location Server and the Fused Location Client of the target UE. The FLS-1 may support HTTP or WebSocket and the IP connection between the Fused Location Client and Fused Location Server.

NOTE:	The IP connection between the Fused Location Client and Fused Location Server can be provided by the 5GS PDU Session.

FLS-2:	Service-based interface exposing fused location data to the applications (e.g. the vertical applications, the applications of ecosystem partners, etc.). The FLS-2 may support HTTP or WebSocket.

FLS-3:	Reference point between the FLS and a database for storing and retrieving location information for the target UE and user profile for the target UE.

NOTE:	The definition of FLS-3 is out of scope of this specification.

FLS-4:	The reference point is used for location retrieval of the target UE from that 3rd party location server. The FLS-X can be a service-based interface. The FLS-4 may support HTTP or WebSocket.

NOTE:	The definition of FLS-4 is out of scope of this specification.

Nnef:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in 3GPP TS23.271 [3].

LM-UU:	Reference point as defined in 3GPP TS 23.434 [13].

LM-S:		Reference point as defined in 3GPP TS 23.434 [13].

NOTE:	If the UE supports MUSIM, the FLS can get the UE location information from the PLMNs via the LM-S reference of each PLMN.

7. X.2	Solution evaluation

The merged architecture defined in figure 7.x.1.1-1 is selected as the basic architecture to merge the Fused Location Server and SEAL LMS.

Based on the merged architecture, the SEAL LMS needs to upgrade to support Le interface.

Based on the merged architecture, all the solutions for other KIs are updated based on this merged architecture.