3GPP TSG-SA WG6 Meeting #49-Bis-e	S6-221658

E-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on solution#3 update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Chunshan Xiong, chunshan.xiong@cictmobile.com

1. Introduction

This contribution updates the solution#3.

2. Reason for Change

The architecture proposed in the paper S6-220872  (https://www.3gpp.org/ftp/tsg_sa/WG6_MissionCritical/TSGS6_048-e/docs/S6-220872.zip) in the SA6#480E meeting as not supported by the companies, but it is included again in the new architecture with FFS.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.3	Solution #3: Functional architecture for fused location service leveraging SEAL location management

7.3.1	Solution description

This solution leverages SEAL to address key issue #1: Architecture enhancement of application enablement for location.

7.3.1.1	Functional architecture

The figure 7.3.1.1-1 identifies the architecture of fused location service enabled by 5GS leveraging SEAL.

Figure 7.3.1.1-1: Functional architecture of fused location service leveraging SEAL location management

In the architecture, the SEAL Location management server (LMS) interacts with the Fused location function to provide location information.

The architecture supports multiple sources of location information including:

-	Location of AMF service exposed by NEF (as defined in 3GPP TS 23.502 [12]);

-	LCS location retrieved from either NEF or GMLC (as defined in 3GPP TS 23.273 [4]);

-	SUPL location retrieved from SLP (as defined in OMA AD SUPL [10]);

-	Location retrieved from 3rd party location server or location management client.

7.3.1.2	Functional components and reference points

The Location management server provides location information of the target UE based on positioning or location data retrived from one or multiple location sources.

Fused location function provides normalized description of location data to the VAS server through the API.

The 3rd Party Location Server provides the location of a certain location technology (typically the network-based positioning) e.g. Bluetooth.

The Location management client represents the client of the requestor/sender UE for location reporting as defined in 3GPP TS 23.434 [13].

The Fused location client represents the client of the sender UE for location reporting via non-3GPP access over FLS-X1 interface.

The location management server is a functional entity that receives and stores user location information and provides user location information to the vertical application server as defined in 3GPP TS 23.434 [13]. The location management server acquires location information from one or more sources including:

-	the NEF (as defined in 3GPP TS 23.501 [5] and 3GPP TS 23.273 [4]) via N33 reference point;

-	the GMLC (as defined in 3GPP TS 23.273 [4]) via Le reference point which is not defined in 3GPP TS 23.434 [13];

-	the SLP (as defined in OMA AD SUPL [10]) of SUPL network which is not defined in 3GPP TS23.434[13];

-	the location management client.

The VAL client and VAL server (vertical application layer entities) are as defined in 3GPP TS 23.434 [13].

The interfaces are described as followed:

LM-UU:	Reference point as defined in 3GPP TS 23.434 [13].

LM-S:		Reference point as defined in 3GPP TS 23.434 [13].

FLS-X2:	The reference point is used for location retrieval of the target UE from 3rd party location server.

NOTE:	The definition of FLS-X2 is out of scope of this specification.

N33:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in OMA AD MLS [11].

FLS-S:	Reference point used for fused location retrieval from Fused location function by VAL Server.

7.3.2	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.