3GPP TSG-SA WG6 Meeting #49-Bis-e	S6-221659

E-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on new solution for the KI#6: FLS Architecture for Fused Location Value Added Services

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Chunshan Xiong, chunshan.xiong@cictmobile.com

1. Introduction

This contribution provides a new solution for the KI#6: Value-Added Location Services.

2. Reason for Change

To support the value-added location services, the FLS needs the value-added location function to generate the related services based on the received UE location information.

3. Proposal

It is proposed to agree with the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change (all new texts)* * * *

7.x	Solution #1: Fused Location Server Architecture for fused location service

7.x.1	Solution description

This solution addresses key issue #6: Value-Added Location Services.

7.x.1.1	Functional architecture

Figure 7.x.1.1-1 identifies the architecture of fused location services to support the value-added location services.

Figure 7. x.1.1-1: Functional architecture of FLS for the Location Value-added Services

The Fused Location Server (FLS) architecture is composed of logical function modules that are not necessarily physical entities and can reside in or co-locate with existing application layer entities as appropriate.

In the architecture, the FLS and Application Specific Server can be within the MNO domain or third party service provider domain.

The FLF (Fused Location Function) of the FLS fuses different location information from multiple resources as described in architecture solutions and provides a better location service/information to the LVSF (Location Value-added Service Function), the LVSF provides the location-added services to the application specific server via the northbound API.

7. x.1.2	Functional components and reference points

The FLF(Fused Location Function) provides location information of the target UE based on positioning or location data retrieved from one or multiple location sources to the LVSF (Location Value-added Service Function).

LVSF provides location value-added service to the ASS(Application Specific Server) via the FLS-2 reference point.

The Fused Location Client represents the client of the target UE to support location value-added service with the FLS.

The LVSF can support none, one or more location value-added services. The LVSF can be pre-configured with a lot of location-added services, and each location-added value service is unique and identified with aanLVAS ID. The ASS requests the LVAS with one or more LVAS ID(s) and the LVSF processes the location value-added service based on the received target UE location information from the FLF. If the ASS provides no LVAS ID or the NONE LVAS ID, the LVSF just forwards the target UE location related messages between the FLF and the ASS without any location value-added processing.

The interfaces are described as followed:

FLS-1:	Reference point supporting location-value added information exchnge between the Fused Location Server and the Fused Location Client of the target UE.

FLS-2:	Service-based interface exposing fused location value-added services to the applications.

7. X.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.