3GPP TSG-SA WG6 Meeting #49-Bis-e	S6-221663

E-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on solution#1 update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Chunshan Xiong, chunshan.xiong@cictmobile.com

1. Introduction

This contribution updates solution#1.

2. Reason for Change

The FLS can get target UE location information from multiple location resources, e.g. from the FLC, SEAL LMS and 3rd party location server. The FLS needs to identify which location source is used to get the UE location information.

3. Proposal

It is proposed to agree with the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.1	Solution #1: Standalone functional architecture for fused location service

7.1.1	Solution description

This solution addresses key issue #1: Architecture enhancement of application enablement for location.

7.1.1.1	Functional architecture

The figure 7.1.1.1-1 identifies the architecture of fused location service enabled by 5GS.

Figure 7.1.1.1-1: Functional architecture of fused location service

The architecture is composed of logical function modules that are not necessarily physical entities and can reside in or co-locate with existing application layer entities as appropriate.

In the architecture, the Fused Location Server (FLS) and Application Specific Server can be within the MNO domain or third party service provider domain.

The FLS architecture supports multiple possible sources of location information including:

-	AMF location service exposed by NEF (as defined in 3GPP TS 23.502 [12]);

-	LCS location retrieved from either NEF or GMLC (as defined in 3GPP TS 23.273 [4]);

-	SUPL location retrieved from SLP (as defined in OMA AD SUPL [10]);

NOTE:	Whether and how the SUPL location service can be exposed by 5GC is within the remit of SA2.

-	Certain RAT-independent positioning retrieved from 3rd party location server or Fused Location Client.

-	Retrieve the target UE Positioning via the FLS-1 interface.

Editor's Note: It is FFS to show the relationship of Fused Location Function with SEAL-LM.

7.1.1.2	Functional components and reference points

The Fused Location Server provides location information of the target UE based on positioning or location data retrieved from one or multiple location sources. The Fused Location Server can get the location information from the Fused Location Client and additionally can get location information via the SEAL LMS. The Fused Location Server selects one or more access types, one or more location methods, and Control Plane /User Plane (e.g. SUPL [10]) methods based on the requested location QoS. The Fused Location Server provides a normalized description of location data to the application-specific server (e.g. of ecosystem partners) through the northbound API.

To retrieve location information from the Fused Location Client, the FLS also needs to support the five types of interaction with the Fused Location Client:

-	Get the UE location information;

-	Provide location notification to the FLC in the target UE and get a guarantee to get location information from the target UE;

-	Install location event triggers in the FLC in the target UE to support the target UE terminated deferred location information;

The 3rd Party Location Server provides the location of a certain location technology (typically the network-based positioning) e.g. Bluetooth.

The Fused Location Client represents the client of the target UE providing the UE-based positioning and location-related information and provides the UE location information to the Fused Location Server via an IP connection.

NOTE: 	How does the Fused Location Client get the UE location information is out of scope of 3GPP.

The NEF (as defined in 3GPP TS 23.501 [5]) exposes location service of 5GC when Fused Location Server is external to MNO domain.

The GMLC (as defined in 3GPP TS 23.273 [4]) provides LCS when Fused Location Server is within the MNO domain.

The SLP (as defined in OMA AD SUPL [10]) provides location of SUPL network.

The interfaces are described as followed:

FLS-1:	Reference point supporting location reporting, location determination, location management and exchange of location contextual information (e.g. UE ID, location capabilities ) between the Fused Location Server and the Fused Location Client of the target UE. The FLS-1 may support HTTP or WebSocket and the IP connection between the Fused Location Client and Fused Location Server

NOTE:	The IP connection between the Fused Location Client and Fused Location Server can be provided by the 5GS PDU Session.

FLS-2:	Service-based interface exposing fused location data to the applications (e.g. the vertical applications, the applications of ecosystem partners, etc.). The FLS-2 may support HTTP or WebSocket.

FLS-3:	Reference point between the FLS and a database for storing and retrieving location information for the target UE and user profile for the target UE.NOTE:	The definition of FLS-3 is out of scope of this specification.

FLS-4:	The reference point is used for location retrieval of the target UE from that 3rd party location server. The FLS-X can be a service-based interface. The FLS-4 may support HTTP or WebSocket.

NOTE:	The definition of FLS-4 is out of scope of this specification.

Nnef:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in OMA AD MLS [11].

LM-UU:	Reference point as defined in 3GPP TS 23.434 [13].

LM-S:		Reference point as defined in 3GPP TS 23.434 [13].

NOTE:	If the UE supports MUSIM, the FLS can get the UE location information from the PLMNs via the LM-S reference of each PLMN.

7.1.1.3	Merged Architecture to support interaction between FLS and SEAL LMS

FLS fuses different location information from multiple resources and provides a better location service/information to the Application Server via its northbound API. And the SEAL LMS can be one of its location sources as described in figure 7.1.1.3-1

The FLS needs to get the location information from other PLMNs if the target UE is with multiple PLMN accesses, in such cased, the FLS-1 and or the LM-S reference point is to provide such location information from different PLMNs.

The FLS-3 reference point is defined for storing and retrieving location information for the target UE and user profile for the target UE.

Figure 7.1.1.3: Merged architecture with Interaction between Fused Location Server and SEAL LMS

7.1.2	Solution evaluation

The merged architecture defined in figure 7.1.1.3-1 is the architecture to merge the Fused Location Server.

Based on the merged architecture, the SEAL LMS needs to upgrade to support Le interface.