3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221832

e-meeting, 22nd June – 1st July 2022	(revision of S6-221574)

Title:	Reply LS on user-requested priority in emergency state

Response to:	LS S6-221506 (C1-224298) on user-requested priority in emergency state

Release:	Rel-17

Work Item:	eMONASTERY2

Source:	SA6

To:	CT1

Cc:

Contact person:	Ingo Wendler

ingo.wendler@sbb.ch

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA6 thanks CT1 for their LS in S6-221506 (C1-224298) on user-requested priority in emergency state.

3GPP TS 23.280 Annex C1 clarifies the different kinds of priorities resulting to a communication priority, presentation priority and floor priority. Hence, requested priority is orthogonal to the type of communication, i.e., emergency, or imminent peril communication and is considered as a separate element that can be provided at initiation of communication. In case both the type of communication (e.g., emergency call) and the user requested priority is present at communication establishment, the resource-priority header indicates emergency and a dedicated indication the requested priority which won’t affect the type of communication, i.e., emergency. The operator policy determines how the MC server handles the indicated user requested priority.

SA6 has discussed the applicability of user-requested priority while in emergency or imminent peril state and wants to answer CT1’s questions as following:

Q1: Does the user requested priority override the emergency/peril priority?

A1: Operators policy or configuration by MC organization will determine if the requested priority overrides the type of communication priority, i.e., emergency/peril.

Q2: Does the user requested priority override the priority as indicated in the MC service user configuration?

A2: Operators policy or configuration by MC organization will determine if the requested priority overrides the priority as indicated in the user`s configuration.

# 2	Actions

To CT1

ACTION:	SA6 kindly requests CT1 to take the above information into account and make relevant changes in their specifications.

# 3	Dates of next TSG SA WG 6 meetings

SA6#50-e	22 – 31 August 2022	Online

SA6#51-e	10 – 19 October 2022	Online