3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221955

e-meeting, 22nd June – 1st July 2022	(revision of S6-221734, 1911)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0084 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to inter-RAT change report via LMC and LMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | R18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the MC service server obtains the inter-RAT information from the MC service UE via the LMS and LMC, the inter-RAT event shall be included in the triggering criteria in location report configuration and the inter-RAT event shall be included in the location information report required if the inter-system RAT changes information is expected from the LMC in case of 4G5G coexistence case. However the NOTE 3 in table 7.5.2.2-1, the NOTE 5 in table 7.5.2.3-1, the NOTE 4 in table 7.5.2.3-2 are not well reflect this. The actual inter-RAT info in the location information report is missing as the MC service server has no means to determine the RAT based on the location like Cell ID. So the serving RAT information is needed to be included as the inter-RAT change information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Update the table 7.5.2.2-1, the table 7.5.2.3-1 and the table 7.5.2.3-2 to reflect that the inter-RAT is conditional mandatory. 2) add the serving RAT to the table 7.5.2.3-1 and table 7.5.2.3-2 as the inter-RAT change information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impact the downstream group to implement the feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.2, 7.5.2.3, 7.5.3.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************* First Change **********************/

#### 7.5.2.2	Location reporting configuration

Table 7.5.2.2-1 describes the information flow from the location management server to the location management client for the location reporting configuration.

Compared to that is defined in 3GPP TS 23.280 [3], the following changes are made:

-	 Allowing the location management server to configure location reporting by inluding inter-RAT trigger information.

-	Adding a new NOTE 3 to clarify the usage of triggering event for inter-RAT changes.

Table 7.5.2.2-1: Location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user to which the location reporting configuration is targeted. |
| Requested non-emergency location information | O (see NOTE 1) | Identifies what location information is requested, except for emergency or imminent peril calls or emergency alerts |
| Requested emergency location information | O (see NOTE 1) | Identifies what location information is requested, for emergency or imminent peril calls or emergency alerts |
| Triggering criteria in non- emergency cases | O (see NOTE 1 and NOTE 3 | Identifies when the location management client will send the location report in non-emergency cases |
| Handling criteria in not reporting location information cases | O | Identifies when the location management client will store location information (e.g. never, off-network, IOPS) |
| Triggering criteria in not reporting location information cases | O (see NOTE 2) | Identifies the causes when the location management client will generate location information |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent and 0 for emergency calls, imminent peril calls and emergency alerts |
| Triggering criteria in emergency cases | O (see NOTE 1) | Identifies when the location management client will send the location report in emergency cases |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting based on Triggering criteria in emergency and non-emergency cases, if configured. NOTE 2: If not present, location information is generated based on Triggering criteria in emergency and non-emergency cases, if configured. NOTE 3: Triggering criteria can contains the inter-RAT trigger information if the inter-system RAT changes information is expected from the LMC in case of LTE eMBMS and 5G MBS co-existence. |  |  |

#### 7.5.2.3	Location information report

Table 7.5.2.3-1 describes the information flow from the location management client to the location management server for the location information reporting.

Compared to that is defined in 3GPP TS 23.280 [3], the following changes are made:

-	Allowing the location management client to report the the inter-RAT information in the location information report;

-	Adding a new NOTE 5 to clarify the usage of triggering event for inter-RAT changes

Table 7.5.2.3-1: Location information report (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID (see NOTE 4) | O | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3 and NOTE 5) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2 and NOTE 5) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, MBS FSA ID(s), with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: In case of an on-demand request of an MC service user the MC service ID shall be provided. In case of an MC service server request or an event-triggered report, no MC service ID is provided. NOTE 5: Triggering event can contain the inter-RAT information if the inter-RAT changes information is expected from the LMC in case of LTE eMBMS and 5G MBS co-existence. |  |  |

Table 7.5.2.3-2 describes the information flow from the location management server to the MC service server for location information reporting.

Compared to that is defined in 3GPP TS 23.280 [3], the following changes are made:

-	Adding a new NOTE 4 to clarify the usage of triggering event for inter-system RAT changes

Table 7.5.2.3-2: Location information report (LMS – MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the MC service ID. |
| Triggering event (see NOTE 3 and NOTE 4) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2 and NOTE 4) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, MB FSA ID(s), with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: Triggering event can contain the inter-RAT information if the inter-RAT information is expected from the LMC in case of LTE eMBMS and 5G MBS co-existence. |  |  |

# /******************* Next Change **********************/

#### 7.5.3.2	Event-triggered location reporting procedure

NOTE 1:	This procedure is valid for single MC system operation only.

The location management server provides location reporting configuration to the location management clients, indicating what information the location management server expects and what events will trigger the sending of this information to the location management server. For eMBMS and MBS co-existing, the MC service server needs to obtain the inter-RAT information. If the inter-RAT information report from the MC service UE is expected, the MC service server may obtain the inter-RAT information from the location management server, and the location management server then performs the procedure as described clause 10.9.3.1 in 3GPP TS 23.280 [3], clause 7.5.2.2 and 7.5.2.3. . In addition to those trigger criteria defined in 3GPP TS 23.280 [3], the new inter-RAT triggers may include, the change of MBS FSA ID, inter-system RAT type changes. The location information report contains the inter-RAT information.

NOTE 2:	How the location management client can identify or obtain information about the case that an inter-RAT change has occurred is implementation specific or out of the scope of this specification.

Similar to the usage of eMBMS bearer for transmitting of location reporting configuration defined in TS 23.280, the location reporting configuration message can also be sent over 5G MBS Session.

# /******************* End of Change **********************/