3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221957

e-meeting, 22nd June – 1st July 2022	(revision of S6-221830)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0105 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL Notification Management service – Information Flows and Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-06-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Recently in 23.700-98 the following EN is added as part of the Edge Notification service architecture : Editor's Note: Whether ENS should be addressed as a generic function provided as part of SEAL is FFS. Many of the applications requires notification functionality and its becoming a more common functionality. The 3GPP document TS 23.434 has identified many set of common services (e.g., group management, configuration management, location management) which is used by many verticals and provided a functional architecture for SEAL and the procedures, information flows and APIs for each service within SEAL in order to support vertical applications over the 3GPP system. This CR aims to make the notification mechanism as SEAL service so that it can be used by verticals and thus offloading the complexity notification logic from the enabler layer |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces notifiction management as seal service and in particular this CR introduces information flows and procedures for the notification management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Notification service needs to be implemented at every VAL layer which requires notification functionality which is redundant and waste of effort. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.3(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 17.3	Procedures and information flows for notification management

### 17.3.1	General

This sub-clause describes the procedure and information flows for notification management service. The notification management procedures apply to on-network VAL service only.

#### 17.3.2	Information flows for notification management

#### 17.3.2.1	Create notification channel request

Table 17.3.2.1-1 describes the information flow create notification channel request from the notification management client to the notification management server.

Table 17.3.2.1-1: Create notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the requesting notification management client |
| Channel type | M | Indicates PULL or PUSH method to be used for delivering the notification messages |
| PUSH channel details (see NOTE) | O | Carries the details of the type of PUSH delivery and its associated data |
| Validity Duration | M | How long the notification channel will last (i.e. channel lifetime) as requested by the notification management client. |
| NOTE : This IE shall be present of the channel type is PUSH |  |  |

#### 17.3.2.2	Create notification channel response

Table 17.3.2.2-1 describes the information flow create notification channel response from the notification management server to the notification management client.

Table 17.3.2.2-1: Create notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Notification URL | O | The URL to receive the notification message if a PULL method is requested. For some PUSH method implementation (such as WebSockets) this URL is used to start the PUSH notification service from the notification management server |
| Callback URL | O | The URL to be shared to the VAL server by the VAL client for sending the notifications to the notification management server |
| Validity duration | O | How long the notification channel will last (i.e. channel lifetime) as granted by the notification management server. |
| Channel ID | M | Unique identifier for the created notification channel |

#### 17.3.2.3	Open notification channel

Table 17.3.2.3-1 describes the information flow open notification channel from the notification management client to the notification management server.

Table 17.3.2.3-1: Open notification channel

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Notification URL | M | The URL to receive the notification message and it is same as received in the Create notification channel response |
| Channel ID | M | Identifies the notification channel and the value shall be same as returned in Create notification channel response |

#### 17.3.2.4	Notification message

Table 17.3.2.4-1 describes the information flow notification message from VAL server to the notification management server and from notification management server to the notification management client.

Table 17.3.2.4-1: Notification Message

| Information element | Status | Description |
| --- | --- | --- |
| VAL Application ID | M | Identity of the VAL application residing in the VAL UE this notification message is targeted |
| VAL Service ID | M | Identity of the VAL service for which this notification is generated |
| Notification data | M | This information element carries the details of the notification data such as type of the event and the data corresponding to the event |

### 17.3.3	Procedure for creating notification channel to receive notifications

Figure 17.3.3-1 below illustrates the procedure for receiving notifications from the notification management server by the notification management client.

Pre-conditions:

1.	The notification management client does not have any notification channel open with the notification management server.

Figure 7.13.3-1: Receiving notifications from notification management server

1.	The VAL client requests the notification management client to subscribe to and receive notifications from the VAL server.

2.	The notification management client creates notification channels (i.e. endpoint URLs), if not created already, to be used by the VAL server to send notification messages. To create notification channel, the notification management client sends a create notification channel request to the notification management server. The desired validity duration for the channels to be used and the notification channel type (PUSH or PULL) are included in the request. The notification management client decides which type of the notification channel to be used based on the UE capability.

3.	The notification management server authenticates the notification management client and authorizes its request.

4.	The notification management server sends the notification management client the Create notification channel response with the endpoint URLs that will be used by the VAL server to send the notification messages and the notification management client to receive the notification messages. The notification management server also includes what is the valid duration for these endpoint URLs to be used and the channel ID to uniquely identify the channel created in the response.

5.	If the notification type is PULL method, the notification management client sends the Open notification channel to the notification management server to start receiving the notification message. For certain PUSH method notification type (such as WebSockets) the notification management client requests the notification management server to start the PUSH notification service with its specific protocol that is outside the scope of this specification.

NOTE 1: This step is not required if the notification client wants to receive the notification messages via PUSH notification server.

6.	The notification management client responds to the VAL client the status of its request as in step 1 to receive notifications from the VAL server. It also includes the callback URL which needs to be used by the VAL client while subscribing to the VAL server for receiving notifications.

7.	The VAL client subscribes to the VAL server for receiving notifications. This request includes the subscription data and the callback URL returned from the notification management server in step 4.

NOTE 2: This step is outside the scope of this specification.

8.	The VAL server generates the notification message for the subscription request from the VAL client.

9.	The VAL server sends the notification message to the notification management server containing the VAL UE or VAL user ID, VAL service ID, VAL application ID and the notification data.

10.	If the delivery method is PULL, the notification management server sends the notification message to the notification management client over the opened notification channel.

NOTE 3: If the delivery method is PUSH via PUSH notification server, the notification management server sends the notification message to the OEM Push server (not shown in the figure and is outside the scope of this specification) to deliver to the notification management client.

11.	The notification management client delivers the received notification message to the appropriate VAL client based on the details as received in the notification message from the notification management server.

Editor's note: How to support different types of PULL and PUSH mechanisms for delivery of notification data is FFS.

* * * End Change * * * *