3GPP TSG-SA WG6 Meeting #50-e	S6-222132

E-meeting, 22th – 31th August 2022

Source:	CATT

Title:	Pseudo-CR on Solution#5 Update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Liping Wu, liping.wu@cictmobile.com

1. Introduction

This contribution updates the solution #5: Location profiling for supporting fused location service enablement.

2. Reason for Change

According to the new architecture of KI#1, the Fused Location Client has been removed and replaced by Location Management Client(LMC), and the Fused Location Function(FLF) is defined to be part of the Location Management Server(LMS) and used as an additional source for location information to enhance the LMS. So the procedure of location profiling for fused location derivation and exposure needs to be updated accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.5	Solution #5: Location profiling for supporting fused location service enablement

7.5.1	Solution description

7.5.1.1	General

The solution discusses the creation of Location profiles at a fused location service at the application enablement layer; and the mapping of Location profiles to one or more vertical applications. Such profiling is based on a variety of factors which correspond to use-case / vertical driven hybrid positioning requirements and policies. This notion of profiling (which is different from the service profile) is based on the various factors which may affect the positioning methods and QoS, and fixes also the communication parameters per vertical requirement-use case, without providing any additional information at the 3rd party app. The attributes that can be used for the Location profile can be some of the following (however how the Location profiles are constructed can be up to implementation):

-	the vertical / application service type;

-	the environment (indoor/outdoor, urban/suburban,);

-	the QoS requirements, e.g. accuracy;

-	the capabilities of the UEs involved;

-	the energy constraints for the devices;

-	preference on certain positioning methods, e.g. RAT-dependent or RAT-independent methods;

-	LCS service level (in case of IIOT vertical);

-	priorities of location methods;

-	whether location augmentation is required;

-	whether location prediction is required;

-	whether sidelink positioning assistance is required;

-	whether proximity-based location estimate is required;

-	whether location verification is required.

An example Location profile can be shown in Table 7.5.1.1-1 below.

Table 7.5.1.1-1: Exemplary location profile attributes

| Profile ID / name | Vertical / use case, environment | Positioning Service Level (for IIOT) / QoS / accuracy | Positioning Method(s) / Priorities | Involved 3GPP functionalities / Priorities | Involved non-3gpp access networks | Required APIs / API info | Other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location profile #1 | Industrial scenario, indoors, mobile robots/ AGVs | Service Level 6 / cm level accuracy / absolute/relative/ both | 1. DL-TDOA, 2. UL-TDOA, 3. Multi-RTT methods, 4. WLAN, 5. motion sensors, 6. Bluetooth | 1. LMF, 2. RAN-LMC, 3. SEAL LMS, .. | 4. WLAN ID,... | NEF APIs, SEAL APIs, .. | Verification / augmentation required |
| Location profile #2 | V2X, outdoor, .. | Decimeter level accuracy /... absolute/relative/both | 1. DL-TDOA, 2. Multi-RTT methods, 3. GNSS-RTK, 4. Sensor fusion, 5. A-GPS | 1. LMF, 2. SEAL LMS, 3. Other UEs | 3. GNSS #x, #y, 4. MEC #x | NEF APIs, MEC APIs, .. | Support for sidelink positioning |

Without this solution, app server needs to consolidate all measurements and interact with different systems to get the required location. The profiling helps the optimization of the process based on the environment, UE context, etc., per profile checking/monitoring. Also, the configuration of combined positioning methods to meet the LCS requirements, done with minimum exposure to the 3rd party/customer.

7.5.1.2	Procedure

The procedure includes the translation of the vertical request to a Location profile and the procedures with the involved entities to derive the requested location report. The fused location function/client may also fetch location reports in an iterative manner based on method priorities, to ensure that the vertical requirement is met, with the minimum signaling/complexity.

Figure 7.5.1.2-1 illustrates a solution for the location profiling for supporting fused location exposure.

Pre-conditions:

1.	The VAL server has registered to receive fused location function services.

Figure 7.5.1.2-1: Location profiling for fused location derivation and exposure

1.	The Fused Location Function(FLF) which is part of the Location Management Server(LMS) configures a set of location service profiles, where each location service profile includes metrics like the positioning method, QoS parameters, location service producers involved, environment/ area type, etc.

NOTE1:	How the profiling attributes are selected/configured is up to implementation.

2.	The FLF receives a location request from a fused location service consumer (VAL server), where this request may include a VAL server ID, location QoS requirements (accuracy, response time,..), location granularity (coordinates, cell-level, civic addresses, topological location), vertical specific support information (planned route, road maps,...), time validity for the requirement, area of validity, event triggering criteria (under which criteria the location report needs to be sent), etc.

3.	The fused location enabler function determines a mapping of the fused location service consumer (VAL server) to a location service profile based on the location request and the information provided within the request.

4.	The FLF sends to the Location Management Client (LMC) optionally the involved 3gpp functions and the configuration of the mapping of the application to a Location profile. This may include the report configuration per Location profile (thresholds for event triggering, periodicity of reporting, format of reporting, minimum time between consecutive reports) as well as the priority of positioning methods and location report granularity (coordinates, cell-level, civic addresses, topological location).

5a. The FLF requests from the LMC a local fused location estimate of the target VAL UE or for the UEs within the application in close vicinity.

5b. The LMC sends to the FLF a local fused location estimate based on the request.

6.	The FLF within the LMS performs a location request to one or more of the following (based on the Location profile):

-	to SEAL LMS, acting as an internal function module, to request the location information for one or more UEs. Such reporting trigger may also be enhanced to indicate the Location profile ID, as well as to indicate the locations of all the UEs with this Location profile in this area.

-	to GMLC directly or via NEF (see TS 23.273), acting as AF. The LCS service request is sent to GMLC or AMF, via NEF using the service-based interface or CAPIF API; or directly to GMLC if allowed to (e.g. fused location function is within the MNO trust domain).

-	to 3rd party location servers as described in KI#1.

7.	The FLFcalculates the fused location estimated based on combining location reports from previous steps, and may also perform additional processing e.g. for location augmentation, and verification based on the Location profile. Then, it checks whether the estimate fulfills the Location profile requirement (based on QoS parameters such as accuracy, response time).

8-9.	If the requirement is not fulfilled, the FLFiteratively requests further location information and re-checks whether the requirement is met or not.

10.	The FLF sends the fused location report to the VAL server.

7.5.2	Solution evaluation

This solution is based on the architecture proposed in the new solution of KI#1. The Fused Location Function (FLF) which is part of the Location Management Server(LMS) could enable the translation/mapping of the vertical request to a location profile, derive the requested location report, and then fetch the aggregate/fused location data from more data sources (such as SEAL LMS, GMLC, 3rd party location servers, etc.) in an iterative manner to ensure that the vertical requirement is met and send the final fused location report to the VAL server at last.