3GPP TSG-SA WG6 Meeting #50-e	S6-222372

e-meeting, 22nd – 31st August 2022	(revision of S6-222206)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0064 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | security credentials IE in Message Gateway registration aligned with 33501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The security mechanism between Message Gateway and MSGin5G Server is TLS as specified in Annex Y.5 of TS33.501.The authorization and authentication between Non-MSGin5G UE and Message Gateway is out of scope of TS23.554. So the security credentials IE in clause 8.2.3 and clause 8.2.4 are not needed in stage 2. The only change in this CR is the security credentials IE. Other changes in these clauses is agreed in S6-221959. This CR is proposed based on S6-221959. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on clause 8.2.3 and 8.2.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE credential information IE is not needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3 and 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: The reference of TS33.501 is moved to NOTE. |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.2.3	Non-MSGin5G UE Registration

Non-MSGin5G UEs (i.e., Legacy 3GPP UEs or a Non-3GPP UEs) are connected to the MSGin5G Server through a Message Gateway. The Message Gateway performs registration with the MSGin5G Server on behalf of the Non-3GPP UEs, based on the registration request from the Non-3GPP UE or on pre-provisioned information. After the procedure is complete, the Message Gateway may communicate the result to the UE to enable MSGin5G services at the Non-3GPP UEs.

The signalling flow is illustrated in figure 8.2.3-1.

Pre-conditions:

1.	The Message Gateway has been pre-provisioned with the MSGin5G Server address and UE Service ID for a Non-3GPP UE.

2.	Both the Non-3GPP UE and Message Gateway have been configured with the necessary credentials to enable authentication and Non-3GPP UE registration at the Message Gateway.

3.	A secured connection has been established between the Message Gateway and the MSGin5G Server.

Figure 8.2.3-1: Non-MSGin5G UE registration

1.	The Non-MSGin5G UE registers at the Message Gateway. Alternatively, the Non-MSGin5G UE registration information is configured at the Message Gateway. The Message Gateway, based on the registration request and pre-provisioned information, registers the UE with the MSGin5G Server, a corresponding UE Service ID and, if available, a Non-MSGin5G UE profile.

Table 8.2.3-1: Non-MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting Non-MSGin5G UE. |
| GW Service ID | M | GW service identifier of the Message Gateway performing registration on behalf of a Non-MSGin5G UE |
| Non-MSGin5G UE Profile | O | Set of parameters describing the Non-MSGin5G UE |
| >Non-MSGin5G UE Communication Availability | O | Communication availability information for the Non-MSGin5G UE to receive messages. This IE informs the MSGin5G Server if the UE has a specific application-level schedule/periodicity to its MSGin5G communications, which may be used to determine whether and when MSGin5G communications are attempted. See Table 8.2.1-3. |

2.	The Message Gateway sends the Non-MSGin5G UE registration request to the MSGin5G Server. The request includes the information detailed in Table 8.2.3-1.

3.	Upon receiving the request, the MSGin5G Server initiates authentication procedures with the Message Gateway on behalf of the Non-MSGin5G Client and  and authorises the Non-MSGin5G UE to receive the MSGin5G service. The MSGin5G Server records the UE's availability for MSGin5G service.

NOTE: The authentication procedures in step 3 are built on top of the transport layer mechanism specified in Annex Y.2 of 3GPP TS 33.501 [16].

4.	The MSGin5G Server returns the result of the registration in the Non-MSGin5G UE registration response message with the information detailed in table 8.2.2-2.

8.2.4	Non-MSGin5G UE De-registration

The Message Gateway performs de-registration with the MSGin5G Server on behalf of the Non-MSGin5G UEs, in order to terminate services from the MSGin5G Server. After the procedure is complete, the Message Gateway communicates the result to the requesting UE.

The procedure assumes that the Non-MSGin5G UE is responsible for initiating the de-registration from the MSGin5G Server. The signaling flow for Non-MSGin5G UE de-registration is illustrated in figure 8.2.4-1.

Pre-conditions:

1.	The Message Gateway successfully performed registration with the MSGin5G Server on behalf of the Non-MSGin5G UE.

Figure 8.2.4-1 : Non-MSGin5G UE de-registration

1.	The Message Gateway determines to de-register the Non-MSGin5G UE with the MSGin5G Server.

2.	The Message Gateway sends a Non-MSGin5G UE de-registration request  to the MSGin5G Server that includes the UE Service ID associated with the Non-MSGin5G UE, as shown in Table 8.2.4-1.

Table 8.2.4-1: Non-MSGin5G UE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the Non-MSGin5G UE. |

3.	Upon receiving the request, the MSGin5G Server may initiate authentication procedures with the Message Gateway on behalf of the Non-MSGin5G Client. The MSGin5G Server deletes any applicable MSGin5G Client Profile information that it has stored.

NOTE: The authentication procedures in step 3 are built on top of the transport layer mechanism specified in Annex Y.2 of 3GPP TS 33.501 [16].

4.	The MSGin5G Server deletes any applicable information that it has stored and replies with a Non-MSGin5G UE de-registration response as shown in table 8.2.4-2.

Table 8.2.4-2: Non-MSGin5G UE De-registration response

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the Non-MSGin5G UE. |
| De-registration result | M | Indication if the de-registration is success or failure |

Editor's Note:	Support for bulk registration and de-registration of Non-MSGin5G UE s (i.e., registering more than one UE Service ID at the same time) is FFS.