3GPP TSG-SA WG6 Meeting #50-e	S6-222091

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

Source:	Ericsson

Title:	ACR scenario selection in DN

Spec:	3GPP TR 23.700-98 v1.1.1

Agenda item:	9.8

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution provides a solution for KI#19.

2. Reason for Change

The selection of ACR scenarios made by EES was mentioned in sol#19 and selection of ACR scenario made by EEC was mentioned in sol#35. They are suitable in EEC triggered initial EAS discovery and EEC triggered T-EAS discovery during ACR.

There is no solution provided for the DN side triggered T-EAS discovery during ACR, where S-EAS and S-EES communicate with each other to determine T-EAS.

During the T-EAS discovery, the T-EAS service continuity support capability (e.g. supported ACR scenarios) is considered together with the service contitnuity support capability of AC, EEC and EES. Then ACR scenario selection can be further decided between S-EAS and S-EES and final result needs to be notified to the EEC and T-EAS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TR 23.700-98.

Rev1: removed ACR scenario description and focus on interaction triggered by EAS.

* * * First Change * * * *

## 7.X	Solution #X: ACR scenario selection in DN

### 7.X.1	Architecture enhancements

None.

### 7.X.2	Solution description

#### 7.X.2.1	General

This solution provides options for the S-EAS or S-EES to make ACR scenario selection.

#### 7.X.2.2	S-EAS decided ACR scenario selection

Figure 7.X.2.2-1: S-EAS decided ACR scenario selection

In Figure 7.X.2.2-1, the S-EAS, after determines the T-EAS in step 1, decides the ACR scenario to be used in the subsequent ACR based on the supported ACR scenarios by the selected T-EAS. The S-EAS includes the selected ACR scenario list in the Selected T-EAS declaration request to the S-EES in step 2. The S-EES responds the S-EAS and further notifies EEC. The EEC stores the selected ACR scenarios to be used. The S-EES also notifies T-EES (if EES is changed) about the selected ACR scenario list. The T-EES responds the S-EES and further notifies T-EAS. Both T-EES and T-EAS stores the selected ACR scenarios to be used for the subsequent ACR.

#### 7.X.2.2	S-EES decided ACR scenario selection

Figure 7.X.2.2.-1: S-EES decided ACR scenario selection

In Figure 7.X.2.2-1, the S-EAS, after determines the T-EAS, request the S-EES to select the ACR scenario to be used in the subsequent ACR including the selected T-EAS information in step 1.

The S-EES, based on the supported ACR scenarios by the selected T-EAS (as indicated by the S-EAS), the supported ACR scenarios by the selected T-EES (as deduced from the selected S-EAS), the supported ACR scenarios by the AC and EEC (e.g. received during initial EAS discovery), selects ACR scenarios and responds the S-EAS in step 2. The S-EES further notifies EEC. The EEC stores the selected ACR scenarios to be used. The S-EES also notifies T-EES (if EES is changed) about the selected ACR scenario list. The T-EES responds the S-EES and further notifies T-EAS. Both T-EES and T-EAS stores the selected ACR scenarios to be used for the subsequent ACR.

### 7.X.3	Solution evaluation

This solution addresses KI#19 in the S-EAS or S-EES decided ACR scenario which allows the S-EAS or the S-EES to determine the ACR scenario(s) to be used for the subsequent ACR.

* * * End of Changes * * * *