3GPP TSG-SA WG6 Meeting #52	S6-230457

Toulouse, France 14th – 18th November 2022	(revision of S6-230338, S6-230226)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0159 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flow for VAL server provisioning to the Identity Management Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For VAL Server provisioning to SEAL Identity Management Server procedure, following EN needs to be removed: Editor's note: Information flows are FFS. Also, API needs to be added for VAL Server provisioning to SEAL Identity Management Server procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added IE table for the procedure and removed EN Added API for the procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Information Elements for the procedure will not be available and stge 3 will not be able to implement it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.2.1 (NEW), 12.3.2.2 (NEW), 13.3.4.2, 12.4.1, 12.4.3 (NEW), 12.4.3.1 (NEW), 12.4.3.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 12.3.2.1	VAL server provisioning request

Table 12.3.2.1-1 describes the information flow from the VAL server to the identity management server for providing provisioning configuration.

Table 12.3.2.1-1: VAL server provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL service specific information | M | Provides the list of VAL service specific information |
| > VAL service ID | M | Identify of the VAL service for which the configuration information is provided. |
| > identity list | M | Identify list of VAL users for the specific VAL service (i.e. VAL User IDs or VAL UE IDs) |

#### 12.3.2.2	VAL server provisioning response

Table 12.3.2.2-1 describes the information flow from the identity management server to the VAL server as a response for providing provisioning configuration.

Table 12.3.2.2-1: VAL server provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * Next Change * * * *

#### 12.3.4.2	Procedure

The procedure for VAL server to provision required information to SEAL identity management server in order to support VAL user authentication is illustrated in figure 12.3.4.2-1.

Figure 12.3.4.2-1: VAL Server provisioning to SEAL Identity Management Server

1.	The VAL server sends a request message to identity management server to provision required information. The request message includes identity of the VAL server, security credentials of the VAL server, and service provider specific information like identity list per VAL service.

2.	Upon receiving the request, the identity management server authorizes the request based on the security credentials provided in the request and considering the service level agreement between VAL service provider and SEAL service provider. If VAL server is authorized to use the SEAL service, then the identity management server stores the details about the VAL server including the list of VAL user IDs per VAL service.

The identity management server sends the response message to the VAL server.

Editor's note:	Whether the VAL server registers with Identity Management Server is FFS.

* * * Next Change * * * *

### 12.4.1	General

Table 12.4.1-1 illustrates the SEAL APIs for identity management.

Table 12.4.1-1: List of SEAL APIs for identity management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_IdmParameterProvisioning | Provide_Configuration | VAL server | Request /Response |

* * * Next Change * * * *

### 12.4.3	SS_IdmParameterProvisioning API

#### 12.4.3.1	General

API description: This API enables the VAL server to provision configuration for the VAL service to the SEAL IM-S.

#### 12.4.3.2	Provide_Configuration operation

API operation name: Provide_Configuration

Description: Provisioning of VAL service configuration to IM-S.

Known Consumers: VAL server.

Inputs: See subclause 12.3.2.1

Outputs: See subclause 12.3.2.2

See subclause 12.3.4.2 for the details of usage of this API operation.

* * * End of Change * * * *