3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230462

e-meeting, 11th – 20th January 2023	(revision of S6-230324, S6-230038)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0155 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New AC-EEC procedure to invoke UE ID request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Qualcomm, China Mobile, KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new procedure is needed to support the retrieval of the UE ID for AC’s (and hence EAS’s) dealing with IPv4 NATed IP address issue. That is when EAS’s direct invocation of UE Identifier API procedure (i.e. Eees_UEIdentifier API) due to NATed IP address fails to identify the UE then the EAS may use the AC to ask the EEC for the UE ID. Therefore, this procedure is provided to enable the AC interact with the EEC to obtain the UE ID and behind the scene pass on the UE ID to the EAS. In turn the EEC would rely on its interaction with the EES to obtain the UE ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clause 8.14.1 & 8.14.2.1 to list the new UE ID retrieval procedure. Also, adding the details of the new procedure (Edge UE ID request) in the new clause 8.14.2.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retrival of the UE ID corresponding to the UE’s NATed IP address is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.1, 8.14.2.1, 8.14.2.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: It adds information flow clauses related to the the new procedure (i.e. 8.14.3.x & 8.14.3.y). It also adds an EN to clause 8.14.1 & 8.14.2.1 & 8.14.x. while making some editorial changes. Rev 2: removed “Registration ID” IE from the Edge UE ID request throughout the CR. Changed EN re Edge UE ID and added a EN re preserving privacy of the UE |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

## 8.14	EDGE-5 APIs

### 8.14.1	General

EEC exposes EDGE-5 APIs corresponding to EEC's capabilities, for the AC to request EEC's services for edge enablement. Using these APIs, ACs request the EEC for EEL services. EDGE-5 APIs include one-time request/response operations for EAS discovery, retrieval of Edge UE ID and ACR operations. Additionally, the AC can request for an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required.

NOTE 1:	Details on how the AC and EEC communicate with each other is out of scope.

NOTE 2:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

Editor's note:	[SA3] How to ensure of user's authorization/consent as well as AC’s authorization in invoking functions exposed by EEC (to AC) which in turn relies on functions exposed by the network (e.g. Location) via EES/NEF is SA3’s responsibility.

### 8.14.2	Procedures

#### 8.14.2.1	General

Following procedures are specified for EDGE-5:

-	Registration;

-	EAS discovery;

-	ACR request;

-	EEC services subscription; and

-	Edge UE ID request.

Editor's note:	The definition of EDGE UE ID is FFS.

# * * * Change 2 * * * *

#### 8.14.2.x	Edge UE ID request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 8.14.2.x-1: Edge UE ID request procedure

1.	The AC sends an Edge UE ID request to the EEC. The request includes AC's security credentials and the EAS ID list for which the AC needs to know the associated AF-specific UE ID(s).

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC process the request from the AC. The EEC uses the UE Identifier API procedure (see clause 8.6.5) to query the EES.

NOTE 1:	This procedure may be used to support the retrieval of the Edge UE ID for AC’s (and EAS’s) dealing with IPv4 NATed IP address issue (i.e. EAS’s direct invocation of UE Identifier API procedure due to NATed IP address fails to identify the UE). Under such a scenario then it is understood that, the AC through application signalling which is outside the scope of this document, would pass on the the Edge UE ID (received in step 3) to the EAS so that it can be used to invoke capability APIs over EDGE-3 or EDGE-7.

Editor's note:	The use of this procedure for other use cases such as preserving the privacy of the UE is FFS.

# * * * Change 3 * * * *

#### 8.14.3.x	Edge UE ID request

Table 8.14.3.x-1 describes information elements of the Edge UE ID request sent by the AC to the EEC.

Table 8.14.3.x-1: Edge UE ID request

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID list | M | List of EAS ID(s) the AC requires to know the associated AF-specific UEID(s) for. |
| Security credentials | M | Security credentials of the AC sending the Edge UE ID request. |

Editor's note:	[SA3] How to ensure EAS ID list IE only contains the EAS IDs that are indeed associated with the AC requesting the Edge UE ID(s), and hence preventing UE ID harvesting by a rogue AC (i.e. AF-specific UE ID) is FFS.

#### 8.14.3.y	Edge UE ID response

Table 8.14.3.y-1 describes information elements of the Edge UE ID response sent by the EEC to the AC.

Table 8.14.3.y-1: Edge UE ID response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the Edge UE ID request was successful. |
| > Edge UE ID list | M | List of all the AF-specific UE IDs the AC asked for in the request and is authorized to use as per EAS ID(s). |
| >> Edge UE ID | M | AF-specific (i.e. EAS-specific) UE ID |
| >> EAS ID | M | Associated EAS ID |
| Failure response | O | Indicates that the Edge UE ID request failed. |
| > Cause | M | Provides the cause for Edge UE ID request failure. |

# * * * End of changes * * * *