3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230483

e-meeting,  11th – 20th January 2023		(revision of S6-230412)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0149 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location service registration and initialization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion of KI#5 in TR 23.700-96, the solution#4 will be considered in the normative phase. The related function, procedure and information flow will be specified in TS 23.434 to support the solution#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the procedure and information flow for location service registration according to the conclusion of KI#5 in TR 23.700-96. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G-enabled fused location service capability will not support Location service registration for SEAL location management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x,9.3.2.x(new),9.3.2.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3	Procedures and information flows for Location management (on-network)

9.3.x	Location service registration procedure

Before the Location Management Server requesting the location information for the target UE, the Location Management Client may register the available location services to the LM Server to report the UE’s location capabilities.

Figure 9.3.x-1 illustrates the procedure of client-triggered location service registration.

Figure 9.3.x-1: Location service registration procedure

1.	The LM Client of a VAL UE sends location service registration request to the LM Server, with the identifier of the UE (e.g. GPSI) and UE-based location capabilities.

2.	The LM Server checks authorization for the VAL UE's registration request.

3.	After successful authorization, LM Server sends location service registration response to the LM Client and stores the UE identifier information and location capabilities.

* * * Next Change * * * *

9.3.2	Information flows for location information

9.3.2.x	Location service registration request

Table 9.3.2.x-1 describes the information flow from the location management client to the location management server for registration of the location service.

Table 9.3.2.x-1: Location service registration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Location capability | O | The information of location capability of VAL UE for which the location service is registered. |
| >access type | O | Identity of the available access type of the VAL UE. |
| >positioning method | O | Identity of the available positioning methods of the VAL UE. |

* * * Next Change * * * *

9.3.2.y	Location service registration response

Table 9.3.2.y-1 describes the information flow from the location management server to the location management client for the location service registration response.

Table 9.3.2.y-1: Location service registration response

| Information element | Status | Description |
| --- | --- | --- |
| Registration status | M | It indicated the registration result. |

* * * End of Changes * * * *