3GPP TSG-SA WG6 Meeting #52-BIS-e	S6-230486

e-meeting, 11th – 20th January 2023	(revision of S6-223351, 230249, 437)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0328 | rev | 4 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ad hoc group call procedures involving multiple MC systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Ad hoc group communication has been studied in TR 23.700-76 and for the ad hoc group communication procedures are agreed on the basis of solution#2 with the following conclusions: i. Information flows specific to Ad hoc group calls will be used. ii. Preconfigured group as specified in Solution #2 to be used for achieving the end-to-end security for both the cases where the participants list is supplied by the initiator of the AHGC and participants list is determined by the MC Service server. iii. Preconfigured group to be used for the AHGC is determined by the MC Service server and not by the initiating client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedures for the case where participants list is supplied by the initiator is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group call feature involving multiple MC systems will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3 (new), 10.19.3.2 (new), 10.19.3.2.x (new), 10.19.3.2.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.19.3	Procedures

* * * Next Change * * * *

#### 10.19.3.2	Ad hoc group call involving multiple MC systems

##### 10.19.3.2.x	Procedure for ad hoc group call setup – Participants list provided by the Initiator

Figure 10.19.3.2.x-1 illustrates the procedure for ad hoc group call setup procedure.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system shares their users' information to the primary MC system.

2.	The authorized MCPTT user/dispatcher belongs to the primary MC system.

3.	The MCPTT server of the primary MC system is where the authorized MCPTT user/dispatcher creates the ad hoc group.

4.	Some users of the ad hoc group may belong to partner MC systems.

5.	The preconfigured group identity and preconfigured group configuration to be used for an ad hoc group have been preconfigured in the MCPTT client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

Figure 10.19.3.2.x-1: Ad hoc group call setup

1.	The MCPTT client of the authorized user initiates an ad hoc group call with multiple users from primary and partner MC systems. An ad hoc group call request message with the information of the MCPTT IDs is routed to the MCPTT server of the primary MC system.

2.	The MCPTT server of the primary MC system forms the ad hoc group by using MCPTT users' information received in step 1 and further determines the preconfigured group to be used for the configuration of the ad hoc group based on the MCPTT users' information received in step 1. The MCPTT server assigns a MCPTT group ID for the newly formed ad hoc group. It identifies the appropriate MCPTT server responsible for the MCPTT users of the ad hoc group. The MCPTT users are automatically affiliated to the ad hoc group.

NOTE 1:	The newly formed ad hoc group information including the MCPTT group ID and the list of users is held in dynamic data in the MCPTT server.

3.	The MCPTT server shall send the ad hoc group call request return message to the MCPTT client containing the below:

i.	The MCPTT ad hoc group ID generated by the MCPTT server (only included when the ad hoc group call is authorized);

ii.	The group ID of the pre-configured group to be used for the ad hoc group call (only included when the ad hoc group call is authorized); and

iii.	Result of whether the ad hoc group call is authorized or not.

If the ad hoc group call request is not authorized, the MCPTT client shall not proceed with the rest of the steps.

4.	The MCPTT server of the primary MC system sends the ad hoc group call request to the affiliated group members of the ad hoc group belonging to the primary MC system.

5.	The MCPTT clients receive in the ad hoc group call request the information of the MCPTT group ID for the ad hoc group and further notify their corresponding MCPTT user. The affiliated group members of the ad hoc group of the primary MC system may accept or reject the call and respond with the group call response.

6.	The primary MCPTT server further initiates an ad hoc group call request message to the MCPTT users of the partner MC system. The ad hoc group call request message is routed to the MCPTT users via the MCPTT server of the partner MC system.

7.	The MCPTT clients receive in the ad hoc group call request the information of the MCPTT group ID for the ad hoc group and further notify their corresponding MCPTT user. The MCPTT users upon receipt of the invitation may accept or reject the call, and respond with the ad hoc group call response. The ad hoc group call response message is routed to the MCPTT server of the primary MC system via the MCPTT server of the partner MC system.

8.	The MCPTT server of the primary MC system provides an ad hoc group call response message to the MCPTT client of the authorized MCPTT user upon receiving response to the corresponding ad hoc group call request in step 1. The ad hoc group call response will consist of the success or failure result and/or detailed reason information in case of failure.

NOTE 2:	The ad hoc group call response message is triggered depending on the conditions to proceed with the call.

9.	Upon successful ad hoc group call setup, a group call is established amongst the multiple group members from primary and partner MC systems. The media plane and floor control resources are established.

##### 10.19.3.2.y	Procedure for ad hoc group call release by MCPTT server – Participants list provided by the Initiator

This procedure focuses on the case where an MCPTT server initiates the termination of an ongoing MCPTT ad hoc group call for all the participants of that group call, since at least one of the termination conditions are met e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCPTT group members are not present.

Procedures in figure 10.19.3.2.y-1 are the signalling control plane procedures for the MCPTT server initiating termination of an ongoing MCPTT ad hoc group call.

Pre-condition:

-	The MCPTT client 1 and MCPTT client 2 belong to primary MC system. The MCPTT client 3 belongs to partner MC system.

Figure 10.19.3.2.y-1: Ad hoc group call release

1.	The MCPTT users on MCPTT client 1, client 2 and client 3 are already part of the ongoing ad hoc group call (e.g. as a result of ad hoc group call setup as specified in clause 10.19.3.2.x).

2.	The MCPTT server would like to release the ad hoc group call which is ongoing e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCPTT group members are not present.

3.	The MCPTT server identifies the participants of the ongoing ad hoc group call and generates ad hoc group call release request to release ongoing session.

4.	The MCPTT server sends a ad hoc group call release request via SIP core towards each participant of the ongoing group call. If the participants belong to partner MC system, then ad hoc group call release request is routed to the MCPTT clients of the parther MC system via the partner MCPTT server.

5.	The MCPTT users are notified about the release of the ad hoc group call.

6.	The MCPTT client(s) receiving ad hoc group call release request, send acknowledgement towards the MCPTT server by sending a ad hoc group call release response.

7.	The MCPTT client 1, client 2 and client 3 have successfully released the floor control and media plane resources associated with the ad hoc group call that is terminated. The MCPTT servers remove the ad hoc group information from the dynamic data held in the MCPTT servers and thus the ad hoc group ceases to exist.