3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230494

e-meeting, 11th – 20th January 2023	(revision of S6-230325, S6-230039)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0156 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating UE Identifier API procedure to enable EEC invoke UE ID request for NATed IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Qualcomm, China Mobile, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the existing UE Identifier API procedure and hence the EES-exposed Eees_UEIdentifier API is used by the EAS only. However, Eees_UEIdentifier API doesn’t support the retrieval of the UE ID for a NATed IP address. The UE Identifier API procedure is required to change so that EEC can also be a new consumer of this API in order to support the retrieval of the UE ID for AC’s (and hence EAS’s) dealing with IPv4 NATed IP address issue (as stated above). So long as EAS’s direct invocation of Eees_UEIdentifier API with UE NATed IP address failes due to lack of support of NATed IP address by NEF, the EAS (through application signaling) may use the AC to ask the EEC for the UE ID. Under such circumstances, EEC would need to interact with the EES to reteive the UE ID; hence the need to update the UE Identifier API procedure in order to allow such an interaction from the EEC. As an example, EEC invocation of Eees_UEIdentifier API may rely on the dual-stack capability of the UE and the usage of the IPv6 in its API request and hence allowing the unique identification of the UE by the NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clauses 8.6.5.1 and 8.6.5.2 to allow the EEC to also be a new consumer of the UE Identifier API. Updating clause 8.6.5.3.2 (making user information IE Optional) in order to allow source IP address of the EEC to be used instead of requiring IP address as an IE in the request body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retrieval of the UE ID corresponding to the UE’s NATed IP address by EEC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5.1, 8.6.5.2, 8.6.5.3.2, 8.6.5.4.1 & 8.6.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Reverses the change made in “User information” IE by making it a Mandatory. Also made the EAS ID IE a list to allow EEC to request UE IDs for a list of EASs associated with a given AC. Also removed NOTE 1 and NOTE 2 from clause 8.6.5.2 as the NOTE in 8.6.5.3.2 suffices. Added ENs to clause 8.6.5.2 $ 8.6.5.3.2 also added a new Pre-conditions. Rev 2: Further updates in Table 8.6.5.3.2-1 |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 8.6.5	UE Identifier API

#### 8.6.5.1	General

EES exposes UE Identifier API to the EAS and EEC in order to provide an identifier uniquely identifying a UE. This API is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. hasn’t already cached). This identifier, called UE ID, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 or EDGE-7. The UE ID is specific to the given EAS and it is represented as a GPSI assigned by the 3GPP Core Network.

Editor's note:	[SA3] [SA2] Check pre-condition #3 to ensure EES is Ok to pass on EAS ID in NEF’s Nnef_UEId as opposed to its own AF ID.

#### 8.6.5.2	Procedure

Figure 8.6.5.2-1 illustrates the interactions between the EES and the EAS.

Pre-conditions:

1.	The EAS or EEC is authorized to discover and to use UE Identifier API provided by the EES.

2.	When the EEC is used to invoke the UE Identifier API with the UE IPv6 address as the input parameter, the UE IPv6 address may or may not be NATed. If NATed however, the IPv6 may not be reused (i.e. assigned to more than one UE simultaneously).

3.	EAS is considered an AF behind EES (as another AF) and EES is authorized to pass EAS ID instead of its own AF ID when it needs to interact with the NEF’s Nnef_UEId_Get (as per TS 23.502 clause 4.15.10 “AF specific UE ID retrieval”).

Editor's note:	[SA3] [SA2] Check pre-condition #3 to ensure EES is Ok to pass on EAS ID in NEF’s Nnef_UEId as opposed to its own AF ID.

Editor's note:	It is FFS whether we need to split this Procedure, into two: the interaction between the EEC with the EES, and the EAS with the EES.

Figure 8.6.5.2-1: UE Identifier API

1.	The EAS or EEC invokes UE Identifier API exposed by the EES.

2.	The EES uses the received user information in the step 1 (e.g. IP address) and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3].

3.	The EES provides the obtained UE identifier as UE ID to the EAS or to EEC (i.e. whichever invoked the API).

Editor's note:	[SA3] Whether and how user's consent is obtained to share the UE identifier with a particular EAS or EEC is SA3's responsibility.

4.	The EAS uses the UE ID received in step 3 to invoke capability exposure API(s) provided by the EES over EDGE-3.

Editor's Note: It is FFS if to find out if there is a need to provide further details on the UE Identifier API procedure when the API is used by EEC? (details such as, the use case where EEC uses this API, the triggers for the EEC invoking the API and the necessity for the usage of UE’s IPv6 address in calling the API, etc)?

#### 8.6.5.3	Information flows

##### 8.6.5.3.1	General

The following information flows are specified for UE Identifier API:

-	UE Identifier request and response.

##### 8.6.5.3.2	UE Identifier API request

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| User information (see NOTE) | M | Information about the User or UE available in the EAS or EEC, e.g. IP address. |
| EAS ID list | M | Identifier of the EAS(s) (aka AFId(s)) for which the UE IDs are requested for by EAS or EEC given the User information (e.g. IP address). |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Security Credentials | M | Security credentials of the EAS or EEC. |
| NOTE: When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. |  |  |

Editor's Note: [SA3] Whether the EAS ID and the EAS Provider ID are part of the security credential is SA3's responsibility.

Editor's Note: Support for obtaining UE ID when IPv6 addresses are NATed (and how to overcome IP address overlap issues) is FFS and requires coordination with SA2.

Editor's Note: [SA3] It is FFS whether the OS can verify the IP address (i.e. User information) provided in the request is indeed the UE’s and whether the UE provided infomation (IP Address) can be trusted.

Editor's Note: It is FFS to determine if EASID (clause 7.2.4) definitions need to be updated in order to be consistent with its use (as per EAS ID list IE) as an AF ID in UE Identifier API.

Editor's Note: The use of Edge UE ID and the defintion of Edge UE ID when the API is used by EEC is FFS.

##### 8.6.5.3.3	UE Identifier API response

Table 8.6.5.3.3-1: UE Identifier API response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the UE identifier request was successful. |
| > UE ID list | O | List of all the AF-specific UE IDs uniquely identifying the UE(s). |
| >> UE ID | M | AF-specific UE ID (i.e. EAS-specific UE ID) |
| >> EAS ID | M | Associated EAS ID |
| Failure response | O | Indicates that the UE identifier request failed. |
| > Cause | O | Indicates the cause of UE identifier request failure |

#### 8.6.5.4	APIs

##### 8.6.5.4.1	General

Table 8.6.5.4.1-1 illustrates the APIs for UE Identifier.

Table 8.6.5.4.1-1: Eees_UE_Identifier API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_UEIdentifier | Get | Request/Response | EAS, EEC |

##### 8.6.5.4.2	Eees_UEIdentifier_Get operation

API operation name: Eees_UEIdentifier_Get

Description: The consumer requests identifier of a UE.

Inputs: See clause 8.6.5.3.2.

Outputs: See clause 8.6.5.3.3.

See clause 8.6.5.2 for details of usage of this operation.