3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230047

# E-meeting, 11th January – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0143 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service operations for the SS_VALServiceAreaConfiguration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The VAL servise area ID support was introduced for SEAL Location Mangament APIs in CR #0136 of 23.434 (agreed in SA6 52 meeting). Thus, the related service operations shall be specified to enable the VAL service area ID functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces new service operations and related information flows for the SS_VALServiceAreaConfiguration API. It is proposed to correct “Geographical coordinates” to “Geographical location” to enable a flexible definition of the VAL service area. The indication of the VAL service area ID changed flag is added to the Location area monitoring notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The service operations with the related procedures and information flows are unspecified for the SS_VALServiceAreaConfiguration API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.16, 9.3.2.21, 9.3.2.22, 9.3.2.23(new), 9.3.2.24(new), 9.3.2.25(new), 9.3.2.26(new), 9.3.2.27(new), 9.3.2.28(new), 9.3.2.29(new), 9.3.13.2, 9.3.13.3(new), 9.3.13.4(new), 9.3.13.5(new), 9.3.13.6(new), 9.3.13.7(new), 9.4.1, 9.4.8.2, 9.4.8.3(new), 9.4.8.4(new), 9.4.8.5(new), 9.4.8.6(new), 9.4.8.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.16	Location area monitoring notification

Table 9.3.2.16-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.16-1: Location area monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UEs currently present (see NOTE) | O | List of the identities of all VAL UEs who are currently present in the given location area |
| UEs moved in/out (see NOTE) | O | List of UEs either moved into the location area or moved out of the location area |
| >> UEs moved in | O | List of the identities of the VAL UEs who moved into the given location area since previous notification. |
| >> UEs moved out | O | List of the identities of the VAL UEs who moved out of the given location area since previous notification. |
| Triggering event | O | Identity of the event that triggered the sending of the notification |
| VAL service area ID changed flag | O | The indication of the VAL service area change event that affects this and further notifications. |
| NOTE: Any one of these shall be present. |  |  |

* * * Next Change * * * *

#### 9.3.2.21	VAL service area configuration request

Table 9.3.2.21-1 describes the information flow from the the VAL server to the location management server for VAL service area configuration request.

Table 9.3.2.21-1: VAL service area configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server. |
| VAL service area IDs list | M | List of one or more VAL service area identifiers. |
| Geographical locations list | M | List of one more geographical locations (e.g., co-ordinates) corresponding to each VAL service area ID. |

* * * Next Change * * * *

#### 9.3.2.22	VAL service area configuration response

Table 9.3.2.22-1 describes the information flow from the location management server to the VAL server for VAL service area configuration response.

Table 9.3.2.22-1: VAL service area configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL Service area configuration status | M | It indicates the result of VAL service area configuration request. |
| List of VAL service area identifiers | O (see NOTE) | If configuration is successful, identifies the list of the configured VAL service area identifiers. |
| NOTE: This IE shall be present only if the configuration status is set to success. |  |  |

* * * Next Change * * * *

#### 9.3.2.23	VAL service area obtain request

Table 9.3.2.23-1 describes the information flow from a VAL server to a location management server for obtain VAL service area identifiers at the LM server.

Table 9.3.2.23-1: Obtain VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| VAL service area IDs | O | Indicates the list of the requested VAL service area identifiers. If this information element is not provided, the VAL server requests all available VAL service area identifiers for the VAL server. |

* * * Next Change * * * *

#### 9.3.2.24	VAL service area obtain response

Table 9.3.2.24-1 describes the information flow from a location management server to the VAL server for obtain VAL service area identifiers at the LM server.

Table 9.3.2.24-1: Obtain VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service area IDs | M | Indicates the list of the requested VAL service area identifiers. |
| Geographical locations | M | List of the geographical locations (e.g., the geographical coordinates) corresponding to each provided VAL service area ID in this response. |

* * * Next Change * * * *

#### 9.3.2.25	VAL service area update request

Table 9.3.2.25-1 describes the information flow from a VAL server to a location management for update VAL service area identifiers at the LM server.

Table 9.3.2.25-1: Update VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service area IDs | M | Indicates the list of the VAL service area identifiers. |
| Geographical locations | M | Indicates the list of the geographical locations (e.g., the geographical coordinates) corresponding to each provided VAL service area ID that shall be updated. |

* * * Next Change * * * *

#### 9.3.2.26	VAL service area update response

Table 9.3.2.26-1 describes the information flow from a location management server to the VAL server for update VAL service area identifiers at the LM server.

Table 9.3.2.26-1: Update VAL service area response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| VAL service area IDs | O (see NOTE) | Indicates the list of the updated VAL service area identifiers. |
| NOTE: This information element shall be present only if the update VAL service area identifier(s) is successful. |  |  |

* * * Next Change * * * *

#### 9.3.2.27	VAL service area subscription request

Table 9.3.2.27-1 describes the information flow from a VAL server to a location management to subscribe VAL service area identifier update events at the LM server.

Table 9.3.2.27-1: VAL service area subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service area IDs | M | Indicates the list of the VAL service area identifiers. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL service area update events. |
| Subscription duration | O | It indicates the subscription duration. If absent, 1 hour is used as default setting. |

* * * Next Change * * * *

#### 9.3.2.28	VAL service area subscription response

Table 9.3.2.28-1 describes the information flow from a location management server to a VAL server for VAL service area subscription response.

Table 9.3.2.28-1: VAL service area subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |

* * * Next Change * * * *

#### 9.3.2.29	VAL service area notification

Table 9.3.2.29-1 describes the information flow for VAL service area notification from a location management server to a VAL server.

Table 9.3.2.29-1: VAL service area notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL service area IDs | M | Indicates the list of the updated VAL service area identifiers. |
| Geographical locations | M | Indicates the list of the geographical locations (e.g., the geographical coordinates) corresponding to each updated VAL service area ID. |

* * * Next Change * * * *

#### 9.3.13.2	Configure VAL service area identifier procedure

Figure 9.3.13.2-1 illustrates the high level procedure of the VAL service area identifier configuration.

Figure 9.3.13.2-1: Configure VAL service area identifier procedure

Procedure:

- the VAL server sends the VAL service area configuration request to the location management server to configure a new VAL service area identifier. In the request message, the VAL server includes the information as specified in table 9.3.2.21-1;

- the location management server shall check if the VAL server is authorized to initiate VAL service area identifier configuration request;

- if the VAL server is authorized to configure the VAL service area identifiers, the VAL server checks the consistency of the provided identifiers (e.g., all VAL service area identifiers shall be unique) and configures the provided VAL service area identifiers; and

- the location management server sends the VAL service area configuration response with the information as specified in table 9.3.2.22-1.

* * * Next Change * * * *

#### 9.3.13.3	Obtain VAL service area identifier procedure

Figure 9.3.13.3-1 illustrates the high level procedure of the VAL service area retrieval.

Figure 9.3.13.3-1: Obtain VAL service area procedure

Procedure:

- the VAL server sends the VAL service area obtain request to the location management server to retrieve the available VAL service area identifiers. In the request message, the VAL server includes the information as specified in table 9.3.2.23-1;

- the location management server shall check if the VAL server is authorized to initiate VAL service area obtain request; and

- if the VAL server is authorized to obtain the VAL service area identifiers, the location management server sends the VAL service area obtain response with the information as specified in table 9.3.2.24-1.

* * * Next Change * * * *

#### 9.3.13.4	Update VAL service area identifier procedure

Figure 9.3.13.4-1 illustrates the high level procedure of the VAL service area update.

Figure 9.3.13.4-1: Update VAL service area procedure

Procedure:

- the VAL server sends the VAL service area update request to the location management server to update the VAL service area identifier(s). In the request message, the VAL server includes the information as specified in table 9.3.2.25-1;

- the location management server shall check if the VAL server is authorized to initiate VAL service area update request;

- if the VAL server is authorized to update the VAL service area identifiers, the location management server updates the VAL service area identifier and provisions the updated VAL service area identifier across SEAL APIs for location management; and

NOTE:	The provisioning of the updated VAL service area identifier(s) across the SEAL APIs for location management is implementation specific.

- the location management server sends the VAL service area update response with the information as specified in table 9.3.2.26-1.

* * * Next Change * * * *

#### 9.3.13.5	VAL service area identifier subscribe procedure

Figure 9.3.13.5-1 illustrates the high level procedure of the subscribe VAL service area update events.

Figure 9.3.13.5-1: Subscribe VAL service area procedure

Procedure:

- the VAL server sends the VAL service area subscription request to the location management server to subscribe for the VAL service area identifier(s) update events. In the request message, the VAL server includes the information as specified in table 9.3.2.27-1;

- the location management server shall check if the VAL server is authorized to susbcribe for the VAL service area identifier(s) update event(s);

- if the VAL server is authorized to subscribe for the VAL service area identifier(s) update events, the location management server creates the subscription based on the provided parameters in the request; and

- the location management server sends the VAL service area subscription response with the information as specified in table 9.3.2.28-1.

* * * Next Change * * * *

#### 9.3.13.6	VAL service area identifier notify procedure

Figure 9.3.13.6-1 illustrates the high level procedure for notification of a VAL server about the VAL service area identifier update events at the location management server.

Figure 9.3.13.6-1: Notify VAL service area procedure

Pre-conditions:

-	the VAL server has an active VAL service area subscription at the location management server; and

-	one or more of the VAL service area identifiers was updated.

Procedure:

- the location management server identifies the list of the updated VAL service area identifiers for which the VAL server subscribed; and

- the location management server sends a VAL service area notification to the VAL server. In the notification message, the location management server includes the information as specified in table 9.3.2.29-1.

* * * Next Change * * * *

#### 9.3.13.7	VAL service area identifier unsubscribe procedure

Figure 9.3.13.7-1 illustrates the high level procedure for notification of a VAL server about the VAL service area identifier update events at the location management server.

Figure 9.3.13.7-1: Unsubscribe VAL service area procedure

Pre-conditions:

-	the VAL server has an active VAL service area subscription at the location management server.

Procedure:

- when the VAL server decides to terminate a VAL service area subscription, it sends a VAL service area unsubscribe request to the location management server. In the request message, the VAL server includes the information as specified in table 9.3.2.21-1;

- the location management server terminates the VAL service area subscription; and

- the location management server sends a VAL service area unsubscribe request to the VAL server, the location management server includes the information as specified in table 9.3.2.22-1.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Update_Event | VAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Update_Event | VAL server |  |
|  | Unsubscribe_VAL_Service_Area_Update_Event | VAL server | Request/Response |

* * * Next Change * * * *

#### 9.4.8.2	Configure_VAL_Service_Area

API operation name: Configure_VAL_Service_Area

Description: Configuration of VAL service area identifiers.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.21

Outputs: See subclause 9.3.2.22.

See subclause 9.3.13.2 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.3	Obtain_VAL_Service_Area

API operation name: Obtain_VAL_Service_Area

Description: Retrieval of VAL service area identifiers.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.23

Outputs: See subclause 9.3.2.24.

See subclause 9.3.13.3 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.4	Update_VAL_Service_Area

API operation name: Update_VAL_Service_Area

Description: Update of the VAL service area identifier.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.25

Outputs: See subclause 9.3.2.26.

See subclause 9.3.13.4 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.5	Subscribe_VAL_Service_Area_Update_Event

API operation name: Subscribe_VAL_Service_Area_Update_Event

Description: Subscription to the VAL service area identifiers update events.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.27

Outputs: See subclause 9.3.2.28.

See subclause 9.3.13.5 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.6	Notify_VAL_Service_Area_Change_Update_Event

API operation name: Notify_VAL_Service_Area_Change_Update_Event

Description: Notifying the VAL server about the VAL service area identifier update events.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.29

Outputs: None.

See subclause 9.3.13.6 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.7	Unsubscribe_VAL_Service_Area_Update_Event

API operation name: Unsubscribe_VAL_Service_Area_Update_Event

Description: Unsubscribing from VAL service area identifier update events.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.21

Outputs: See subclause 9.3.2.22.

See subclause 9.3.13.7 for the details of usage of this API operation.

* * * End of changes * * * *