3GPP TSG-SA6 Meeting #53	S6-231085

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023	(revision of S6-230884, S6-230595, S6-230495)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0159 | rev | 5 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting dynamic EAS instantiation triggering and notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FS_eEDGEAPP study, the enhancement of dynamic EAS instantiation triggering was studied as Key Issue #9; and concluded to support (1) reporting for a need of EAS instantiations to the ECSP management system and (2) notifications of the dynamic EAS instantiation triggering results as specified in clause 7.32 of TR 23.700-98. |  |  |  |  |  |  |  |  |
| Summary of change: |  | <S6-230495> In order to support for the enhancement of dynamic EAS instantiation triggering, the following changes are proposed: - Clause 8.12: added EAS discovery subscription request for a condition of dynamic EAS instantiation triggering; and added the details of reporting for a need of EAS instantiations to the ECSP management system and notifications of the dynamic EAS instantiation triggering results - Clause 8.5.2.3.2: added how to handle the EAS discovery subscription request for instantiable EAS(s) <S6-230595> In order to resolve the editor’s notes, the following changes are proposed: - Clause 8.5.2.3.2: specified the operational conditions as per processing the EAS discovery subscription requests - Clause 8.5.3.4: added a new indication IE in the EAS discovery subscription request for triggering EAS instantiations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancement of dynamic EAS instantiation triggering within the EDGEAPP architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12, 8.5.2.3.2, 8.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#9 - Solution #32 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S6-230495 which was agreed in SA6#52-bis-e |  |  |  |  |  |  |  |  |

1st CHANGE

8.12	Dynamic EAS instantiation triggering

8.12.1	General

The EES may trigger the EAS instantiation dynamically due to e.g., EAS discovery request, EAS discovery subscription request, UE mobility, upon receiving EEC Registration request containing AC profile.

Upon receiving the EAS discovery request with EAS discovery filter from the EEC or the S-EES during the procedures for EAS discovery or ACR, the EES may fail to discover and select the EAS that matches the UE location and the requesting application characteristics specified in table 8.5.3.2-2 due to no EAS is available or instantiated. The EES may trigger the ECSP management system (which is specified in TS 28.538 [22]) to instantiate the EAS serving the AC in the EDN before returning the EAS information to the EEC or S-EES, based on the pre-configured dynamic EAS instantiation information about which EAS can be dynamically instantiated at the associated EDN.

Upon receving one or more EAS discovery subscription request(s) for the availability of an EAS, EES may determine if there is a need for EAS instantiation based on the pre-configured information about instantiable EASs. If such a need for EAS instantiation determined, EES may send a report for a need of the EAS instantiation to the ECSP management system to consider instantiating the requested EAS by invoking an MnS API of the ECSP management system. When the requested EAS has been instantiated, the EES may obtain the EAS profile during the EAS registration procedure and notify the availability change event of the requested EAS with the EAS profile to the corresponding EECs via the EAS discovery notification proccedure as specified in the clause 8.5.2.3.

NOTE 1:	The ECSP management system is responsible for the authorization of the dynamic EAS instantiation.

NOTE 2:	The ECSP management system can provide the pre-configured dynamic EAS instantiation information to the EES. Such a mechanism is out of scope of this release of the present document.

NOTE 3:	Dynamic EAS instantiation information at the EES is left implementation specific in this version of the specification.

NOTE 4:	When determining a need for EAS instantiation, EES can further consider the requested service characteristics (e.g. location, latency) by EEC or service load/capacity (e.g. number of service sessions) of EAS, which is upon implementation and out the scope of this specification.

Editor’s note:	The MnS APIs for EAS instantiation request/notification should be provided by the ECSP management system, which is the scope of SA5.

Editor’s note: Relationship between “pre-configured information about instantiable EASs” and “instantiable EAS information” of the EES Profile needs to be established and consistent usage of these IEs is FFS.

2nd CHANGE

8.5.2.3.2	Subscribe

Figure 8.5.2.3.2-1 illustrates the EAS discovery subscription procedure between the EEC and the EES. This subscription enables EES to inform EEC of various EAS discovery related events of interest to EEC (e.g. EAS discovery notification and EAS dynamic information).

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11;

3.	The EES is configured with ECSP's policy for EAS discovery; and

4.	The EEC has optionally acquired a Notification Target Address to be used in its subscriptions to notifications.

NOTE 1:	Details of ECSP's policy are out of scope.

NOTE 2:	How the EEC acquires the notification target address or a notification channel URI to receive the notifications is out of scope of this release. The notification target address can terminate at the EEC (e.g. in an IoT device) if the deployment supports EEC reachability, or it can terminate at a push notification service. Details of the push notification service are out of scope of this release.

Figure 8.5.2.3.2-1: EAS discovery subscription

1.	The EEC sends an EAS discovery subscription request to the EES. The EAS discovery subscription request includes the EECID along with the security credentials, Event ID, and may include EAS discovery filters and EAS dynamic information filters to subscribe to information about particular EAS(s) or a category of EASs (e.g. gaming applications) or dynamic information about EAS(s).

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to subscribe for information of the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. The EES may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.3. If the request is authorized, the EES creates and stores the subscription for EAS discovery.

3.	If the processing of the request was successful, the EES sends an EAS discovery subscription response to the EEC, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC shall send an EAS discovery subscription update request prior to the expiration time. If an EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed.

In the case of subscription to an EAS availability change event, if there is no instantiated EAS that matches the requested EAS discovery filters and such EAS is instantiable based on the pre-configured information about instantiable EASs, the request is treated as successful. If the EEC indicates EAS Instantiation Triggering in the EAS discovery subscription request, the EES may trigger dynamic instantiation of the EAS as specified in the clause 8.12; otherwise, the EES does not trigger the EAS instantiation.

In the case of subscription to an EAS dynamic information change event, if the EES is unable to determine the instantiated EAS information using the inputs in the EAS discovery subscription request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery subscription request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the EAS discovery subscription request fails, the EEC may resend the EAS discovery subscription request again, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery subscription request.

3rd CHANGE

8.5.3.4	EAS discovery subscription request

Table 8.5.3.4-1 describes the information elements for EAS discovery subscription request from the EEC to the EES.

Table 8.5.3.4-1: EAS discovery subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Event ID | M | Event ID: - EAS availability change - EAS dynamic information change |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address | O | The Notification target address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| EAS discovery filters | O | Set of characteristics to determine matching EASs (as detailed in Table 8.5.3.2-2). Applicable for "EAS availability change" event |
| EAS dynamic information filters | O | List of dynamic information changes (as detailed in Table 8.5.3.4-2) about EAS, the EEC is interested in. Applicable for "EAS dynamic information change" event |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| EAS Instantiation Triggering Indication | O | Indicates to the EES that EAS instantiation triggering may be performed for the current request if needed. |

Table 8.5.3.4-2: EAS dynamic information filters

| Information element | Status | Description |
| --- | --- | --- |
| List of dynamic information filters | M | List of EAS dynamic information required by the EEC per EAS. |
| > EASID | M | Identifier of the EAS |
| > ACIDs | O | Flag to notify change in list of ACIDs served by the EAS |
| > EAS Description | O | Flag to notify change in description of the EAS. |
| > EAS Endpoint | O | Flag to notify change in EAS endpoint |
| > EAS Features | O | Flag to notify any change in features provided by the EAS |
| > EAS Schedule | O | Flag to notify change in availability schedule of the EAS (e.g. time windows) |
| > EAS Service Area | O | Flag to notify change in change in geographical service area that the EAS serves |
| > EAS Service KPIs | O | Flag to notify change in characteristics of the EAS. |
| > EAS Status | O | Flag to notify change in the status of the EAS (e.g. enabled, disabled, etc.) |
| > Service continuity support | O | Flag to notify change in EAS support for service continuity. |

END OF CHANGES