3GPP TSG-SA WG6 Meeting #53	S6-231303

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0199 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reliability for application enabler servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reliable Edge computing was studied in Sol#48 of TR 23.700-98. It was commented that the reliablity is applicable for all enabler servers and it is required to provide generic mechanism to support reliablity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Using NF Set concept as defined in TS 23.501/23.502 as a base, reliability for application enabler server is defined using 3 roles – consumer, producer and registrar. - Generic description of NF Set / AES set is specified in Clause X.1, X.2 and X.3 - Consumer Subscription for producer availability change in Clause X.4.1 - Context transfer between producer AESs in Clause X.4.2 - Consumer discovering equivalent AES from AES Set in Clause X.4.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reliability support for application enabler servers will not be available in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (New), X.1 (New), X.2 (New), X.3 (New), X.4 (New), X.4.1 (New), X.4.2 (New), X.4.3 (New), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to KI#23, Sol#48 of TR 23.700-98 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# X	Reliability for application enabler server

## X.1	General

This clause defines three roles which may be implemented by different application enablement layers (e.g. EDGEAPP). The Application Enabler Server (AES) providing the services is called producer role, the AES or Application Enabler Client (AEC) consuming the service is called consumer role and the entity where producer registers is called registrar role.

An AES instance can be deployed such that several AESs are present within an AES Set to provide distribution, redundancy and scalability together as a Set of AES instances. This can be achieved when the equivalent AESs share the same context data.

Editor’s note:	How equivalent AESs are determined (e.g. based on KPIs, service area) to group into an AES set (e.g. by O&M or other means) is FFS.

The binding indication may be used by the producer and consumer to indicate a particular context that is bound to an AES server instance or AES server set. The basic principle for binding is to exchange binding information between consumer and producer so that they know how to connect to alternative in case of service failure depending on level of binding.

NOTE 1:	An AES can be replaced by an alternative AES within the same AES Set in the case of scenarios such as failure, load balancing, and load re-balancing.

The clause provide generic mechanism using different roles to achieve reliability for application enabler layer. If required, the specific application enabler layer, which implements this roles, may add application enabler specific details to achieve reliability.

NOTE 2:	In current release, only AES Set is considered. Further enhancements (e.g. AES Service Set) can be considered in future release.

## X.2	AES Set

Equivalent AESs may be grouped into AES Sets, e.g. several SEAL server instances are grouped into an SEAL server Set. AESs within a AES Set are interchangeable because they share the same context data, and may be deployed in different locations, e.g. different data centres.

NOTE 1:	The actual mapping of instances to a given Set is up to deployment.

## X.3	Reliability of AES instances within the same AES Set

When the AES producer instance is not available, another AES producer instance within the same AES Set is selected.

The consumer may subscribe to availability change notifications of AES instance from the registrar as specified in clause X.4.1. If the consumer is notified by the registrar or detects by itself (e.g. request is not responded) that the AES producer instance is not available anymore, another available AES producer instance within the same AES Set is selected by the consumer.

NOTE 2:	The registrar role is optional, and if the specific enabler layer, do not implement registrar role, then the reliability can be achived when consumer detects by itself about unavailability of the producer.

* * * Second Change * * * *

## X.4	Procedures

### X.4.1	Consumer subscribes for availability change event

The consumer can subscribe to the producer availability change information event. Figure X.4.1-1 illustrate procedure for consumer to subscribe to registrar for producer availability change information.

Figure X.4.1-1:	Consumer subscribing for producer availability change event

1)	To perform the subscription, the consumer sends subscription message with parameter indicating the subscription to the producer availability change event, identity of the producer and AES set identity.

2)	The registrar verifies the consumer and also the validity of the status of the producer entity. The registrar sends response back to the consumer indicating the result of the subscription.

NOTE:	If there already exists another type subscription between consumer and registrar, then the subscription for producer availability change event can be merged into exiting subscription.

Figure X.4.1-2 illustrate procedure for registrar to send notification to the consumer for producer availability change event.

Figure X.4.1-1:	Notification for producer availability change event

1)	Upon occurance of an event that satisfies trigger conditions for notifying a subscribed consumer about producer availability change, the registrar sends notification message to the consumer indicating the producer identity, AES set ID, and possible list of equivalent AESs from the same AES set.

Upon receiving the notification, the consumer performs reslection based on binding information.

* * * 3rd Change * * * *

### X.4.2	AES context transfer

AES Context Transfer Procedures allow transfer of Service Context of an AES from a Source AES Instance to the Target AES Instance e.g. before the Source AES can gracefully close its AES Service. In order to transfer context among AES instances of same AS set, the context push or pull mechanism can be ustilized. Each application enaler layer which uses the roles defined in this clause, needs to specify contect transfer mechanism among the AES producer instances within same AES set with application enabler layer specific details.

* * * 4th Change * * * *

### X.4.3	AES discovery from AES Set

Figure X.4.3-1 illustrate procedure for consumer to discover a set of AESs with specific AES set.

Figure X.4.3-1:	Consumer discovering AESs from AES set

1)	The consumer sends a request to the registrar to discover a list of equivalent AESs to connect to when the current AES is unavailable. The request includes an AES set identity, an AES identity where consumer is currently connected to, and an AES endpoint address. The request may include an indication to indicate the registrar to select the equivalent AES to connect to when the current AES is unavailable.

2)	The registrar checks whether the consumer is authorized or not. If the consumer is authorized, the registrar identifies the list of AESs from the same AES set based on the AES set identity and other parameters, such as service areas and supported key performance indexes (KPIs). If the request includes the indication to perform the selection, then the registrar selects one of the AES to which the consumer can connect. The registrar sends a response to the consumer including a list of equivalent AESs or one selected AES (if the indication for the selection is included in the request).

If the response includes the list of equivalent AESs then the consumer selects one of the equivalent AES to connect, based on different parameters, such as the service areas and the KPIs.

* * * End of Change * * * *