3GPP TSG-SA WG6 Meeting #54-e	S6-231333

e-meeting, 17th – 26th April 2023	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0387 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications related to authorization binding procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR emphasizes the importance of the binding (i.e., association) between MC gateway client and MC gateway UE, so that the MC service server (or SIP core) controls the bearer towards the MC gateway UE. Hence, the MC service server is aware of the unique IP address of MC service users, i.e., non-3GPP devices behind the MC gateway UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add clarification text related to authorization of binding between MC gateway client and MC gateway UE in clause 11.5.1 - Adding the scenario of having an MC service user on non-3GPP device that is behind the MC gateway UE in clause 9.2.2.3.2 and clause 9.2.2.3.3 related to bearer control be MC service server, or SIP core. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The full control of bearers towards the MC gateway UE is unclear. It is also unclear how the MC service server is aware of the unique IP addresses of MC service users behind the MC gateway UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1, 9.2.2.3.2, and 9.2.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 11.5.1	Connection authorisation mechanisms

#### 11.5.1.1	General

The connection of non-3GPP devices via an MC gateway UE require authorisation verification by the MC system. The authorisation of binding between the MC gateway client and MC gateway UE, i.e., MC gateway client to MC gateway UE association, ensures the control of 3GPP based access and the bearer needed for the MC gateway UE by the MC service server. Hence, the MC service server is aware of the unique IP address per MC service user behind the MC gateway UE to assure requesting the proper radio resources over Rx interface, thus route the media and signalling accordingly.

The binding authorisation is supported on two different types of non-3GPP devices, those which can host MC service client and those which cannot host MC service clients.

# * * * * Second change * * * *

##### 9.2.2.3.2	Control of bearers by SIP core

In this scenario, bearer control is performed by the SIP core alone, as shown in figure 9.2.2.3.2-1 below.

Figure 9.2.2.3.2-1: Bearer control by SIP core

For MC gateway UE when the bearer control is performed by the SIP core alone, the scenario is shown in figure 9.2.2.3.2-2. The authorised binding between the MC gateway client to MC gateway UE described in clause 11.5.1 facilitates bearer control of the MC gateway UE serving the non-3GPP devices.

Figure 9.2.2.3.2-2: Bearer control by SIP core in MC gateway UE scenario

# * * * * Third change * * * *

##### 9.2.2.3.3	Control of bearers by MC service server

In this scenario, bearer control is performed by the MC service server alone, as shown in figure 9.2.2.3.3-1 below.

Figure 9.2.2.3.3-1: Bearer control by MC service server

For MC gateway UE when the bearer control is performed by the MC service server alone, the scenario is shown in figure 9.2.2.3.3-2. The authorised binding between the MC gateway client to MC gateway UE described in clause 11.5.1 facilitates bearer control of the MC gateway UE serving the non-3GPP devices via the MC gateway UE.

Figure 9.2.2.3.3-1: Bearer control by MC service server in MC gateway UE scenario

# * * * * End of changes * * * *