3GPP TSG-SA WG6 Meeting #54-e	S6-231407

17th – 26th April 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0348 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Bundle EAS requirement EN and editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds the description of 7.2.10 to resolve the EN for Bundle requirements and to clarify the operation of the EEL. Edit Correction in Related Procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve EN in AC, EAS and EES profiles. Add description of the Bundle informations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not resolved. Misreading due to incorrect notation in related procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.10, 8.2.2, 8.2.4, 8.2.6 (B) 8.4.4.2.2, 8.15.1 (F) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.2.10	EAS bundle information

The EAS bundle information can be a list of EASIDs or a bundle ID, EAS bundle requirements. EAS bundle ID establishes an association between the EASs. When included in the EAS profile, EAS bundle ID denotes the bundle to which the EAS belongs. When included in the AC profile EAS bundle ID is used to perform different Edge Enabler Layer operations, such as EAS discovery. Edge Enabler Layer handles the EASs belonging to the same bundle as required by related EAS bundle requirements as described in clause 8.2.10. If there is no bundle requirement in the AC, EAS, or EES profiles, the bundle information can be used to perform different Edge Enabler Layer operations, such as EAS discovery, depending on the ECSP policy or service provider implementation.

NOTE 1 :	Both, EAS bundle ID and EAS bundle requirements, are provided by the ASP.

NOTE 2:	Bundle ID is necessary when the affinity between bundled EASs is strong (e.g., co-deployment and co-migration is essential), and the related ASPs which provide the AC and bundled EASs established the bundle. List of EASIDs is required when the affinity between the bundled EASs is weak (e.g., co-deployment and co-migration is only "nice to have").

* * * Next Change * * * *

8.2.2	AC Profile

An AC Profile includes information about AC used to determine services and service characteristics required.

Table 8.2.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC, also indicates the AC ability of handling bundled EAS ACR. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| List of required EAS bundle information | O | Information related to the EAS bundles which AC requires. |
| > Bundle ID or list of EASID | M | A list of EASIDs or a bundle ID as described in clause 7.2.10. |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |

* * * Next Change * * * *

8.2.4	EAS Profile

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > Bundle ID | M | Bundle ID as described in clause 7.2.10. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| Allowed MNO information | O | Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability of handling bundled EAS ACR. |
| EAS Transport layer service continuity support | O | This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation |
| General context holding time duration (NOTE) | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |
| NOTE: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). |  |  |

* * * Next Change * * * *

8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| > List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| >> Bundle ID | M | A bundle ID as described in clause 7.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (see NOTE) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability of handling bundled EAS ACR. |
| NOTE: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

* * * Next Change * * * *

8.4.4	EES Registration

8.4.4.1	General

The EES Registration procedure allows an EES to provide information to an ECS in order to enable provisioning EES(s) to an EEC.

The EES registration update procedure allows an EES to update the ECS, if there is a change in the information at the EES.

The EES uses the EES de-registration procedure to remove its information from the ECS.

8.4.4.2	Procedures

8.4.4.2.1	General

8.4.4.2.2	EES registration

Pre-conditions:

1.	The EES has the address (e.g. URI) of the ECS;

2.	Both, the EES and ECS, have the necessary credentials to enable communications.

Figure 8.4.4.2.2-1: EES Registration procedure

1.	The EES sends the EES registration request to the ECS. The request from the EES includes the EES profile and EES security credentials. The request may include a proposed expiration time for the registration.

2.	Upon receiving the request from the EES, the ECS verifies the security credentials of the EES and stores the EES registration information obtained in step 1. If the EES profile includes bundle ID, the ECS stores the information and associates the EES with other EESs providing the same EAS bundle information.

3.	The ECS sends an EES registration response indicating success or failure of the registration operation. The ECS may provide an updated expiration time to indicate to the EES when the registration will automatically expire. To maintain the registration, the EES shall send a registration update request prior to the expiration time. If a registration update request is not received prior to the expiration time, the ECS shall treat the EES as implicitly de-registered.

* * * Next Change * * * *

8.15	EAS Information provisioning

8.15.1	General

EAS information provisioning procedure allows the EEC to exchange information with the EES about selected EAS or ACR scenario selection.

When service continuity is required, service continuity scenarios may be combined to perform ACR detection in one or more of the EEC, the EES and the EAS; the related procedures are specified in clauses 8.15.2, 8.6.3 and referred to in clause 8.8.2. The selection of ACR scenario(s) may be performed by the EEC or the EES for a given AC and the selected EAS from the common supported ACR scenarios of AC, EEC, selected EES and selected EAS. The selection of ACR scenario(s) for each EAS in EAS bundles may be performed speparately by the EEC or the EES for a given AC and the selected EAS(s) in a bundle, based on the service continuity support by AC, EEC, EES and EAS(s). In addition, the EEC or EES can further determine the one or more ACR scenario(s) for the selected EAS based on the AC service KPI.

Editor's Note:	It is FFS whether the ACR scenario selection for EAS bundles is needed to extend to multiple EES scenario, depending on the feedback from SA5.

NOTE:	How to select ACR scenario(s) at the EEC or EES considering AC service KPI is implementation specific.

Information about instantiable EAS may be provided to the EEC as in clause 8.3.3 and 8.5. Triggering the instantiation of an EAS by the EEC may be announced to the EES by including the selected EASID in the EAS information provisioning request without including the selected EAS endpoint.

The EAS information provisioning request types supported are:

-	"ACR scenario selection announcement". Inform the EES about the EAS that has been selected by the EEC and may provide the selected ACR scenario list to the EES. For the EAS bundles scenario, the selected bundled EAS(s) and the selected ACR scenario list for EAS bundles by EEC are included and notified to the EES.

-	"ACR scenario selection request". Inform the EES to perform ACR scenario selection. For the EAS bundles scenario, the request may inform the EES to determine the ACR scenario list for EAS bundles.

* * * END of Change * * * *