3GPP TSG-SA WG6 Meeting #54e	S6-231452

e-meeting,  17th – 26th April 2023		(revision of S6-231226)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0193 | rev | 1 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add “Location service update” procedure and information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Location service registration procedure from LM Client to LM Server has been documented and agreed, the Location service update procedure and information flow should also be added in case the LM Client desires to update the location service resgistered to LM Server before. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Location service registration update procedure and information flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G-enabled fused location service capabilities will not support Location service update for SEAL location management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x(new),9.3.2.x(new),9.3.2.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3	Procedures and information flows for Location management (on-network)

9.3.x	Location service registration update procedure

Figure 9.3.x-1 illustrates the procedure of client-triggered location service registration  update. The Location Management Client may update its supported location capabilities (e.g.  location access type, position methods) which has registered to the Location Management Server before.

Pre-condition:

The Location Management Client has registered to the Location Management Server.

Figure 9.3.x-1: Location service registration update procedure

1.	The LM Client decides to trigger the registration  update request to LM Server to update the location capabilities which have registered to the LM Server before.

2.	The LM Client sends location service registration update request to the LM Server with the identifier of the VAL UE, the VAL service ID and updated location capabilities.

3.	The LM server checks the authorization for the VAL UE's registration  update request. If the authorization is successful, the LM server will update the UE’s location capabilities received in step 2.

4.  The LM Server sends location service registration  update response to the LM Client.

* * * Next Change * * * *

9.3.2.x	Location service registration update request

Table 9.3.2.x-1 describes the information flow from the location management client to the location management server for update the registered location service.

Table 9.3.2.x-1: Location service registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is updated. |
| Location capability | O | The information of location capability of VAL UE for which the location service is updated. |
| >Location access type (NOTE 2) | O (NOTE 1) | Identifies the updated location access type of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| >Positioning method | O (NOTE 1) | Identifies the updated positioning methods of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| NOTE 1: At least one of these rows shall be present. NOTE 2: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. |  |  |

* * * Next Change * * * *

9.3.2.y	Location service registration update response

Table 9.3.2.y-1 describes the information flow from the location management server to the location management client for the location service registration update response.

Table 9.3.2.y-1: Location service registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Update status | M | It indicated the update result. |

* * * End of Changes * * * *