3GPP TSG-SA WG6 Meeting #54-e	S6-231607

e-meeting, 17th – 26th April 2023	(revision of S6-231524)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0364 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the MC service user ID list within preconfigured user regroup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CR introduces a new procedure to update the MC service user ID list within an ongoing preconfigured user regroup. The introduced procedure includes the case when a new user matches the predefined MC service user criteria, or another no longer matches the criteria. The MC service user criteria is known at the MC service server as described in clause 10.15.3.2.2. The introduced update matches stage 1 requirement [R-6.6.4.2-002b] in 3GPP TS 22.280. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the precondition to allow adding and removing MC service clients based on MC service user criteria known at the MC service server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements related to new MC service clients which meet/no longer meet the criteria of the ongoing preconfigured group regroup are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | The following new clauses: 10.15.3.4.X, 10.15.2.xx, and 10.15.2.yx. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.15.3.4.x	Updating the MC service ID list in a preconfigured regroup group based on user regroup

Figure 10.15.3.4.x-1 illustrates the procedure to add or remove an MC service user based on the MC service user criteria known at the MC service server.

NOTE 1: 	This procedure applies to MC service users that have been added and are to be added to the preconfigured regroup group based on MC service user criteria known at the MC service server. If only the criteria are to be applied, then only the criteria are included in the request and the group and user lists are not included

Preconditions:

- A preconfigured regroup group has been established.

-	The MC service server has selected MC service user 2 to participate in the preconfigured regroup group based on MC service user criteria known at the MC service server.

NOTE 2:	Other participants of the preconfigured regroup group are not shown in figure 10.15.3.4.x-1 for simplicity.

Figure 10.15.3.4.x-1: Procedure to update the MC service ID list

1.	The MC service server detects that MC service client 2, which has been earlier included in the preconfigured regroup group, no longer meets the MC service user criteria known at the MC service server.

2.	The MC service sends MC service client 2 a preconfigured user regroup remove request.

3.	The MC service client 2 notifies the MC service user of the preconfigured user regroup removal.

4.	The MC service client 2 may send the MC service server a preconfigured user regroup remove response to acknowledge the preconfigured user regroup removal.

5.	The MC service server updates the preconfigured regroup group (including the MC service user ID list) and potential group communication.

6.	The MC service server detects that the MC service client 3 meets the MC service user criteria.

7.	The MC service server sends the MC service client 3 a preconfigured regroup request.

8.	The MC service client 3 notifies the MC service user of the preconfigured regroup request.

9.	The MC service client 3 may send the MC service server a preconfigured regroup response.

10.	The MC service server updates the preconfigured regroup group (including the MC service user ID list), and potential group communication.

# * * * * Second change* * * *

#### 10.15.2.xx	Preconfigured user regroup remove request (MC service server – MC service client)

Table 10.15.2.xx-1 describes the information flow preconfigured user regroup remove request from the MC service server to MC service client.

Table 10.15.2.xx-1 Preconfigured user regroup remove request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user to be removed from the regroup group |
| MC service group ID | M | MC service group ID of the regroup group from which the MC service user is to be removed |

#### 10.15.2.xy	Preconfigured user regroup remove response (MC service client – MC service server)

Table 10.15.2.xy-1 describes the information flow preconfigured user regroup remove response from the MC service client to the MC service server.

Table 10.15.2.xy-1 Preconfigured user regroup remove response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user to be removed from the preconfigured regroup |
| Result | M | Result of the preconfigured user regroup removal operation |

# * * * * End of changes * * * *