3GPP TSG-SA WG6 Meeting #54e	S6-231616

e-meeting, 17th – 26th April 2023	(was S6-231500, S6-231169)

Source:	Deutsche Telekom

Title:	Information flows for predictive inter-PLMN slice service continuity

Spec:	3GPP TS 23.435 v.1.1.0

Agenda item:	8.10

Document for:	Approval

Contact:	Vasil Aleksiev, vasil.aleksiev@magenta.at

1. Introduction

The NSCE is part of application enabler/SEAL and TS23.435 has a procedure for predictive inter-PLMN slice service continuity.

2. Reason for Change

The existing procedure does not have information flows and APIs described.

4. Proposal

This pCR propose to introduce information flows and APIs for existing predictive inter-PLMN slice service continuity procedure.

* * * First Change * * * *

### 9.13.3	Information flows

#### 9.13.3.1	General

The following information flows are specified for predictive inter-PLMN slice service continuity:

-	Inter-PLMN application service continuity requirement

- 	Slice modification notification

#### 9.13.3.2	Inter-PLMN application service continuity requirement request

Table 9.13.3.2-1 and Table 9.13.3.2-2 describe information elements inter-PLMN application service continuity requirement request and response between the VAL server and the NSCE server.

Table 9.13.3.2-1: Inter-PLMN application service continuity requirement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Service Continuity Requirement | M | The service continuity requirement which can be the expected or predicted migration of the VAL application or a list of VAL UEs within the application to a target area. |
| Target PLMN ID | M | PLMN identifier of the target PLMN |
| Slice identifier | O | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Target Service Area | O | The target area can be represented as the geographical coordinates / set of waypoints outside the original service area, where the VAL application/ UE(s) is expected or predicted to move. |
| Application QoS requirements | O | The QoS requirements / KPIs for the VAL service |

Table 9.13.3.2-2: Inter-PLMN application service continuity requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement) |

#### 9.13.3.3	Inter-PLMN slice modification notify

Table 9.13.3.3-1 describes information elements for the inter-PLMN slice modification notify message from the NSCE server to the VAL server or the VAL client (via NSCE client).

Table 9.13.3.3-1: Inter-PLMN slice modification notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The identifier of the VAL application which is impacted by the slice modification |
| VAL UE ID list | O | The identifiers of the VAL UEs which are impacted by the slice modification |
| Slice identifier | M | The slice identifier (S-NSSAI, NSI ID or ENSI) which is used and/or modified to extend slice availability to the target service area |
| PLMN ID | M | PLMN identifier of the PLMN where modification was performed |
| Target Service Area | M | The target area, can be represented as the geographical coordinates / set of waypoints outside the original service area, for which the modification applies. |

### 9.13.4	APIs

#### 9.13.4.1	General

Table 9.13.4.1-1 illustrates the API for inter-PLMN application service continuity exposure.

Table 9.13.4.1-1: Inter-PLMN application service continuity requirement

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_InterPLMN_Continuity | Inter-PLMN_Continuity_Requirement | Request /Response | VAL server |
| SS_NSCE_InterPLMN_Slice_Modification_Notify | Inter-PLMN_Slice_Modification_Notify | Notify | VAL server or VAL client |

#### 9.13.4.2	SS_NSCE_Inter-PLMN_Continuity API

API operation name: Inter-PLMN_Continuity_Requirement

Description: The consumer requests to have inter-PLMN slice service continuity

Known Consumers: VAL server.

Inputs: See table 9.13.3.2-1.

Outputs: See table 9.13.3.2-2.

See clause 9.13.2 for details of usage of this operation.

#### 9.13.4.3	SS_NSCE_Inter-PLMN_slice modification notify API

API operation name: Inter-PLMN_Slice_Modification_Notify

Description: The NSCE notifies about slice modification

Known Consumers: VAL server.

Inputs: None.

Outputs: See table 9.13.3.3-1.

See clause 9.13.2 for details of usage of this operation.

* * * End of Changes * * * *