3GPP TSG-SA WG6 Meeting #55	S6-231739

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

# Source:	InterDigital

# Title:	New SID on further enhancements of the Edge Enablement Layer

# Document for:	Approval

# Agenda Item:	10

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on further enhancements of the Edge Enablement Layer

Acronym:	FS_EDGEAPP_Ph3

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| SMARTER | S1 | 720005 | (Stage 1 of 5G) New Services and Markets Technology Enablers |

Editor's Note:	The above study will be replaced with the normative parent work item as soon as this is approved.

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | System aspects for Edge Computing (SA2) |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | Security aspects of Edge Computing (SA3) |
| 870029 | Study on enhancements of edge computing management | OAM aspects of Edge Computing (SA5) |
| 880030 | Study on charging aspects of Edge Computing | Charging aspects of Edge Computing (SA5) |
| 880042 | Architecture for enabling Edge Applications | Application architecture for Edge Enabler Layer (SA6) |

# 3	Justification

3GPP SA6 within its Rel-17 work on TS 23.558 defined the overall application layer architecture to enable edge applications over 3GPP networks.

Rel-17 work includes fundamental features such as ECS discovery, service provisioning, EAS discovery, EEC/EAS/EES registrations, network and Edge Enabler Layer capability exposure, service continuity support with seamless service continuity and EEC context continuity etc., along with cardinality rules, deployment options, involved relationships and mapping with ETSI MEC and GSMA OP architectures.

In Rel-18, the Edge Enabler Layer (EEL) architecture was enhanced to support emerging industry requirements and complete functionalities that were not fully specified in Rel-17.

Rel-18 work includes new architecture requirements and enhancements to Rel-17 edge application architecture and support for enhancements such as enhanced notification service, enablement of service APIs exposed by EAS, definition of EDGE-5 APIs, application traffic filter exposure, linkage between EASs, enhancements to service continuity planning, simultaneous EAS connectivity in ACR, support of NAT deployed within the edge data network, EAS selection synchronization, application traffic influence for initially selected EAS, EEL service differentiation, ACR scenario combination, alignment of EDGEAPP and ETSI MEC, enhancement of dynamic EAS instantiation triggering, ACR between EAS and cloud application server, support of constrained devices for Edge, support for EEL SEAL capability access, edge services support across ECSPs, support for roaming UEs, edge enabler layer support for EAS synchronization, discovery of a common EAS, support of federated EAS service, EAS discovery in edge node sharing, reliable Edge service.

Rel-17 and Rel-18 mostly focussed on defining and enhancing the Edge Enablement Layer (EEL) capabilities. In parallel, other 3GPP study and work items that could benefit from edge computing capabilities were independently developed. This may have resulted in vertical deployments that will have limited, inappropriate, or impossible edge computing support. It is necessary, to foster edge computing adoption, that different type of application deployments and diversified edge application verticals are supported by the EEL.

In Rel-19, the Edge Enabler Layer (EEL) architecture requires enhancements for enabling diversified edge verticals and other enablement layers that benefit from edge computing, such as metaverse, AI/ML, PIN, UAV, V2X; full support for Rel-18 edge collaborative use cases that may not be complete, such as group of UEs, EAS bundles, composite EAS and combination thereof, including ACR, SCP and other aspects not completed in Rel-18; EEL performance monitoring, EEL reliability and possible enhancements to features documented in TS 23.558 may also be considered.

# 4	Objective

The SA6 objectives of this study item include the following:

- Identify key issues, develop corresponding architectural requirements and potential enhancements to the edge enablement layer as required, including:

  - support for Metaverse vertical edge enablers;

  - support for AI/ML vertical edge enablers;

  - support for PIN enablement layer edge enablers;

  - support for UAV enablement layer edge enablers;

  - support for V2X enablement layer edge enablers;

  - full support of collaborative use cases (e.g., common EAS, EAS bundles, composite EAS), including ACR, SCP and other aspects which were not completed in Rel-18;

  - support for EEL performance monitoring;

  - support for EEL reliability;

- identify potential solutions as required, including the information flows and the APIs satisfying the architectural requirements and enhancements identified in bullet 1) above.

# Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.abc | Study on further enhancements on Application Architecture for enabling Edge Applications | TSG#103 | TSG#104 | Michel Roy, InterDigital michel.roy@interdigital.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Roy, Michel, InterDigital <michel.roy@interdigital.com>

# 7	Work item leadership

SA6

# 8	Aspects that involve other WGs

SA2 for system aspects, SA3 for security aspects.

# 9	Supporting Individual Members

Supporting IM name

InterDigital