3GPP TSG-SA WG6 Meeting #55	S6-231860

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	MEC and EDGEAPP deployments in relation with CAPIF

Spec:	3GPP TR 23.958 v0.4.0

Agenda item:	EDGEAPP_EXT

Document for:	Approval

Contact:	diego.preciado_rojas@nokia.com

1. Introduction

In general ETSI MEC can be seen as an AF which may interact with 5GS to influence traffic routing decisions, access network capabilities (monitoring, provisioning, policy handling, charging, etc.), either directly (if the MEC is considered a trusted AF) or indirectly through the NEF (if the MEC is considered an untrusted AF). NEF may support CAPIF functionality for network exposure (specifically API provider domain functions). On the EDGEAPP side, the Edge Enabler Layer exposes Service APIs towards the EASs that include capabilities provided by EES as well as capabilities provided by the 3GPP core network: the EES may re-expose the network capabilities of the 3GPP core network to the EAS(s) as per the CAPIF architecture. Therefore, an integration/coexistance of ETSI MEC and EDGEAPP through CAPIF is pertinent. Such an integration/coexistance allows also EASs to invoke MEC App services and vice versa.

2. Reason for Change

Supply some informative content to allow an alignment between EDGEAPP and ETSI MEC using CAPIF for exposing and invoking Service APIs (exposed from both platforms). Otherwise, EAS/MEC APP access to services offered by ETSI MEC from EDGEAPP (and vice versa) is not clear, neither the access to northbound APIs exposed by SCEF/NEF.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]		GSMA PRD OPG.02 - “Operator Platform Telco Edge Requirements Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2022/10/Operator-Platform-Telco-Edge-Requirements.-v.3-October22.pdf.

[4]	ETSI GS MEC 003 (V3.1.1), "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	ETSI GS MEC 011 (V3.1.1), “Multi-access Edge Computing (MEC); Edge Platform Application Enablement”.

[6]	ETSI GS MEC 040 (V3.1.1), “Multi-access Edge Computing (MEC); Federation enablement APIs”.

[7]	3GPP TS 23.222: Common API Framework for 3GPP Northbound APIs

[8]	ETSI GS MEC 031 (V3.1.1), “Multi-access Edge Computing (MEC); MEC 5G Integration”.

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

AEF	API Exposing Function

AF	Application Function

AMF	API Management Function

API	Application Program Interface

APF	API Publishing Function

AS	Application Server

CAPIF	Common API Framework

CCF	CAPIF Core Function

EAS	Edge Application Server

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EEL	Edge Enabler layer

EES	Edge Enabler Server

ETSI	European Telecommunications Standards Institute

MEC	Multi-access Edge Computing

MEP	MEC Platform

* * * Next Change * * * *

# 7	Deployment Considerations

This section captures different deployment possibilities of aligned EDGEAPP and ETSI MEC in an operator network and any other deployment considerations w.r.t EDGEAPP, ETSI MEC and GSMA OP as per TS 23.558.

## 7.1	Deployment Alignment between MEC and EDGEAPP

### 7.1.1	Deployments in relation with CAPIF

3GPP provides a framework for supporting common capabilities for northbound APIs (e.g., onboarding, authentication, authorization, discovery, auditing, etc.) through the so-called common API framework (CAPIF) [7]. The functional model of CAPIF enables an API invoker to access (and invoke) service APIs and supports API exposing functions in publishing the API towards the API invokers. To enable the common supporting capabilities among API invokers and API providers, the CCF is introduced as the functional entity in charge of, among other tasks, authenticating an API invoker, providing authorization for the API invoker prior to accessing the service API, publishing, storing and supporting the discovery of service APIs information, etc.

As from Annex A in TS 23.558 [2], an EAS may assume the role of:

- API invoker: for accessing northbound APIs exposed by SCEF/NEF or consuming service APIs offered by other EASs, or

- API provider: exposing its service APIs for consumption by other API invokers,

whereas the EES may act like AEF and CCF (the latter being dispensable if, e.g., a centralized instance of CCF is available, see Clause A.5.3 in TS 23.558 [2]).

Additionally, as from Annex B in ETSI GS MEC 031 [8], and as it is reproduced in Figure 7.1.1-1 (depicting only the MEC host), a MEC App may assume the role of:

- API invoker: for accessing northbound APIs exposed by SCEF/NEF or consuming MEC services available in the MEP, or

- API provider: registering (publishing) its service APIs and exposing them for consumption by other API invokers,

whereas the MEP may assume the role of API provider, CCF, and API invoker.

Figure 7.1.1-1: Relationship of MEC functional entities/reference points and CAPIF

As shown in dashed lines in Figure 7.1.1-1, a MEC service produced by a MEC App or MEP can be mapped into the API provider domain in CAPIF. A MEC App or MEP consuming a service can be mapped into an API invoker in CAPIF. MEP includes a service registry which covers parts of the functionality of the CCF.

Since the different functional entities of both EDGEAPP and ETSI MEC may assume several roles, the number of scenario combinations is high, which becomes worse if several trust domains are considered. Therefore, rather than providing an exhaustive study of possible CAPIF-based deployments, alignment is presented based upon the generic scenario presented in Figure 7.1.1-2, in which it is assumed that:

- MEC platform and EDGEAPP co-exist in the network within the same PLMN trust domain,

- API invokers want to access MEC services provided by MEP or MEC Apps, or API invokers want to consume EDGEAPP services provided by EASs,

- REST-HTTP transport is used

NOTE 1:	Enhancements of the functional entities might be needed to fully support the functionalities of the CAPIF functional entities, namely CCF, API provider domain functions and API invoker

NOTE 2:	Extension of the reference point in both platforms might be needed to support the functionalities of the CAPIF-related reference points (see reference points in Figure 7.1.1-2)

Figure 7.1.1-2: Synergized CAPIF-based deployment of EDGEAPP and ETSI MEC

On the left-hand side of Figure 7.1.1-2, EES implements CCF 1 to manage the Service APIs exposed by EAS and EES, EAS implements the API Provider domain functions to expose EAS Service APIs to EDGEAPP EASs (and possibly ETSI MEC Applications acting as API invokers). EES implements API Provider domain functions to expose EES Service APIs (e.g., UE location API, AC information exposure API, etc.) to API invokers (e.g., EASs, ETSI MEC Applications).

On the right-hand side of Figure 7.1.1-2, MEP implements an instance of CCF (i.e., CCF 2) to manage the MEC Services exposed by MEP and MEC App. Both MEP and MEC App implement API Provider domain functions to expose MEC Services via CAPIF-2.

CCF 1 and CCF 2 interact via CAPIF-6 reference point, which supports publishing the service APIs information and discovering the service APIs information on both platforms. The CCF 1 and the CCF 2 publish the service API provided by its connected API exposing function(s) and obtains the service API information provided by other CCF.

An API invoker (e.g., EAS or MEC App) connected to the any of the CCF can discover and invoke service APIs provided by the API exposing function connected to the othe CCF.