3GPP TSG-SA6 Meeting #55	S6-231926

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0326 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EES specfic UE ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In meeting SA6#54-e the contribution S6-23635 has been agreed which introduced an EES specific UE ID to be used by the EEC for EDGE-1 operations. This leads to a situation where the EEC now could use the EES specific UE ID to identify the UE and hence it can be seen as an alternative to GPSI. This contruibution proposes to include the EES specific UE ID as an alternative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 8.6.5.2 and clause 8.6.5.3.2 the EES specific UE ID is used as an alternative to GPSI. The changes from the pervious agreed contribution which introduces the EES specific UE ID is highlighted in Yellow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EES specific UE ID usage is still not clear in relation with the GPSI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-231635 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.6.5.2	Procedure

Figure 8.6.5.2-1 illustrates the interactions between the EES and the EAS.

Pre-conditions:

1.	The EAS or EEC is authorized to discover and to use UE Identifier API provided by the EES.

2.	When the EEC is used to invoke the UE Identifier API with the UE IPv6 address as the input parameter, the UE IPv6 address may or may not be NATed. If NATed however, the IPv6 may not be reused (i.e. assigned to more than one UE simultaneously). If the EEC already has the UE ID (GPSI as per clause 7.2.6), and it needs the Edge UE ID to share with an AC/EAS, this procedure can still be used to retrieve Edge UE ID.

3.	EAS is considered an AF behind EES (as another AF) and EES is authorized to pass EAS ID instead of its own AF ID when it needs to interact with the NEF’s Nnef_UEId_Get (as per TS 23.502 clause 4.15.10 "AF specific UE ID retrieval").

Editor's note:	[SA3] [SA2] Check pre-condition #3 to ensure EES is Ok to pass on EAS ID in NEF’s Nnef_UEId as opposed to its own AF ID.

Figure 8.6.5.2-1: UE Identifier API

1.	The EAS or EEC invokes UE Identifier API exposed by the EES. If it is the EAS invoking the API and it recognizes that the UE’s IP address is a public IP address, i.e., the UE is behind a NAT, the Port Number and associated IP address should be included in user information.

2.	The EES uses the received user information in the step 1 (e.g. IP address) and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3]. If it is the EEC invoking the API with only UE IP address, it shall be interpreted by EES that EEC is requesting the UE ID for interaction with EES (hence EES shall use its own AFId towards NEF and consequently the UE ID is EES specific).

3.	The EES provides the UE identifier to the EAS or to EEC (i.e. whichever invoked the API). The UE identifier returned in the response which is referred to as UE ID may be the 3GPP Core Network assigned UE ID (aka AF-sepecific UE ID; see TS 23.502 clause 4.15.10) or the EES-generated Edge UE ID as defined in clause 7.2.x. If UE ID is included in the request received from EEC (GPSI as per clause 7.2.6 or UE ID which is EES specific provided to the EEC), the EES can provide the Edge UE ID based on the received UE ID and step 2 can be skipped. For EEC requesting the UE ID for interaction with EES, the EES returns its 3GPP Core Network assigned UE ID (aka AF specific UE ID) to the EEC.

Editor's note:	[SA3] Whether and how user's consent is obtained to share the UE identifier with a particular EAS or EEC is SA3's responsibility.

4.	The EAS uses the UE ID received in step 3 to invoke capability exposure API(s) provided by the EES over EDGE-3 and/or EDGE-7 depending on the UE ID type. The EEC can use the UE ID which is EES specific received in step 3 to invoke API(s) provided by the EES over EDGE-1 reference point.

NOTE:	UE ID of type CN-assigned can be used over EDGE-1, EDGE-3 and EDGE-7 whereas the UE ID of type EES-generated Edge UE ID can be used over EDGE-3 only.

* * * Next Change * * * *

##### 8.6.5.3.2	UE Identifier API request

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| User information (see NOTE 1) (see NOTE 3) | O | Information about the User or UE available in the EAS or EEC, e.g. IP address. |
| UE ID (see NOTE 2) (see NOTE 3) | O | UE ID in the form of GPSI as per clause 7.2.6 or UE ID which is EES specific provided to the EEC. |
| EAS ID list | O | Identifier of the EAS(s) for which the UE IDs are requested for by EAS or EEC given the User information (e.g. IP address). |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Security Credentials | M | Security credentials of the EAS or EEC. |
| NOTE 1: When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. When EAS invokes the API, it may recognize the UE IP address is a public IP address different from the actual UE IP address (private IP address), i.e., the UE is behind a NAT, and should therefore include the Port Number and associated IP address as part of the User information. NOTE 2: This IE is used when invoked by the EEC and if the EEC hasve the UE ID already in a form not desired to be shared with the EAS. NOTE 3: At least one of them shall be present. |  |  |

Editor's Note: [SA3] Whether the EAS ID and the EAS Provider ID are part of the security credential is SA3's responsibility.

Editor's Note: [SA3] It is FFS whether the IP address (i.e. User information) provided in the request can be verified that is indeed the UE’s and whether the UE provided infomation (IP Address) can be trusted.

Editor's Note: It is FFS to determine if EASID (clause 7.2.4) definitions need to be updated in order to be consistent with its use (as per EAS ID list IE) as an AF ID in UE Identifier API.

##### 8.6.5.3.3	UE Identifier API response

Table 8.6.5.3.3-1: UE Identifier API response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the UE identifier request was successful. |
| > UE ID list | M | List of all the UE IDs Identifier uniquely identifying the UE(s). |
| >> UE ID | M | AF-specific UE ID or Edge UE ID |
| >> UE ID type | M | Indication whether the UE ID is CN assigned AF-specific UE ID or Edge UE ID. |
| >> EAS ID | O | It is present if the EAS ID was provided in the request (see EAS ID list in Table 8.6.5.3.2-1). |
| Failure response | O | Indicates that the UE identifier request failed. |
| > Cause | O | Indicates the cause of UE identifier request failure |

* * * END of Change * * * *