3GPP TSG-SA WG6 Meeting #55	S6-232018

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231841)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0373 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Details of EAS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following Editor’s Note: Editor's Note: In the information flows the details of EAS information is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added details of the EAS information in the information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.2.5.1, 8.14.3.12, 8.14.3.13, 8.14.3.14, 8.14.3.15, 8.14.3.16, 8.14.3.17, 8.14.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.14.2.5.1	General

Following are supported for EEC services subscription:

-	Subscribe;

-	Notify;

-	Subscription update; and

-	Unsubscribe.

* * * Next Change * * * *

#### 8.14.3.12	EEC services subscription request

Table 8.14.3.12-1 describes information elements for the EEC services subscription request sent by the AC to the EEC.

Table 8.14.3.12-1: EEC services subscription request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile of the AC requesting subscription |
| Security credentials | M | Security credentials of the AC. |
| Notification details | M | Details on how the EEC can notify the AC. |
| Required EEC services | M | EEC handled services required by the AC |
| > EAS discovery | O | Indicates that AC requires EEC to handle EAS discovery. |
| >> List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |
| > EAS dynamic information | O | Indicates that the AC requires EAS dynamic information subscription |
| >> List of EAS dynamic information filters | M | List of EAS dynamic information filters as described in Table 8.5.3.4-2. |
| > ACR | O | Indicates that the AC requires ACR related services for the following EASs |
| >> List of S-EAS information | M | S-EAS information corresponding to the ACR operation |
| >>> EASID | M | Identifier of the S-EAS to which the subscription relates. |
| >>> EAS endpoint | M | Endpoint of the S-EAS to which the subscription relates. |
| >>> ACR operation | O | Indicates if AC requires ACR monitoring or EEC managed ACR corresponding to each S-EAS |

#### 8.14.3.13	EEC services subscription response

Table 8.14.3.13-1 describes information elements for the EEC services subscription response sent by the EEC to the AC.

Table 8.14.3.13-1: EEC services subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EEC services subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Confirmed EEC services | M | List of services EEC confirms to handle |
| >> EAS discovery | O | Indicates that the EEC will handle EAS discovery |
| >> EAS dynamic information | O | Indicates that the EEC will handle EAS dynamic information subscription |
| >> ACR | O | Indicates that the EEC will handle ACR. |
| >>> List of S-EAS information | M | Information of the S-EASs EEC will handle. |
| >>>> EASID | M | Identifier of the S-EAS to which the subscription relates. |
| >>>> EAS endpoint | M | Endpoint of the S-EAS to which the subscription relates. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

#### 8.14.3.14	EEC services notification

Table 8.14.3.14-1 describes information elements for the EEC services notification sent by the EEC to the AC.

Table 8.14.3.14-1: EEC services notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the notification. |
| Notification details | M | Information related to the notification |
| > EAS discovery | O | Indicates that the notification relates to EAS discovery |
| >> List of EAS profiles | M | EAS profiles associated with the notification. Each element is described in clause 8.2.4 |
| >>> Lifetime | O | Time interval or duration during which the information in the EAS profile is valid and supposed to be cached in the AC (e.g. time-to-live value for an EAS Endpoint) |
| > EAS dynamic information | O | Indicates that the notification relates to EAS dynamic information subscription |
| >> List of EAS profile | M | EAS profiles associated with the notification. Each element is described in clause 8.2.4 |
| >>> Lifetime | O | Time interval or duration during which the information in the EAS profile is valid and supposed to be cached in the AC (e.g. time-to-live value for an EAS Endpoint) |
| > ACR | O | Indicates that the notification relates to ACR |
| >> List of S-EAS information | M | ACR operation information and information of the EAS to which the notification relates. |
| >>> EASID | M | Identifier of the S-EAS to which the subscription relates. |
| >>> EAS endpoint | M | Endpoint of the S-EAS to which the subscription relates. |
| >>> ACR detection | O | Indicates that the EEC detected a need for ACR |
| >>> ACR decision | O | EEC’s decision information related to an ACR detection |
| >>> ACR execution status | O | Indicates the status of ACR execution |
| >>>> Status | M | ACR status such as initiated, in-progress, complete |
| >>>> T-EAS information | O | Information of the T-EAS corresponding to the S-EAS |

#### 8.14.3.15	EEC services subscription update request

Table 8.14.3.15-1 describes information elements for the EEC services subscription update request sent by the AC to the EEC.

Table 8.14.3.15-1: EEC services subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the update request. |
| Security credentials | M | Security credentials of the AC. |
| Notification details | O | Details on how the EEC can notify the AC. |
| Required EEC services | O | EEC handled services required by the AC |
| > EAS discovery | O | Indicates that AC requires EEC to handle EAS discovery. |
| >> List of EAS discovery filters | M | List of EAS discovery filters as described in Table 8.5.3.2-2. |
| >>> Preferred ECSP list | O | Preferred ECSP list corresponding to each set of EAS discovery filters. |
| > EAS dynamic information | O | Indicates that the AC requires EAS dynamic information subscription |
| >> List of EAS dynamic information filters | M | List of EAS dynamic information filters as described in Table 8.5.3.4-2. |
| > ACR | O | Indicates that the AC requires ACR related services for the following EASs |
| >> List of S-EAS information | M | S-EAS information corresponding to the ACR operation |
| >>> EASID | M | Identifier of the S-EAS to which the subscription relates. |
| >>> EAS endpoint | M | Endpoint of the S-EAS to which the subscription relates. |
| >>> ACR operation | O | Indicates if AC requires ACR monitoring or EEC managed ACR corresponding to each S-EAS |

#### 8.14.3.16	EEC services subscription update response

Table 8.14.3.16-1 describes information elements for the EEC services subscription update response sent by the EEC to the AC.

Table 8.14.3.16-1: EEC services subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EEC services subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Confirmed EEC services | M | List of services EEC confirms to handle |
| >> EAS discovery | O | Indicates that the EEC will handle EAS discovery |
| >> EAS dynamic information | O | Indicates that the EEC will handle EAS dynamic information subscription |
| >> ACR | O | Indicates that the EEC will handle ACR |
| >>> List of S-EAS information | M | Information of the S-EASs EEC will handle. |
| >>> EASID | M | Identifier of the S-EAS to which the subscription relates. |
| >>> EAS endpoint | M | Endpoint of the S-EAS to which the subscription relates. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

#### 8.14.3.17	EEC services unsubscribe request

Table 8.14.3.17-1 describes information elements for the EEC services unsubscribe request sent by the AC to the EEC.

Table 8.14.3.17-1: EEC services unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | EEC services subscription identifier provided by the EEC during subscription. |
| Security credentials | M | Security credentials of the AC sending the EEC services unsubscribe request. |

#### 8.14.3.18	EEC services unsubscribe response

Table 8.14.3.18-1 describes information elements for the EEC services unsubscribe response sent by the EEC to the AC.

Table 8.14.3.18-1: EEC services unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EEC services unsubscribe request was successful. |
| Failure response | O | Indicates that the EEC services unsubscribe request failed. |
| > Cause | M | Provides the cause for EEC services unsubscribe request failure. |