3GPP TSG-SA WG6 Meeting #55	S6-232041

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231774)

Source:	China Mobile

Title:	Add the archirtecture requirements

Spec:	3GPP TS 23.433 v1.3.0

Agenda item:	8.14

Document for:	Approval

Contact:	Xiaohui Shi, shixiaohui@chinamobile.com

1. Introduction

Add the archirtecture requirements in TS 23.433 v1.3.0

2. Reason for Change

This contribution proposes to specify the Data transmission rate control requirements

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.433 v1.3.0.

* * * First Change * * * *

6.2	Data transmission requirements

6.2.1	Description

This clause specifies the data transmission requirements for SEALDD service.

6.2.2	Requirements

[AR-6.2.2-a] The SEALDD service shall provide a mechanism for application signalling data transmission and application media data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-b] The SEALDD service shall provide a mechanism to support the data transmission quality requirement configurations and measurements for the application data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-c] The SEALDD service shall provide a mechanism for application data transmission between VAL client(s) and VAL server(s) with guaranteed quality.

[AR-6.2.2-d] The SEALDD service shall provide a mechanism for application data packaging and un-packaging to support the data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-e] The SEALDD service shall provide a mechanism for E2E redundant data transmission between VAL client and VAL server.

[AR-6.2.2-f] The SEALDD service shall provide a mechanism to support the packet/data duplication, elimination and error recovery between VAL client and VAL server.

6.3	Data storage requirements

6.3.1	Description

This clause specifies the data storage requirements for SEALDD service.

6.3.2	Requirements

[AR-6.3.2-a] The SEALDD service shall provide a mechanism for data storage supporting the CRUD operations.

[AR-6.3.2-b] The SEALDD service shall provide a mechanism to support the data storage status management.

6.4	SEALDD server discovery and selection requirements

6.4.1	Description

This clause specifies the SEALDD server discovery and selection requirements for SEALDD service.

6.4.2	Requirements

[AR-6.4.2-a] The SEALDD service shall provide a mechanism for supporting the SEALDD server discovery and selection for VAL server.

[AR-6.4.2-b] The SEALDD service shall provide a mechanism to provide the information of SEALDD server to VAL/SEALDD client.

6.5	MSGin5G message transfer requirements

6.5.1	Description

This clause specifies the MSGin5G message transfer requirements for SEALDD service.

6.5.2	Requirements

[AR-6.5.2-a] The SEALDD service shall provide a mechanism to support the SEALDD traffic transmission using MSGin5G message.

6.6	Data transmission bandwidth control requirements

6.6.1	Description

This clause specifies the data transmission bandwidth control requirements for SEALDD service.

6.6.2	Requirements

[AR-6.6.2-a] The SEALDD service shall provide a mechanism to support the transmission bandwidth control for VAL application.