3GPP TSG-SA WG6 Meeting #55	S6-232094

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231702)

Source:	Samsung

Title:	Procedures and APIs for Network Slice Information delivery

Spec:	3GPP TS 23.435 v1.2.0

Agenda item:	8.10

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

This pCR proposes procedures for subscription and notification for NS information delivery.

2. Reason for Change

This pCR proposes to remove the EN “Subscribe-notify procedure for Network Slice Information delivery will be defined” without introducing subscribe-notify model for Network Slice Information devliery. Also, this contribution provides API description for Network Slice Information.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v1.2.0.

* * * First Change * * * *

9.17.2.1	Network Slice Information delivery request

This subclause depicts the procedure of the Network Slice Information delivery to the VAL server via NSCE server, when the VAL server requests the Network Slice Information after registration.

Pre-condition:

1. 	The NSCE server should have the agreement with MNO (NOP) for retrieval of ServiceProfile, if the NSCE server is the external entity.

Figure 9.17.2.1-1: Network Slice Information delivery request

1.	The NSCE server retrieves the Network Slice ServiceProfile from 5GS (e.g., NSMF) when the NSCE server acting as a NSP prepares a Network Slice to be provided. The NSCE server follows the procedure to request/receive the Network Slice Service Profile with 'getMOIAttributes' operation as specified in 3GPP TS 28.532[7].

NOTE:	If NSCE server and NSMF are in same operator, then the NSCE server gets access directly to NSMF.      The delivered Network Slice Service Profile contains the values of attributes such as PLMN, S-NSSAI, SST, maximum number of UEs, maximum number of PDU sessions, Slice Coverage Area, Latency, and, Data volume, which specify the Network Slice characteristics, as specified in clause of ServiceProfile in 3GPP TS 28.541 [10].

2.	The NSCE server, as Network Slice as a Service, creates and stores the Network Slice information. When NSCE server retrieves the Network Slice Information, it is necessary for NSCE server to convert the attributes in Network Slice ServiceProfile to the Network Slice information for readable information and to compose the Network Slice information, according to the NSP's policy.

In order to reduce to request often the Network Slice Information Retrieval, the NSCE server stores the Network Slice information.

3.	The VAL server requests the Network Slice Information to the NSCE server. If the VAL server needs to know the specific attribute value for its service, then the attribute name of Network Slice Information (e.g., S-NSSAI, SST, Slice Coverage Area, etc.) can be added in the Request message.

4.	The NSCE server performs to check whether the requesting VAL server is registered or not. The NSCE server identifies which the Network Slice Customer is authorized to use.

5.	The NSCE server sends the Network Slice Information, if the VAL server is registered and authorized. The NSCE server rejects to the request of the Network Slice Information, if not registered.

* * * Next Change * * * *

9.17.4	APIs

9.17.4.1	General

Table 9.17.4.1-1 illustrates the APIs for the Network Slice Information delivery.

Table 9.17.4.1-1: List of APIs for Network Slice Information delivery

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_NSCE_NSInfoDelivery | NSInfoDelivery_request | Request/Response | VAL Server |

9.17.4.2	SS_NSCE_NSInfoDelivery Get operation

API operation name: SS_NSCE_NSInfoDelivery Get

Description: The consumer requests to get Network Slice Information.

Inputs: See clause 9.17.3.1.

Outputs: See clause 9.17.3.2.

See clause 9.17.2.1 for details of usage of this operation.

* * * End of Change * * * *