3GPP TSG-SA WG6 Meeting #55	S6-232147

Berlin, Germany 22nd – 26th May 2023	(revision of S6-232129)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0111 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service API status monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution#5 in TR 23.700-97 studied the service API status monitoring which requires the APF to publish/update the service API status to CCF. This is also applicable for inter-CCF service API publish. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add service API status in service API publish/update and interconnection API publish request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service API status monitoring feature is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1, 8.3.2.2, 8.6.2.1, 8.6.2.2, 8.25.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, API provider name (optional), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Shareable information | O (see NOTE) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API is not allowed to be shared. |  |  |

The Service KPIs is defined as below:

Table 8.3.2.1-2: Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |

* * * Next Change * * * *

#### 8.3.2.2	Service API publish response

Table 8.3.2.2-1 describes the information flow service API publish response from the CAPIF core function to the API publishing function.

Table 8.3.2.2-1: Service API publish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of publishing the service API information |
| Service API published information reference | O (see NOTE) | The information which can be used for referencing the information (set) about the published service API by the API publishing function. |
| Service API information | O (see NOTE) | The authorized service API information. |
| NOTE: This information element is included when the Result indicates success. |  |  |

* * * Next Change * * * *

#### 8.6.2.1	Service API update request

Table 8.6.2.1-1 describes the information flow service API update request from the API publishing function to the CAPIF core function.

Table 8.6.2.1-1: Service API update request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API published information reference | M | The information (set) provided by the CAPIF core function about the published service API which can be used for reference by the API publishing function. |
| Service API information | M | The service API information includes the service API name, service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format which is required to replace the existing service API information |
| Reason | O | The reason of the update (e.g. change log) |

Editor's Note: It is FFS how to monitor service API status when the APF is unable to update service API status.

* * * Next Change * * * *

#### 8.6.2.2	Service API update response

Table 8.6.2.2-1 describes the information flow service API update response from the CAPIF core function to the API publishing function.

Table 8.6.2.2-1: Service API update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

* * * Next Change * * * *

#### 8.25.2.1	Interconnection API publish request

Table 8.25.2.1-1 describes the information flow interconnection API publish request from CAPIF core function to CAPIF core function.

Table 8.25.2.1-1: Interconnection API publish request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which publishes APIs, may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information includes the service API name, API provider name (optional), service API type, service API status (e.g. active, inactive),communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Service API category | O (see NOTE 1) | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O (see NOTE 2) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE 1: At least one of the Service API information or Service API category shall be present. NOTE 2: If the shareable information is not present, the service API is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

* * * END of Change * * * *