3GPP TSG-SA WG6 Meeting #56	S6-232295

Goteborg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0117 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Specification of revocation procedure for adding default value on AEF and triggering by resource owner client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A revocation procedure of RNAA needs to be corrected about triggering by AEF. There is a use case where user wants to restore to a default value (e.g., modify QoS), but the procedure is not considered. Also, there is a use case that resource owner client sends revocation, but it is not specified in the exsisting TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The revocation procedure is added about default value at step5 and step5a of triggering by AEF. And, new revocation procedure specified on the TS for triggering by the resource owner client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The revocation procedure lacks about restoring default value and triggering by resource owner clinet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Exsisting clauses: 8.23.3, 8.23.4 New clauses: 8.23.X, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.23.3	Procedure for CAPIF revoking API invoker authorization initiated by AEF

Figure 8.23.3-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the AEF.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

3.	Authorization details of the AEF are available with the CAPIF core function.

Figure 8.23.3-1: Procedure for revoking API invoker authorization initiated by AEF

1.	The AEF triggers the revocation of the API invoker authorization.

2.	The AEF sends revoke API invoker authorization request to the CAPIF core function with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CAPIF core function invalidates the API invoker authorization corresponding to the service API.

4.	The CAPIF core function sends a revoke API invoker authorization response to the AEF.

5.	Upon successful revocation of API invoker authorization corresponding to the service API at the CAPIF core function, the AEF invalidates the API invoker authorization corresponding to the service API. Optionally, a default value to be restored successful revocation may be retained at the AEF as per the operator policy, the AEF restores to the default.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * Next Change * * * *

### 8.23.4	Procedure for CAPIF revoking API invoker authorization initiated by CAPIF core function

Figure 8.23.4-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the CAPIF core function.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

Figure 8.23.4-1: Procedure for revoking API invoker authorization initiated by CAPIF core function

1.	The CAPIF core function triggers the revocation of the API invoker authorization.

2.	The CAPIF core function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API.

4.	The AEF sends a revoke API invoker authorization response to the CAPIF core function.

5.	The CAPIF core function invalidates the API invoker authorization corresponding to the service API.

5a.	Optionally, a default value to be restored successful revocation may be retained at the AEF as per the operator policy, the AEF restores to the default.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * Next Change * * * *

### 8.23.X	Procedure for CAPIF revoking API invoker authorization initiated by resource owner client

Figure 8.23.x-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the resource owner client.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

Figure 8.23.x-1: Procedure for revoking API invoker authorization initiated by resource owner client

1.	The resource owner client triggers the revocation of the API invoker authorization to the CAPIF Core function.

2.	The CAPIF core function invalidates the API invoker authorization corresponding to the service API.

3.	The CAPIF core function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API.

4.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API. Optionally, the AEF restores the default values.

5.	The AEF sends a revoke API invoker authorization response to the CAPIF core function.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * End of Change * * * *