3GPP TSG-SA WG6 Meeting #56	S6-232348

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0432 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | APIs for EDGE-5 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Removal of EN in Clause 8.14.4 Editor's note: API details are FFS. It is requried to define APIs for EDGE-5 interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added APIs for all procedrues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EDGE-5 interface procedures will be incomplete and stage#3 will not be able to implement the same. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.4, 8.14.4.1 (new), 8.14.4.2 (new), 8.14.4.2.1 (new), 8.14.4.2.2 (new), 8.14.4.2.3 (new), 8.14.4.3 (new), 8.14.4.3.1 (new), 8.14.4.4 (new), 8.14.4.4.1 (new), 8.14.4.5 (new), 8.14.4.5.1 (new), 8.14.4.5.2 (new), 8.14.4.5.3 (new), 8.14.4.5.4 (new), 8.14.4.6 (new), 8.14.4.6.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.14.4	APIs

* * * Next Change * * * *

#### 8.14.4.1	General

Table 8.14.4.1-1 illustrates the API for AC registration.

Table 8.14.4.1-1: Eees_EECRegistration API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eeec_ACRegistration | Request | Request/Response | AC |
|  | Update |  |  |
|  | Deregister |  |  |
| Eeec_EASDiscovery | Request | Request/Response | AC |
| Eeec_ACRTrigger | Request | Request/Response | AC |
| Eeec_Services | Subscribe | Subscribe/Notify | AC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eeec_UEId | Request | Request/Response | EAS |

#### 8.14.4.2	Eeec_ACRegistration API

##### 8.14.4.2.1	Eeec_ACRegistration_Request operation

API operation name: Eeec_ACRegistration_Request

Description: The consumer requests to register the AC on the EEC.

Inputs: See clause 8.14.3.2.

Outputs: See clause 8.14.3.3.

See clause 8.14.2.2.2 for details of usage of this operation.

##### 8.14.4.2.2	Eeec_ACRegistration_Update operation

API operation name: Eeec_ACRegistration_Update

Description: The consumer requests to update the registered information of the AC on the EEC.

Inputs: See clause 8.14.3.4.

Outputs: See clause 8.14.3.5.

See clause 8.14.2.2.3 for details of usage of this operation.

##### 8.14.4.2.3	Eeec_ACRegistration_Deregister operation

API operation name: Eeec_ACRegistration_Deregister

Description: The consumer requests to de-register the AC from the EEC.

Inputs: See clause 8.14.3.6.

Outputs: See clause 8.14.3.7.

See clause 8.14.2.2.4 for details of usage of this operation.

#### 8.14.4.3	Eeec_EASDiscovery API

##### 8.14.4.3.1	Eeec_EASDiscovery_Request operation

API operation name: Eeec_EASDiscovery_Request

Description: The consumer requests for one time EAS discovery information.

Inputs: See clause 8.14.3.8.

Outputs: See clause 8.14.3.9.

See clause 8.14.2.3 for details of usage of this operation.

#### 8.14.4.4	Eeec_ACRTrigger API

##### 8.14.4.4.1	Eeec_ACRTrigger_Request operation

API operation name: Eeec_EASDiscovery_Request

Description: The consumer requests to trigger ACR.

Inputs: See clause 8.14.3.10.

Outputs: See clause 8.14.3.11.

See clause 8.14.2.4 for details of usage of this operation.

#### 8.14.4.5	Eeec_Services API

##### 8.14.4.5.1	Eeec_Services_Subscribe operation

API operation name: Eeec_Services_Subscribe

Description: The consumer subscribes for EEC Services related events.

Inputs: See clause 8.14.3.12.

Outputs: See clause 8.14.3.13.

See clause 8.14.2.5.2 for details of usage of this operation.

##### 8.14.4.5.2	Eeec_Services_Notify operation

API operation name: Eeec_Services_Notify

Description: The consumer is notified about EEC Services related events.

Inputs: See clause 8.14.3.14.

Outputs: None.

See clause 8.14.2.5.3 for details of usage of this operation.

##### 8.14.4.5.3	Eeec_Services_UpdateSubscription operation

API operation name: Eeec_Services_UpdateSubscription

Description: The consumer updates an existing subscription for EEC Services related events.

Inputs: See clause 8.14.3.15.

Outputs: See clause 8.14.3.16.

See clause 8.14.2.5.4 for details of usage of this operation.

##### 8.14.4.5.4	Eeec_Services_Unsubscribe operation

API operation name: Eeec_Services_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed EEC Services related events.

Inputs: See clause 8.14.3.17.

Outputs See clause 8.14.3.18.

See clause 8.14.2.5.5 for details of usage of this operation.

#### 8.14.4.6	Eeec_UEId API

##### 8.14.4.6.1	Eeec_UEId_Request operation

API operation name: Eeec_UEId_Request

Description: The consumer requests for EAS specific UE ID.

Inputs: See clause 8.14.3.19.

Outputs: See clause 8.14.3.20.

See clause 8.14.2.6 for details of usage of this operation.

* * * End of Change * * * *