3GPP TSG-SA WG6 Meeting #56	S6-232378

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0020 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN Service Continuity subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PIN Service Continuity Update procedure in clause 8.9.2.1.4 defines a request sent from the PIN Server to the AS and contains the following EN. Editor's note: Whether subscription/notification mechanisms are used for informing the AS over PIN-9 reference point is FFS. Because AS API is not standardized, it is proposed to replace the request sent to the AS by a notification and define the realted AS subscription mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Service continuity subscription/notification is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure cannot be realized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2.1.4, 8.9.2.1.6 (new), 8.9.2.1.6.1 (new), 8.9.2.1.6.2 (new), 8.9.2.1.6.2.1 (new), 8.9.2.1.6.2.2 (new), 8.9.2.1.6.2.3 (new), 8.9.2.1.6.2.4 (new), 8.9.3.1, 8.9.3.10 (new), 8.9.3.11 (new), 8.9.3.12 (new), 8.9.3.13 (new), 8.9.3.14 (new), 8.9.3.15 (new), 8.9.3.16 (new), 8.9.4, 8.9.4.1 (new), 8.9.4.2 (new), 8.9.4.3 (new), 8.9.4.4 (new), 8.9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.9.2.1.4	PIN Service Continuity Update procedure

The PIN Service Continuity Update procedure allows the PEMC to inform the PIN Server about a service continuity request received at the PEMC for a PINE, and it allows the PIN Server to inform the related AS of the same if such functionality is supported by the AS.

The PIN Server may provide, in the response, policy information to the PEMC about service continuity for the concerned PINE.

Pre-conditions:

1.	PINE has an application session that goes through a PEGC; and

2.	PEGC needs to be replaced (e.g., PEGC will become unavailable or PEGC is unreachable).

Figure 8.9.2.1.4-1: PIN Configuration Update procedure

1.	The PEMC sends a PIN Configuration Service Continuity Update request to the PIN Server. The PIN Configuration Service Continuity Update request includes security credentials, service continuity information, PINE identifier(s), PIN Gateway identifier, and Service identifier.

2.	Upon receiving the request, the PIN Server validates the request and checks if the PEMC is authorized to request service continuity update. If the request is authorized and valid, if the PIN Server has not authorized the PINE for service continuity, the PIN Server validates if the PINE is authorized and determines policy information about service continuity for the PINE. If the PIN Server has already authorized the PINE for service continuity, the PIN Server updates the configuration for the PINE and service indicated in the request.

3.	If the PIN Server needs to update the AS with service continuity information, the PIN Server sends a PIN Service Continuity notification to the AS. The PIN Service Continuity notification includes PIN identifier, PINE identifier(s), PEGC identifier, service identifier and may include an application session identifier and terminating endpoints on the PINE and PEGC.

4. 	Upon receiving the notification, the AS validates the notification If the notification is valid, the AS updates the service continuity configuration related to the PINE and service indicated in the notification.

6.	If the processing of the PIN Configuration Service Continuity Update request was successful, the PIN Server sends a PIN Configuration Service Continuity Update response to indicate that the request processing was successful and may include policy information about service continuity for the PINE. Otherwise, the PIN Server sends a PIN Configuration Service Continuity Update response indicating that the request processing failed and can include appropriate reasons.

Upon receiving the PIN Configuration Service Continuity Update response, the PEMC uses the information provided in the response to inform the PINE(s) or the PEGC from which the service continuity request originated as in clause 8.9.2.1.2.

* * * Next Change * * * *

##### 8.9.2.1.6	PIN Service Continuity subscription

###### 8.9.2.1.6.1	General

The PIN service continuity subscription is used by the Application Server to be notified of PIN service continuity events by the PIN Server.

The PIN service continuity notification includes PIN service continuity information.

###### 8.9.2.1.6.2	Procedure

8.9.2.1.6.2.1	PIN service continuity subscribe

Figure 8.9.2.1.6.2.1-1 illustrates the PIN service continuity subscribe procedure.

Pre-conditions:

1.	The application server has already received the address of the PIN server;

2.	The application server has been authorized to communicate with PIN server;

Figure 8.9.2.1.6.2.1-1: PIN service continuity subscribe

1.	The application server sends the PIN service continuity subscribe request to the PIN server. The PIN service continuity subscribe request includes the application server identifier along with the security credentials.

2.	Upon receiving the request from the application server, the PIN server checks if the application server is authorized to subscribe. The authorization check may apply to an individual PIN. If the request is authorized, the PIN server creates and stores the subscription for application server.

3.	If the processing of the request is successful, the PIN server sends a PIN service continuity subscribe response to the application server, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the application server shall send a PIN service continuity update request prior to the expiration time. If a new PIN service continuity update request is not received prior to the expiration time, the PIN server shall treat the application server as implicitly unsubscribed.

8.9.2.1.6.2.2	PIN service continuity notify

Figure 8.9.2.1.6.2.2-1 illustrates the PIN service continuity notify procedure.

Pre-conditions:

1.	The application server has subscribed with the PIN server as specified in clause 8.9.2.1.6.2.1.

Figure 8.9.2.1.6.2.2-1: PIN service continuity notify procedure

1.	When a service continuity event occurs at the PIN server that satisfies triggering conditions, the PIN server notifies subscribed application servers.

2.	The PIN server sends an PIN service continuity notification to the subscribed application server(s) related to the service continuity event and includes the PIN service continuity information.

The PIN service continuity information includes the PIN server identifier, PINE identifier(s), PEGC identifier, service identifier and may include an application session identifier and terminating endpoints on the PINE and PEGC.

3.	Upon receiving the notification, the application server uses the information provided in the notification to update the service continuity configuration related to the PINE and service indicated in the notification.

8.9.2.1.6.2.3	PIN service continuity update

Figure 8.9.2.1.6.2.3-1 illustrates the PIN service continuity update procedure.

Pre-conditions:

1.	The application server has subscribed with the PIN server as specified in clause 8.9.2.1.6.2.1.

Figure 8.9.2.1.6.2.3-1: PIN service continuity update

1.	The application server sends a PIN service continuity update request to the PIN server. The PIN service continuity update request includes the security credentials and the subscription identifier. It may also include notification target address and proposed expiration time.

2.	Upon receiving the request from the application server, the PIN server checks if the application server is authorized to update the subscription information. If the request is authorized, the PIN server updates the stored subscription.

3.	If the processing of the request is successful, the PIN server sends a PIN service continuity update response to the application server, which may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the application server shall send a PIN service continuity update request prior to the expiration time. If a PIN service continuity update request is not received prior to the expiration time, the PIN server shall treat the application server as implicitly unsubscribed.

8.9.2.1.6.2.4	PIN service continuity unsubscribe

Figure 8.9.2.1.6.2.4-1 illustrates the PIN service continuity unsubscribe procedure.

Pre-conditions:

1.	The application server has subscribed with the PIN server as specified in clause 8.9.2.1.6.2.1.

Figure 8.9.2.1.6.2.4-1: PIN service continuity unsubscribe

1.	The application server sends a PIN service continuity unsubscribe request to the PIN server. The PIN service continuity unsubscribe request includes the security credentials and the subscription identifier.

2.	Upon receiving the request from the application server, the PIN server checks if the application server is authorized to unsubscribe. If the request is authorized, the PIN server cancels the subscription indicated by the subscription identifier.

3.	If the processing of the request is successful, the PIN server sends a PIN service continuity unsubscribe response to the application server.

* * * Next Change * * * *

#### 8.9.3.1	General

The following information flows are specified for Service Continuity:

-	PIN Management PEGC Service Continuity request and response

-	PIN Management PEGC Configuration request and response

-	PIN Configuration Update request and response

-	PIN Management PEGC Discovery request and response

-	PIN service continuity subscribe

-	PIN service continuity notify

-	PIN service continuity update

-	PIN service continuity unsubscribe

* * * Next Change * * * *

#### 8.9.3.10	PIN service continuity subscribe request

Table 8.9.3.10-1: PIN service continuity subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Application server ID | M | Unique identifier of the application server. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN ID | M | The identifier of PIN. |
| Notification Target Address | M | The Notification target address (e.g., URL) where the notifications destined for the application server should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

#### 8.9.3.11	PIN service continuity subscribe response

Table 8.9.3.11-1: PIN service continuity subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response (see NOTE) | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |
| NOTE: One IE is included in the response. |  |  |

#### 8.9.3.12	PIN service continuity notify

Table 8.9.3.12-1 describes information elements in the PIN service continuity notification from the PIN server to application server.

Table 8.7.3.12-1: PIN service continuity notification

| Information element | Status | Description |
| --- | --- | --- |
| PIN server identifier | M | Identifier of the PIN server sending the notification. |
| PIN identifier | M | Identifier of the PIN. |
| PINE identifier(s) | M | Identifier(s) of PINE(s) |
| PEGC identifier | M | Identifier of the PEGC |
| Service identifier | M | Identifier of a service |
| Application session identifier | M | Identifier of the application session. |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) on the PINE and PEGC |

#### 8.9.3.13	PIN service continuity update request

Table 8.9.3.13-1: PIN service continuity update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| Notification Target Address | O | The Notification target address (e.g., URL) where the notifications destined for the application server should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

#### 8.9.3.14	PIN service continuity update response

Table 8.9.3.14-1: PIN service continuity update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response (see NOTE) | O | Indicates that the subscription update request failed. |
| > Cause | M | Indicates the cause of subscription update request failure |
| NOTE: One IE is included in the response. |  |  |

#### 8.9.3.15	PIN service continuity unsubscribe request

Table 8.9.3.15-1: PIN service continuity unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |

#### 8.9.3.16	PIN service continuity unsubscribe response

Table 8.7.3.16-1: PIN service continuity unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the unsubscribe request was successful. |
| Failure response (see NOTE) | O | Indicates that the unsubscribe request failed. |
| > Cause | M | Indicates the cause of unsubscribe request failure |
| NOTE: One IE is included in the response. |  |  |

* * * Next Change * * * *

### 8.9.4	APIs

#### 8.9.4.1	General

Table 8.9.4.1-1 illustrates the API for AS service continuity subscription.

Table 8.9.4.1-1: Ppinserver_ASServiceContinuitySubscription API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ppinserver_ASServiceContinuity | Subscribe | Subscribe/Notify | AS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

#### 8.9.4.2	Ppinserver_ ASServiceContinuity_Subscribe operation

API operation name: Ppinserver_ASServiceContinuity_Subscribe

Description: The consumer subscribes for service continuity information.

Inputs: See clause 8.9.3.10.

Outputs: See clause 8.9.3.11.

See clause 8.9.2.1.6.2.1 for details of usage of this operation.

#### 8.9.4.3	Ppinserver_ ASServiceContinuity_Notify operation

API operation name: Ppinserver_ASServiceContinuity_Notify

Description: The consumer is notified with service continuity information.

Inputs: See clause 8.9.3.12.

Outputs: None.

See clause 8.9.2.1.6.2.2 for details of usage of this operation.

#### 8.9.4.4	Ppinserver_ ASServiceContinuity_ UpdateSubscription operation

API operation name: Ppinserver_ASServiceContinuity_UpdateSubscription

Description: The consumer updates an existing subscription for service continuity information.

Inputs: See clause 8.9.3.13.

Outputs: See clause 8.9.3.14.

See clause 8.9.2.1.6.2.3 for details of usage of this operation.

#### 8.9.4.5	Ppinserver_ ASServiceContinuity_ Unsubscribe operation

API operation name: Ppinserver_ASServiceContinuity_Unsubscribe

Description: The consumer cancels an existing subscription for service continuity information.

Inputs: See clause 8.9.3.15.

Outputs: See clause 8.9.3.16.

See clause 8.9.2.1.6.2.4 for details of usage of this operation.

* * * End of Change(s) * * * *