3GPP TSG-SA6 Meeting #56	S6-232501

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0464 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup on CAS decided ACR scenario via the last S-EES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During 3GPP SA6#55, two contributions have been agreed, and clauses 8.8.2A.6 and 8.8.3.10 were added to TS 23.558 v18.3.0 as a result. Since they were added at the same time, the reference of clause 8.8.3.10 in clause 8.8.2A.6 is still missing. In addtion, in clause 8.8.2A.6, the UE ID is assumed to be known by the CAS in the pre-condition to facilitate the discovery of the T-EAS when ACR is performed from the CAS to the EAS. However, in step 3 of the procedure, the use of the UE ID in the T-EAS discovery is not clarified. Other typos and a formating error also need to be fixed in clause 8.8.2A.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial fix in the precondition of clause 8.8.2A.6. - Editorial fix in step 3 of clause 8.8.2A.6 by moving it to a new line. - Adding a reference to clause 8.8.3.10 in clause 8.8.2A.6. - Adding in step 3 of clause 8.8.3.10, the use of UE ID in T-EAS discovery request for the S-EES to identify the UE for which the discovery is being performed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is unclear and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.2A.6	CAS decided ACR scenario via last S-EES

In this scenario, the CAS detects the need for ACR and makes the decision about whether to perform the ACR and starts the ACR at a proper time.

During the ACR execution phase, the CAS may need to determine an EES in the service area of the EEC before continuing with T-EAS discovery. The CAS may send the EAS discovery request to the S-EES that selects an appropriate T-EES and T-EAS as per clause 8.8.3.2 where the CAS acts as a S-EAS.

Pre-conditions:

-	The CAS has a business relationship with the ECSP, otherwise, the EES will reject the CAS request during authorization.

-	Prior to being connected to the CAS, the EEC was connected to an S-EES that may still hold its context and the CAS knows the S-EES (e.g. as in clause 8.8.3.10 when ACR is performed from EAS to CAS).

-	AC has requested and forwarded the UE ID as per clause 8.14.2.6 while performing ACR from EAS to CAS as described in clause 8.8.2A or the CAS knows UE ID using the procedure defined in clause 8.6.5.

Figure 8.8.2A.6-1: CAS decided ACR with T-EAS Discovery via S-EES

Phase I: ACR Detection

1.	The CAS detects the need for ACR. The CAS either receives ACR management notifications from S-EES indicating that ACR may be required ("ACR monitoring" event), or self detects the need for ACR (e.g. upon receipt of a "user plane path change" event).

Phase II: ACR Decision

2.	The CAS makes the decision to perform the ACR.

Phase III: ACR Execution

3.	Same as steps 3 to 5 in clause 8.8.2.4 where the CAS acts as an S-EAS and it uses the UE ID in the T-EAS discovery request for the S-EES to identify the UE for which the discovery is being performed. If ACR monitoring event is used in step 1, the T-EAS discovery in step 3 of clause 8.8.2.4 is skipped by the CAS.

4.	If the S-EES can communicate with the EEC, step 6 from clause 8.8.2.4 is performed to notify the EEC of the selected T-EES. Otherwise, the notification is sent to the EEC via the SEAL notification management service as described in clause 9.1 if the EEC has subscribed to receive notifications from the S-EES or via Application Triggering via core network.

5.	Same as step 7 in clause 8.8.2.4 where the CAS acts as an S-EAS.

Phase IV: Post-ACR Clean up

6.	Same as step 8 to step 9 in clause 8.8.2.4 where the CAS acts as an S-EAS.

7.	If the S-EES can communicate with the EEC, step 10 from clause 8.8.2.4 is performed. Otherwise, the notification is sent to the EEC via the SEAL notification management service as described in clause 9.1 if the EEC has subscribed to receive notifications from the S-EES or via Application Triggering via core network.

* * * END of Change * * * *