3GPP TSG-SA WG6 Meeting #56	S6-232505

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0230 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRM device triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-97 contains KI #4 on Device triggering. Introducing Device Triggering in NRM allows for a generic mechanism which can be used by any VAL servers and completes the normative work for the KI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added procedure for device triggering in NRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Device triggering in SEAL on behalf of VAL applications is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New: 14.3.x; 14.3.2.y1; 14.3.2.y2; 14.4.2.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR 23.700-97 CR 0004 |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.x	Device triggering

#### 14.3.x.1	General

A service  may initiate a device trigger to a UE to cause it to establish a connection to the SEAL layer, to connect to a VAL server, to provide updated information, etc. The NRM Server may initiate the device trigger itself (implicit) or it may be initiated by a request that it receives from VAL Servers (explicit).

#### 14.3.x.2	Device Triggering via NRM procedure

Figure 14.3.x-1: Device Triggering via NRM Procedure

1). The device triggering procedure may be initiated based as an optional interaction between a VAL Server and the NRM Server, For example, the VAL Server may send an explicit API request, or the VAL Server may send another message based on which the NRM Server determines to start the device triggering procedure.

2). The NRM Server determines to initiate the device triggering. The NRM Server may use UE Availability and/or pre-configured information to determine the timing of the Device Triggering request, e.g. the trigger may be sent to ensure that a target UE in PSM mode is reachable when resuming communications.

3).	The NRM Server performs the device triggering procedure described in 3GPP TS 23.682 [7] clause 5.2. The procedure requires that the UE Identifier, port number(s) and protocol information are available at the NRM Server.

As part of the procedure, the NRM Server receives a Device Triggering delivery status report from SCEF/NEF indicating the success of the delivery.

4).	If a request was received in step 1, the NRM Server responds to the request.

Based on the trigger purpose derived from the payload, the targeted NRM Client or VAL Client performs the corresponding actions (e.g., connect to the VAL Server).

.

* * * Next Change * * * *

#### 14.3.2.y1	Device triggering request

Table 14.3.y1-1 describes the information flow for the Device triggering request from the VAL server to the NRM Server.

Table 14.3.2.y1-1: Device triggering request

| Information element | Monitoring | Description |
| --- | --- | --- |
| VAL UE ID | M | UE ID to be triggered. |
| Port number | M | Device triggering port number |
| Triggering purpose | O | Indicates the device triggering purpose which may be included in the payload |

#### 14.3.2.y2	Device triggering response

Table 14.3.2.y2-1 describes the information flow for Device triggering response from the NRM server to the VAL server.

Table 14.3.2.y2-1: Device triggering response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the operation. |

* * * Next Change * * * *

#### 14.4.2.z	Request_Device_Triggering

API operation name: Request_Device_Triggering

Description: Requesting device triggering.

Known Consumers: VAL server.

Inputs: See subclause

Outputs: See subclause

* * * End of Changes * * * *