SA WG6 Meeting #57	S6-232812

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0027 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PINE remove |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Section 8.5.8.2.3, the PIN ID is missing in step 1 and step 2, because it should be identified that in which PIN, this PINE is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PIN ID in the PIN status notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that if one PINE joins multiple PIN and in which PIN that this PINE is removed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.2.3, 8.5.9.3.4, 8.5.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.8.2.3	Procedure of PEMC removes the PIN elements from a PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with PEMC;

Figure 8.5.8.2.3-1: Remove a PIN element from a PIN by PEMC

1.	The PEMC decides to remove the PINE (identified by GPSI, PIN client ID and in certain PIN identified by PIN ID).

2.	The PEMC sends the PIN status notify to the PINE that this PINE has been removed from the PIN that identified by PIN ID.

3-4.	The PEMC sends the PIN status notify to the PEGC and PIN server containing the details of the PIN client that is removed from the PIN. The details of the PINE include PIN client ID, GPSI and etc.

5-7.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that is removed from the PIN. The PEGC disables the access control information for this PINE.

* * * Start of Change 2 * * *

8.5.9.3.4	PIN status notify

Table 8.5.9.3.4-1 describes information elements in the PIN status notify from the PEMC to PEMC/PEGC/PINE/PIN server.

Table 8.5.9.3.4-1: PIN status notify

| Information element | Status | Description |
| --- | --- | --- |
| PIN Status event type | M | Identifies PIN status event types contained in the notification; more than one PIN status event type can be provided. PIN status event types - PIN modification - PINE management (PINE join/leave/PINE removed) - PIN profile update - PIN state (Activated or de-activated) |
| PIN ID | M | The identifier of PIN. |
| PINE management | O | Indicates the PINE join/leave the PIN, this IE is present if the PIN status event type is PINE management. |
| > PINE join into the PIN | O | Indicates the PINEs join into the PIN |
| >> PIN client ID | M | The identifier of the joined PINE |
| >> PIN client profile(s) | O | Profiles of PIN clients that joins the PIN. The PIN client profiles are further described in Table 8.2.2.3. |
| > PINE leaves the PIN | O | Indicates the PINEs leaves from the PIN |
| >> PIN client ID | M | The identifier of the leaved PINE |
| PIN modification | O | Indicates the modification of PIN, this IE is present if the PIN status event type is PIN modification. |
| > Updated PIN profile | M | Updated PIN profile information that includes identifiers of PIN clients authorized to use as PEGC or PEMC. |
| >> PEGC information | O | Includes the PEGC information defined in Table 8.5.2.3.3-1. |
| >> PEMC information | O | Includes the PEMC information. |
| >>> PEMC address | O | Assigned IP/MAC address or port number of PEMC |
| >>> Identifier of PEMCs | O | Indicates the PINE identifier authorized to be the PEMCs of this PIN. |
| PIN profiles update | O | Indicates a PIN profile update that indicated in section 8.2.2, this IE is present if the PIN status event type is PIN profile update. |

* * * Start of Change 3 * * *

8.5.9.2.2	PIN status notify

Figure 8.5.9.2.2-1 illustrates the PIN status notify procedure.

Pre-conditions:

1.	The PINE/PEGC/PIN has subscribed with the PEMC for the PIN status as specified in clause 8.5.9.2.1.

Figure 8.5.9.2.2-1: PIN status notify procedure

1.	When an event occurs at the PEMC that satisfies trigger conditions for notifying (e.g. to provide updated PIN status, for example when a PINE joins into the PIN) a PINE/PEGC/PIN server.

The status information of PIN is updated including the following cases:

-	PIN modification: The PIN modification includes the PEMC/PEGC relocation and the PINE joins into PIN or leaves the PIN;

-	PIN profile updates: The PIN profiles is referred to the section 8.2.2, and the updated PIN profile may be coordinated to other PINEs;

-	PIN state changes: Whenever the PIN state is changed from activated to de-activated or vice-versa;

2.	The PEMC sends an PIN status notify to the PINE/PEGC/PIN server with the updated PIN status information.

The PIN status information notification includes the following parameters:

-	PIN modification: If the PEMC decides the PEMC/PEGC relocation, the PEMC includes the parameters about the newly assigned PEMCs and PEGCs in the PIN status update request, for example, the PEMC/PEGC ID or address, the PEGC information in Table 8.5.2.3.3-1 about access control information and etc;

-	PINE management: If the PEMC decides the PINE to join or leave the PIN, the PEMC includes the parameters about the newly added/removed PINE in the PIN status update request, for example, the PIN client ID or address, the PIN client profile and etc. Also if the PINE is removed from the PIN, the parameters about the removed PINE should be included in the PIN status update request;

-	PIN profile update: If the PIN profile is updated, the PEMC includes the parameters about updated PIN profile in the PIN status update request;

-	PIN state changes: Whenever the PIN state is changed from activated to de-activated or vice-versa;

3.	Upon receiving the notification, the PINE, PEGC or PIN server updates the PIN profile according to the information in the PIN status notify.

* * * End of Change * * *