3GPP TSG-SA WG6 Meeting #57	S6-232823

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0124 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing AEF APIs in clause 11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, eCAPIF |  |  |  |  |  | Date: |  |  | 2023-10-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the AEF APIs corresponding to clause 8.15, 8.17 and 8.18 are missing. The description of 11.2.3 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new APIs related to procedures in clauses 8.15, 8.17, 8.18 in clause 11, and update the API table accordingly. Correct the description of 11.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The APIs for the Rel-18 features are not specified and the downstream group cannot start their work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, 11.2.3, (new)11.2.x, (new)11.y, (new) 11.y.1, (new) 11.y.2, (new) 11.y.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

11	API exposing function APIs

11.1	General

Table 11.1-1 illustrates the API exposing function APIs.

Table 11.1-1: List of API exposing function APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| AEF_Security API | Revoke_Authorization | CAPIF Core Function | Request/ Response |
|  | Initiate_Authentication | API Invoker | Request/ Response |
|  | Service_API_invocation _Authentication | API Invoker | Request/ Response |
| AEF_Access_Control API | Access_Control | API Invoker | Request/ Response |
|  | Access_Control_With_Cascaded_AEFs | API Invoker AEF | Request/ Response |

# /******************************** Next Change **********************/

11.2.3	Initiate_Authentication operation

API operation name: Initiate_Authentication

Description: Authentication between the API invoker and the AEF prior to service API invocation.

Known Consumers: API Invoker.

Inputs: Refer subclause 8.14.2.

Outputs: Refer subclause 8.14.2.

See subclause 8.14.3 for the details of usage of this API operation.

# /******************************** Next Change **********************/

11.2.x	Service_API_invocation _Authentication operation

API operation name: Service_API_invocation _Authentication

Description: Authentication between the API invoker and the AEF upon the service API invocation.

Known Consumers: API Invoker.

Inputs: Refer subclause 8.15.2.1.

Outputs: Refer subclause 8.15.2.2.

See subclause 8.15.3 for the details of usage of this API operation.

# /******************************** Next Change **********************/

11.y	AEF_Access_Control API

11.y.1	General

API description: This API enables the API invoker to communicate with the AEF or cascaded AEFs for access control of service API.

11.y.2	Access_Control operation

API operation name: Access_Control

Description: Control the access of service API by the API invoker based on policy or usage limits.

Known Consumers: API Invoker.

Inputs: Refer subclause 8.17.2.1.

Outputs: Refer subclause 8.17.2.2.

See subclause 8.17.3 for the details of usage of this API operation.

11.y.3	Access_Control_With_Cascaded_AEFs operation

API operation name: Access_Control_With_Cascaded_AEFs

Description: Provide access control based on two cascaded API Exposing Function (AEF) instances.

Known Consumers: API Invoker, AEF.

Inputs: Refer subclause 8.18.2.1.

Outputs: Refer subclause 8.18.2.2.

See subclause 8.18.3 for the details of usage of this API operation.