3GPP TSG-SA WG6 Meeting #57	S6-232868

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0169 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | delivery mechanism for the recipient is not registered |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposed to add a new delivery mechanism for the recipient is not registered to fulfill the requirement [R-5.1.2-005] specified in TS TS22.262, i.e. [R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available. This delivery mechanism is different from the store and forward mechanism specified in clause 8.3.6 of 3GPP TS 23.554 since it applies in the terminating side and not triggered by the requirement from sender. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | add a new delivery mechanism for the recipient is not registered to fulfill the requirement [R-5.1.2-005] specified in TS TS22.262. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement [R-5.1.2-005] specified in TS TS22.262 is not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.x	MSGin5G deferred message

This procedure applies when an MSGin5G message is received at the MSGin5G Server for delivery and the message cannot be delivered to the recipient since the recipient UE is unavailable (disconnected or power off), i.e. not registered; otherwise, the procedure detailed in clause 8.3.2 applies.

This procedure applies to the MSGin5G Server that is the home of the terminating MSGin5G Service ID based on the configuration from MSGin5G service provider. If the deferred message service is enabled, this procedure applies to all MSGin5G messages received at the MSGin5G Server for delivery.

Figure 8.3.x-1 shows the procedure for providing deferred message services.

Pre-conditions:

1.	MSGin5G message termination handling, the MSGin5G Servers deliver messages to an MSGin5G Service endpoint based on the terminating MSGin5G Service ID. For each MSGin5G Server in the path of delivery, it forwards the messages to the next MSGin5G Server until it reaches to the MSGin5G Server that is the home of the terminating MSGin5G Service ID.

2.	The MSGin5G message reaches to the MSGin5G Server that is the home of the terminating MSGin5G Service ID.

3.	The UE with the terminating MSGin5G Service ID is not registered.

Figure 8.3.x-1: MSGin5G deferred message procedure

1.	MSGin5G Server tries to deliver the MSGin5G message to the UE with the terminating MSGin5G Service ID but no UE with the terminating MSGin5G Service ID has registered.

2.	The MSGin5G Server checks whether the value of the Store and forward flag IE (see Table 8.3.2-1) in the MSGin5G message indicates that store and forward services are requested by the sender. If the  store and forward services are requested by the sender, the procedure specified in clause 8.3.6 applies.

3.	If the store and forward services are not requested by the sender, the MSGin5G Server checks whether the deferred message service is enabled by the MSGin5G service provider. If the deferred message service is enabled, the MSGin5G Server stores the MSGin5G message and continues the rest steps in this clause, otherwise the MSGin5G Server discards the MSGin5G message and skipped all the rest steps except step 8b.

4.	Before either the Message expiration time is expired or the maximum deferred time is expired, the MSGin5G Server may trigger the Recipient UE based on the MSGin5G device triggering procedure in clause 8.9.3.

NOTE:	the the maximum deferred time is configured on the MSGin5G Server by the MSGin5G Service provider

5.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes deferred status information in the Delivery Status IE, e.g., that the delivery had been deferred.

6a.	The recipient UE becomes available due to its registration.

7a.	 the MSGin5G Server attempts delivery of the request using the procedure specified in clause 8.3.3.

8a.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which status information in the Delivery Status IE, e.g., that the message was discarded.

6b.	The recipient UE has not became available due to its registration before the Message expiration time is expired or the maximum deferred time is expired.

8b.	The MSGin5G Server discards the MSGin5G message and may send a message response as defined in table 8.3.2-3 which status information in the Delivery Status IE, e.g., that the message was discarded.

If the recipient UE has became available due to its registration before the Message expiration time is expired or the maximum deferred time is expired, step 6a, 7a and 8a apply, otherwise step 6b and 8b apply.